| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0100-v1 |
| --- | --- |

| Title: | On Maximum Block Size for Chroma Transform Skip |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Tsukuba Masaru Ikeda Yoichi Yagasaki Teruhiko Suzuki 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81-50-3750-2779 Takeshi.Tsukuba@sony.com Masaru.Ikeda@sony.com Yoichi.Yagasaki@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In VVC WD, there is an inconsistency between maximum transform block size for chroma and transform skipped block size for chroma; Maximum transform skipped block size for chroma is always set to (1  <<  ( log2_transform_skip_max_size_minus2 + 2 ) regardless of chroma format while maximum transform block size for chroma is set depending on chroma format. This contribution proposes to derive maximum block size for chroma transform skip based on log2_transform_skip_max_size_minus2, SubWidthC, and SubHeightC.

At normal QP condition, it is reported that proposal gives average BD-rate differences of (0.00 %, -0.01 %, -0.01 %) for AI, (0.00 %, 0.02 %, 0.06 %) for RA and (0.02 %, 0.15 %, -0.06 %) for LB, respectively.

At low QP condition (QP=2, 7, 12, 17), it is reported that proposal gives average BD-rate differences of (0.00%, 0.00 %, 0.01%) for AI, (0.00%, 0.00 %, 0.00%) for RA and (0.00 %, 0.02 %, 0.00 %) for LB, respectively. Especially, for SCC sequence, it is reported that proposal provides some BD-rate saving: (-0.14%, -0.13%, -0.11%) for AI, (-0.06%, 0.24%, 0.24%) for RA and (-0.39%, -0.25%, -0.26%) for LB.

Introduction

In VVC WD [1], maximum transform skipped block size (log2_transform_skip_max_size_minus2) is signaled in PPS and a variable MaxTsSize is derived as below:

MaxTsSize = 1  <<  ( log2_transform_skip_max_size_minus2 + 2 )	(*)

Maximum transform block (maxTbWidth, maxTbHeight) are derived as below:

maxTbWidth = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / SubWidthC	 (1068)

maxTbHeight = ( cIdx  = =  0 ) ? MaxTbSizeY : MaxTbSizeY / SubHeightC	(1069)

Table 1-1 shows maximum transform block size and maximum transform skipped block size with log2_transform_skip_max_size_minus2 equal to 3.

As can be seen, there is an inconsistency between maximum transform block size for chroma and transform skipped block size for chroma; Maximum transform skipped block size for chroma is always set to 32x32 regardless of chroma format while maximum transform block size for chroma is set depending on chroma format. Especially, in case of MaxTbSizeY equal to 32, the value of maximum transform skipped block size (32x32) for chroma is invalid at 420 and 422.

Table 1-1: maximum transform block size and maximum transform skipped block size

| MaxTbSizeY | chroma format | Max transform block size (WxH) for Luma | Max transform block size for Chroma | Max transform skipped block size for Luma | Max transform skipped block size for Chroma |
| --- | --- | --- | --- | --- | --- |
| 64 | 420 | 64x64 | 32x32 | 32x32 | 32x32 |
| 64 | 422 | 64x64 | 64x32 | 32x32 | 32x32 |
| 64 | 444 | 64x64 | 64x64 | 32x32 | 32x32 |
| 32 | 420 | 32x32 | 16x16 | 32x32 | 32x32 |
| 32 | 422 | 32x32 | 32x16 | 32x32 | 32x32 |
| 32 | 444 | 32x32 | 32x32 | 32x32 | 32x32 |

Proposal

To resolve an inconsistency between maximum transform block size and maximum transform skipped block size for chroma, this contribution proposes to derive maximum transform skipped block size for chroma based on log2_transform_skip_max_size_minus2, subWidthC, and subHeightC as blow:

Semantic changes

log2_transform_skip_max_size_minus2 specifies the maximum block size used for luma transform skip, and shall be in the range of 0 to 3, inclusive.

The variable MaxTsSize is set equal to 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ).

The variables MaxTsWidthY and MaxTsHeightY are derived as below:

MaxTsWidthY = 1  <<  ( log2_transform_skip_max_size_minus2 + 2 )	(X1)

MaxTsHeightY = 1  <<  ( log2_transform_skip_max_size_minus2 + 2 ) 	(X2)

The variables MaxTsWidthC and MaxTsHeightC are derived as below:

MaxTsWidthC = MaxTsWidthY / SubWidthC 		(X3)

MaxTsHeightC = MaxTsHeightY / SubHeightC 		(X4)

In addition, signaling condition of transform_skip_flag is modified using MaxTsWidth{Y,C} and MaxTsHeight{Y,C} as below:

Table 2-1: Changes for signaling condition of transform_skip_flag

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsWidthY MaxTsSize && tbHeight <= MaxTsHeightY MaxTsSize && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsWidthC MaxTsSize && hC <= MaxTsHeightC MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsWidthC MaxTsSize && hC <= MaxTsHeightC MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

Table 1-1 becomes Table 2-2 by applying proposed changes.

Table 2-2: maximum transform block size and maximum transform skipped block size

| MaxTbSizeY | chroma format | Max transform block size (WxH) for Luma | Max transform block size for Chroma | Max transform skipped block size for Luma | Max transform skipped block size for Chroma |
| --- | --- | --- | --- | --- | --- |
| 64 | 420 | 64x64 | 32x32 | 32x32 | 16x16 |
| 64 | 422 | 64x64 | 64x32 | 32x32 | 32x16 |
| 64 | 444 | 64x64 | 64x64 | 32x32 | 32x32 |
| 32 | 420 | 32x32 | 16x16 | 32x32 | 16x16 |
| 32 | 422 | 32x32 | 32x16 | 32x32 | 32x16 |
| 32 | 444 | 32x32 | 32x32 | 32x32 | 32x32 |

Simulation Results

The proposed method has been implemented on top of VTM-7.0 and following simulations were conducted under common test condition [2]:

- Test1: VTM-7.0 vs Proposal at normal QP condition

- Test2: VTM-7.0 vs Proposal at low QP condition with first 100 frames (QP=2, 7, 12, 17)

Table 3-1 to 3-2 shows the results of each Test1 and Test2, respectively.

Table 3-1: Results of Test1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.03% | 100% | 100% |
| Class A2 | 0.00% | -0.02% | -0.02% | 101% | 102% |
| Class B | 0.00% | -0.01% | -0.03% | 98% | 99% |
| Class C | 0.00% | -0.01% | -0.05% | 101% | 102% |
| Class E | -0.01% | 0.02% | 0.02% | 101% | 102% |
| Overall | 0.00% | -0.01% | -0.01% | 100% | 101% |
| Class D | 0.00% | 0.00% | -0.01% | 103% | 102% |
| Class F | -0.01% | 0.04% | -0.01% | 99% | 100% |
| Class SCC | -0.01% | 0.06% | 0.01% | 96% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.03% | 0.11% | 101% | 101% |
| Class A2 | -0.01% | 0.07% | 0.10% | 102% | 102% |
| Class B | -0.01% | 0.02% | 0.06% | 100% | 100% |
| Class C | -0.01% | 0.00% | 0.00% | 101% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.02% | 0.06% | 101% | 101% |
| Class D | -0.01% | -0.04% | 0.03% | 104% | 105% |
| Class F | 0.00% | -0.05% | -0.03% | 99% | 101% |
| Class SCC | 0.01% | 0.06% | 0.07% | 99% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.37% | -0.19% | 98% | 98% |
| Class C | 0.01% | -0.11% | -0.04% | 98% | 99% |
| Class E | 0.02% | 0.17% | 0.15% | 100% | 100% |
| Overall | 0.02% | -0.15% | -0.06% | 98% | 99% |
| Class D | 0.02% | -0.02% | 0.28% | 104% | 105% |
| Class F | -0.11% | -0.22% | 0.55% | 98% | 99% |
| Class SCC | -0.16% | -0.02% | 0.01% | 99% | 100% |

Table 3-2: Results of Test2

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 100% | 101% |
| Class A2 | 0.00% | -0.01% | 0.00% | 100% | 101% |
| Class B | -0.01% | 0.01% | 0.02% | 99% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | -0.01% | 0.00% | 0.02% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.01% | 99% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 98% | 101% |
| Class F | -0.02% | -0.03% | 0.01% | 97% | 99% |
| Class SCC | -0.14% | -0.13% | -0.11% | 100% | 100% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.02% | -0.01% | 99% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 99% | 101% |
| Class C | 0.00% | -0.01% | 0.00% | 97% | 97% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class D | 0.00% | -0.01% | -0.01% | 99% | 100% |
| Class F | 0.00% | 0.13% | 0.16% | 98% | 100% |
| Class SCC | -0.06% | 0.24% | 0.24% | 98% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.00% | 0.01% | 100% | 100% |
| Class C | 0.00% | 0.01% | 0.01% | 99% | 100% |
| Class E | -0.02% | 0.07% | -0.04% | 100% | 101% |
| Overall | 0.00% | 0.02% | 0.00% | 99% | 100% |
| Class D | 0.01% | 0.01% | -0.02% | 100% | 101% |
| Class F | 0.03% | 0.05% | 0.01% | 98% | 100% |
| Class SCC | -0.39% | -0.25% | -0.26% | 96% | 100% |

Conclusion

It is recommended to adopt proposed maximum transform skipped block size derivation into the next VTM/VVC WD.

Reference

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7), ” JVET-P2001, Oct. 2019

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, Apr. 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).