| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0104-v1 |
| --- | --- |

| Title: | On worst-case complexity of LFNST |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hilmi E. Egilmez, Alican Nalci, Muhammed Coban, Vadim Seregin, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Email: | hegilmez@qti.qualcomm.com analci@qti.qualcomm.com mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes two alternative designs for LFNST without increasing the worst-case complexity in terms of multiplications per coefficient. The first design aims to simplify the existing 8x8 LFNST design by replacing the L-shaped, 48-sample support pattern with the 8x8-square. It is experimentally shown that the proposed method lead to  -0.09% AI and -0.06% RA BD-rates over VTM-7.0. The second design keeps the L-shaped, 48-sample support pattern, but reduces the number of normatively zeroed-out coefficients. The experimental results of the second design show that -0.02% AI and -0.02% RA BD-rates are achieved over VTM-7.0.

# Introduction

In VTM-7.0, 8x8 LFNST is defined by a collection of 16x48 transform matrices, and it produces at most 16 transform coefficients by transforming 48 DCT-2 coefficients in the L-shaped region with 48 samples as shown in Fig. 1.

Fig 1: L-shaped pattern with 48 samples used in current VTM for 8x8 LFNST

While 8x8 LFNST allows up to 16 non-zero coefficients for non-8x8 TU sizes (TUs with more than 64 samples), 8x8 TU is a special case, where up to 8 non-zero coefficient are allowed, and the remaining 56 coefficients are normatively zeroed out. In the VVC working draft, the following highlighted line (i.e., nonZeroSize) defines the maximum number of non-zero coefficients allowed.

_____________________________________________________________________________________________

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(1149)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(1150)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(1151)

nLfnstSize = 1  <<  log2LfnstSize		(1152)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(1153)

# Proposed Method

In this contribution, two simple methods are proposed for 8x8 LFNST without increasing the worst-case complexity in terms of multiplications per coefficient.

## Method 1: Replacing the L-shaped LFNST transform region

Method 1 replaces the irregular, L-shaped transform region with the 8x8-square block illustrated in Fig. 2.

Fig 2: 8x8 LFNST with regular square-shaped support

As part of Method 1, the transform matrices for 8x8 LFNST had to be trained. The set of trained 16x64 matrices are attached to this submission in “8x8_lfnst.h” file. Since the dimension of the matrices are increased from 16x48 to 16x64, 2Kbytes of additional fixed memory is needed. Yet, the worst-case in terms of number of multiplications (that is equal to 8 currently in VVC) is maintained. Moreover, the additional logic and checks required to handle irregular, L-shaped transform support is removed.

The proposed method can be reflected on the VVC working draft as follows:

_____________________________________________________________________________________________

...

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(1149)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48 64 :  16	(1150)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(1151)

nLfnstSize = 1  <<  log2LfnstSize		(1152)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(1153)

...

- The array d[ x ][ y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, the following applies:

d[ x ][ y ] =  v[ x + ( y << log2LfnstSize ) ] 		(1157)

d[ x ][ y ] =  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(1157)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ x ][ y ] =  v[ y + ( x << log2LfnstSize ) ] 		(1158)

d[ x ][ y ] = ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(1158)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

...

In addition, the 16x48 LFNST matrices in lowFreqTransMatrix[ m ][ n ] are to be integrated according to the matrices in the “8x8_lfnst.h” file.

____________________________________________________________________________________________

## Method 2: Reducing the number of normatively zeroed-out coefficients

In the 14th JVET meeting, based on the discussions, it was concluded that zeroing-out process is undesirable unless it is urgently necessary (see the part of meeting notes copied below).

“JVET-N0364: From implementation perspective, there is no problem that this further zeroing-out is necessary. More cases of zeroing are undesirable if not urgently necessary. Furthermore, it is mentioned that 16x16 DST-VII would be an equivalent worst case not resolved here.”

In the current LFNST design, the worst-case number of multiplications per coefficient is equal to 8 (due to the case of 4x4 LFNST applied to 4x4 TUs, where (8x16)/16 = 8 multiplications per coefficient are needed).

The second-worst case is due to the 8x8 LFNST applied to 8x8 TUs, where (8x48)/64 = 6 multiplications per coefficient are needed. Noting that the worst-case can be still maintained if up to 10 non-zero coefficients were allowed (i.e., (10x48)/64 = 7.5 multiplications per coefficients are needed), Method 2 proposes to reduce the number of unnecessarily zeroed-out coefficients by allowing two more coefficients, so that up to 10 non-zero coefficients can be allowed.

Method 2 can be reflected on the VVC working draft as follows:

_____________________________________________________________________________________________

…

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(1149)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(1150)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(1151)

nLfnstSize = 1  <<  log2LfnstSize		(1152)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(1153)

  - If ( nTbW = = 4  &&  nTbH  = = 4 ) is true, the following applies:

nonZeroSize = 8

  - Otherwise, if ( nTbW = = 8  &&  nTbH  = = 8 ) is true, the following applies:

nonZeroSize = 10

  - Otherwise, the following applies:

nonZeroSize = 16

…

_____________________________________________________________________________________________

# Experimental Results

The proposed methods are implemented on top of VTM-7.0 and evaluated under common test conditions (CTC). Tables 1 and 2 present the results of Method 1 and 2, respectively. Note that low-delay B results are omitted, since LFNST is disabled in CTC for low-delay B.

Table 1.  Results of Method 1 over VTM-7.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.12% | 0.14% | 101% | 101% |
| Class A2 | -0.07% | -0.01% | -0.10% | 99% | 99% |
| Class B | -0.07% | 0.04% | -0.02% | 102% | 104% |
| Class C | -0.07% | 0.05% | 0.04% | 100% | 103% |
| Class E | -0.09% | -0.01% | 0.20% | 99% | 95% |
| Overall | -0.09% | 0.00% | 0.04% | 100% | 101% |
| Class D | -0.03% | 0.02% | -0.10% | 101% | 103% |
| Class F | -0.08% | -0.05% | 0.02% | 101% | 107% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.04% | 0.02% | 99% | 99% |
| Class A2 | -0.08% | 0.11% | -0.03% | 98% | 97% |
| Class B | -0.06% | 0.20% | 0.18% | 101% | 103% |
| Class C | -0.03% | 0.10% | 0.02% | 98% | 94% |
| Class E |  |  |  |  |  |
| Overall | -0.06% | 0.11% | 0.06% | 99% | 99% |
| Class D | -0.05% | -0.09% | -0.03% | 100% | 99% |
| Class F | -0.07% | 0.06% | 0.09% | 100% | 101% |

Table 2.  Results of Method 2 over VTM-7.0 under CTC

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.17% | 0.05% | 101% | 102% |
| Class A2 | -0.01% | 0.05% | -0.08% | 99% | 100% |
| Class B | -0.02% | -0.02% | -0.09% | 102% | 105% |
| Class C | -0.03% | -0.09% | 0.00% | 100% | 101% |
| Class E | -0.03% | -0.05% | 0.05% | 100% | 95% |
| Overall | -0.02% | -0.06% | -0.02% | 101% | 101% |
| Class D | -0.01% | -0.07% | -0.15% | 101% | 105% |
| Class F | 0.00% | -0.08% | 0.00% | 101% | 105% |
|  | Random Access |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.11% | 0.00% | 102% | 103% |
| Class A2 | -0.03% | -0.05% | 0.00% | 99% | 99% |
| Class B | -0.03% | 0.04% | 0.16% | 101% | 103% |
| Class C | -0.02% | -0.06% | -0.01% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.04% | 0.05% | 100% | 100% |
| Class D | -0.03% | -0.31% | -0.03% | 100% | 101% |
| Class F | 0.01% | 0.01% | -0.02% | 101% | 102% |

The following table compares the proposed methods and VTM-7.0 in terms of all relevant aspects.

Table 3: Comparison of VTM-7.0 and proposed methods. In all cases the worst-case number of multiplication per coefficient is equal to 8.

|  | Transform shape | max. # non-zero coeff | Matrix size / memory | BD-rate gains |
| --- | --- | --- | --- | --- |
| VTM-7.0 | Irregular, L-shaped | 4x4: 8, 8x8: 8, other: 16 | 8x8: 16x48 / 8 Kbytes | Anchor |
| Method 1 | Regular, 8x8 squared | 4x4: 8, 8x8: 8, other: 16 | 8x8: 16x64 / 10 Kbytes | AI: 0.09%, RA: 0.06% |
| Method 2 | Irregular, L-shaped | 4x4: 8, 8x8: 10, other: 16 | 8x8: 16x48 / 8 Kbytes | AI: 0.02%, RA: 0.02% |

# Conclusions

This document proposes two methods for 8x8 LFNST without increasing the worst-case number of multiplications per coefficients. Method 1 replaces the L-shaped transform region in VTM-7.0 with the 8x8-square block, which provides about 0.1% average AI coding gain by its larger transform support. As the other alternative, Method 2 reduces the number of coefficients that are unnecessarily zeroed-out without any impact on complexity. It is recommended to adopt one of the proposed alternatives in VVC.

# References

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th Meeting, Geneva, CH, 1–11 Oct. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).