| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0105 |
| --- | --- |

| Title: | Non-CE4: BCW and SIF derivation for pairwise |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | A. Robert, T. Poirier, F. Le Léannec, F. Galpin | Email: | antoine.robert@interdigital.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com franck.galpin@interdigital.com |
| Source: | Interdigital |  |  |

_____________________________

# Abstract

The merge pairwise candidate derives SIF flag and BCW index with two distinct methods in the VTM-7.0. This contribution proposes a harmonization of these derivations by setting both to default or by inheritance from the input candidates.

For the first proposed method, some luma BD-rate loss is observed in RA (0.01%) and some gain is obtained in LDB (-0.01%). But for the second proposed method, the BD-rate performances are the same as VTM-7.0 in RA and some gain is obtained in LDB (-0.02%). The encoding and decoding times are unchanged compared to VTM-7.0 with both methods.

# Introduction

During Gothenburg JVET meeting, a switchable interpolation filter (SIF) in case of half-pel precision JVET-O0057 [1] has been adopted. Consequently, the merge pairwise candidate needs to derive the SIF flag in addition to the BCW index from the input candidates used for averaging.

In order to harmonize the design of the current VVC, it is proposed to derive the BCW index and the SIF flag for the merge pairwise candidate in the same way.

# Description

## Pairwise candidate construction

The merge pairwise candidate is constructed as an average of the two first candidates of the regular merge list, denoted Cand0 and Cand1 respectively in the following. The same process is performed for each reference picture list independently as described in Figure 1.

Figure 1: Pairwise candidate construction for reference frame list Lx

In VTM-7.0 [2], the SIF flag is derived as:

SIFflagPW = (SIFflagCand0 == SIFflagCand1) ? SIFflagCand0 : false

While BCW index is derived as:

BCWPW = default

In the following, the simulations are conducted using the VTM-7.0 reference software with VTM configuration files under CTC.

## Derivation as BCW index

The first way to harmonize these derivations is to use the BCW index strategy that aims at not using these tools for the merge pairwise candidate. The derivations are performed in that case as:

SIFflagPW = false

BCWPW = default

Table 1 and Table 2 show the simulation results in random access and low delay B configurations. The proposed derivation as default brings some BD-rate performance loss in random access (especially in class A1 0.03%) and a small increase in low delay B with the same complexity in encoding/decoding time.

Table 1: Simulation results of merge pairwise candidate derivation of SIF flag and BCW index as default in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.11% | 0.09% | 100% | 101% |
| Class A2 | -0.01% | 0.02% | 0.04% | 100% | 98% |
| Class B | 0.00% | 0.05% | 0.00% | 100% | 100% |
| Class C | 0.01% | 0.01% | 0.05% | 100% | 100% |
| Overall | 0.01% | 0.04% | 0.04% | 100% | 100% |
| Class D | -0.01% | 0.06% | 0.10% | 100% | 101% |
| Class F (optional) | -0.01% | 0.00% | -0.03% | 100% | 100% |

Table 2: Simulation results of merge pairwise candidate derivation of SIF flag and BCW index as default in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | -0.01% | -0.08% | -0.07% | 100% | 101% |
| Class C | -0.02% | -0.16% | -0.02% | 100% | 99% |
| Class E | 0.02% | 0.12% | -0.28% | 99% | 100% |
| Overall | -0.01% | -0.06% | -0.10% | 100% | 100% |
| Class D | 0.03% | 0.14% | 0.18% | 100% | 100% |
| Class F (optional) | 0.05% | 0.00% | 0.02% | 100% | 100% |

## Derivation as SIF flag

The second way to harmonize these derivations is to use the SIF flag strategy for both derivations. The merge pairwise candidate inherits the BCW index of Cand0 when both input candidates use the same BCW index. The derivations are then performed as:

SIFflagPW = (SIFflagCand0 == SIFflagCand1) ? SIFflagCand0 : false

BCWPW = (BCWCand0 == BCWCand1) ? BCWCand0 : default

Table 3 and Table 4 show the simulation results in random access and low delay B configurations over VTM-7.0. The proposed BCW index inheritance keeps the same trend in random access, close to 0.00% for all classes. And in low delay B, the performances seem better (-0.02%) even in class D with -0.08%. The complexity in encoding/decoding time remains the same as the VTM-7.0.

Table 3: Simulation results of merge pairwise candidate derivation of SIF flag and BCW index as SIF flag in random access

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over fixed VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.07% | 100% | 101% |
| Class A2 | -0.01% | 0.01% | -0.01% | 100% | 98% |
| Class B | 0.00% | 0.04% | 0.03% | 100% | 100% |
| Class C | 0.01% | 0.04% | 0.01% | 100% | 102% |
| Overall | 0.00% | 0.01% | 0.02% | 100% | 100% |
| Class D | -0.01% | 0.00% | 0.04% | 100% | 101% |
| Class F (optional) | 0.00% | -0.02% | -0.05% | 100% | 101% |

Table 4: Simulation results of merge pairwise candidate derivation of SIF flag and BCW index as SIF flag in low delay B

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over fixed VTM-5.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class B | 0.01% | -0.07% | -0.07% | 101% | 100% |
| Class C | -0.04% | -0.24% | -0.20% | 100% | 99% |
| Class E | -0.05% | 0.34% | -0.27% | 99% | 99% |
| Overall | -0.02% | -0.03% | -0.16% | 100% | 100% |
| Class D | -0.08% | 0.61% | -0.10% | 100% | 98% |
| Class F (optional) | 0.05% | -0.18% | 0.17% | 100% | 99% |

# Conclusion

This contribution describes two simple SIF flag and BCW index harmonized derivations. In the first method, both are set to the default value and deactivated for the merge pairwise candidate. In the second method, both inherit from the input candidates if both share the same information.

The first method decreases the BD-rate performances in RA configuration (0.01% with 0.03% for A1). But in LDB configuration, the BD-rate performances are increased (-0.01%). With the second method, the BD-rate performances remain the same as VTM-7.0 in RA configuration and can reach gains in LDB configuration (-0.02% with -0.04% for C and -0.08% for D). And in any cases, encoding and decoding complexity are identical to VTM-7.0.

The second proposed derivation gives a better trade-off and should be considered in the next VTM version.

# References

- “CE4: Switchable interpolation filter”, A. Henkel, I. Zupancic, B. Bross, M. Winken, H. Schwarz, D. Marpe, T. Wiegand, document JVET-O0057, 15th Meeting: Gothenburg, SE, 3–12 July 2019.

- Text of Versatile Video Coding (Draft 7), document JVET-P2001, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration(s)

Interdigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).