| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0107-v1 |
| --- | --- |

| Title: | CE3-related: Flag-free Lossless Coding in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alican Nalci, Hongtao Wang, Hilmi E. Egilmez, Muhammed Coban, Marta Karczewicz 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This document proposes a flag-free approach for lossless coding in VVC. Currently, lossless coding is possible via existing syntax with TS residual coding. However, TS residual coding has lesser bit-rate savings as compared to regular coefficient coding as observed in the 16th Geneva meeting. In this document, it is proposed to use regular coefficient coding without additional lossless-specific syntax or signaling. In this case, a lossless coded block or an explicit residual coding switch flag (e.g. as in CE3-2.5) are not signaled but inferred based on the value of the quantization parameter (QP) and TS mode. Based on this inference, RRC coding is used instead of TSRC for lossless.

- The simulation results show overall bit-rate savings of -5.23% AI, -5.58% RA, and -4.96% LDB with Class F: -1.39% AI, -2.40% RA, and -2.12% LDB and Class TGM: 1.54% AI, 0.59% RA, and 1.18% LDB.

- The simulation results with Luma BDPCM enabled show overall bit-rate savings of -6.76% AI, -5.96% RA, and -5.54% LDB with Class F: -5.74% AI, -4.37% RA, and -3.70% LDB and Class TGM: -1.59% AI, -0.71% RA, and 0.16% LDB.

- The simulation results with Luma and Chroma BDPCM enabled show overall bit-rate savings of -7.70% AI, -6.21% RA, and -5.84% LDB with Class F: -7.60% AI, -5.22% RA, and -4.33% LDB and Class TGM: -4.78% AI, -1.90% RA, and -0.05% LDB.

# Introduction

In VTM-7.0, lossless coding is possible via existing syntax with TS enabled both for luma and chroma [1]. This form of lossless coding can only use transform skip residual coding (TSRC) and is unable to switch between TSRC and regular residual coding (RRC). In the 16th JVET Geneva meeting it was observed that RRC had more coding-gains as compared to doing TSRC [2]. CE3 was established to investigate both bit-rate savings for both residual coding methods for lossless coding.

In this document, it is proposed to infer the lossless mode based on the QP value and TS flag at the block level. This removes the need for signaling a lossless-related flag and enables the use of RRC for lossless coding. This results in considerable bit-rate savings for lossless coding.

# Proposed Method

In this proposal if TS flag is TRUE and internal QP value (which is clipped at 4) is less than or equal to 4 then it is inferred that a block is lossless coded. This decision is then used to bypass TSRC for lossless coding and use RRC instead. This results in improved bit-rate savings while satisfying the “objective of lossless coding for free” in VVC without introducing additional syntax elements for lossless. Following spec change shows the approach:

#### 7.3.9.10 Transform unit syntax

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_cbf_luma[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( transform_skip_flag[ x0 ][ y0 ][ 0 ] && ( qP > 4 ) ) |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( transform_skip_flag[ xC ][ yC ][ 1 ] && ( qP > 4 ) ) |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( transform_skip_flag[ xC ][ yC ][ 2 ] && ( qP > 4 ) ) |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| … |  |

# Results

In the results presented below, VTM-7.0 is used as the anchor with “lossless.cfg” config file on top of the CTC configurations. BDPCM is disabled for both camera captured and screen content. As outlined in CE3 description, the experiments were run for 100 frames only for AI, RA and LDB configurations. Additional results with BDPCM enabled for Luma, and for both Luma and Chroma are also provided. Finally, low QP lossy results were also obtained to make sure the proposed method has no impact on the lossy coding performance.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Proposal |  | Anchor | Proposal |  | Anchor | Proposal |  |
| Class A1 | 2.2 | 2.4 | -5.42% | 2.2 | 2.4 | -6.25% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 1.9 | -8.13% |  |  |  |
| Class B | 2.2 | 2.3 | -4.01% | 2.3 | 2.4 | -3.83% | 2.3 | 2.4 | -3.84% |
| Class C | 1.9 | 2.0 | -4.97% | 2.4 | 2.6 | -5.36% | 2.4 | 2.6 | -5.29% |
| Class D | 1.9 | 2.0 | -4.63% | 2.8 | 2.9 | -5.25% | 2.7 | 2.9 | -5.24% |
| Class E | 2.8 | 2.9 | -3.82% |  |  |  | 3.1 | 3.3 | -6.41% |
| Class F | 5.3 | 5.3 | -1.39% | 33.7 | 33.8 | -2.40% | 50.7 | 50.2 | -2.12% |
| TGM | 11.8 | 11.6 | 1.54% | 107.1 | 106.1 | 0.59% | 124.9 | 122.6 | 1.18% |
| Overall | 2.1 | 2.2 | -5.23% | 2.2 | 2.3 | -5.58% | 2.6 | 2.7 | -4.96% |
| Enc Time[%] | 93% |  |  | 99% |  |  | 97% |  |  |
| Dec Time[%] | 107% |  |  | 101% |  |  | 108% |  |  |

Table 1. Flag-free Lossless coding with RRC.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Proposal |  | Anchor | Proposal |  | Anchor | Proposal |  |
| Class A1 | 2.2 | 2.4 | -5.81% | 2.2 | 2.4 | -6.51% |  |  |  |
| Class A2 | 1.6 | 1.8 | -9.99% | 1.7 | 1.9 | -8.40% |  |  |  |
| Class B | 2.2 | 2.3 | -5.58% | 2.3 | 2.5 | -4.30% | 2.3 | 2.4 | -4.33% |
| Class C | 1.9 | 2.1 | -6.41% | 2.4 | 2.6 | -5.79% | 2.4 | 2.6 | -5.74% |
| Class D | 1.9 | 2.1 | -7.50% | 2.8 | 2.9 | -5.77% | 2.7 | 2.9 | -5.63% |
| Class E | 2.8 | 3.0 | -6.93% |  |  |  | 3.1 | 3.4 | -7.29% |
| Class F | 5.3 | 5.7 | -5.74% | 33.7 | 34.9 | -4.37% | 50.7 | 51.2 | -3.70% |
| TGM | 11.8 | 12.0 | -1.59% | 107.1 | 107.6 | -0.71% | 124.9 | 124.0 | 0.16% |
| Overall | 2.1 | 2.3 | -6.76% | 2.2 | 2.4 | -5.96% | 2.6 | 2.7 | -5.54% |
| Enc Time[%] | 104% |  |  | 102% |  |  | 101% |  |  |
| Dec Time[%] | 97% |  |  | 98% |  |  | 108% |  |  |

Table 2. Flag-free Lossless coding with RRC with BDPCM on for Luma.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Proposal |  | Anchor | Proposal |  | Anchor | Proposal |  |
| Class A1 | 2.2 | 2.4 | -6.61% | 2.2 | 2.4 | -7.03% |  |  |  |
| Class A2 | 1.6 | 1.8 | -10.43% | 1.7 | 1.9 | -8.47% |  |  |  |
| Class B | 2.2 | 2.3 | -6.31% | 2.3 | 2.5 | -4.53% | 2.3 | 2.4 | -4.54% |
| Class C | 1.9 | 2.1 | -7.18% | 2.4 | 2.6 | -5.99% | 2.4 | 2.6 | -5.90% |
| Class D | 1.9 | 2.1 | -8.96% | 2.8 | 2.9 | -6.06% | 2.7 | 2.9 | -5.84% |
| Class E | 2.8 | 3.1 | -9.09% |  |  |  | 3.1 | 3.4 | -7.92% |
| Class F | 5.3 | 5.8 | -7.60% | 33.7 | 35.3 | -5.22% | 50.7 | 51.6 | -4.33% |
| TGM | 11.8 | 12.4 | -4.78% | 107.1 | 109.1 | -1.90% | 124.9 | 123.8 | -0.05% |
| Overall | 2.1 | 2.3 | -7.70% | 2.2 | 2.4 | -6.21% | 2.6 | 2.7 | -5.84% |
| Enc Time[%] | 107% |  |  | 104% |  |  | 102% |  |  |
| Dec Time[%] | 93% |  |  | 96% |  |  | 103% |  |  |

Table 3. Flag-free Lossless coding with RRC with BDPCM on for Luma and Chroma.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 99% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 101% | 104% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | 0.00% | 0.00% | 99% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 99% |

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 101% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101% | 101% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 4. AI and RA and LDB Results for low-QP tests.

# Recommendation

It is recommended to adopt the flag-free lossless coding method into VVC.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

- A. Nalci, H. Wang, H. E. Egilmez, M. Coban, M. Karczewicz “AHG18: BDPCM for Lossless,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-P0082, 16th Meeting: Geneva, CH, 1–11 October 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).