| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0109-v1 |
| --- | --- |

| Title: | CE3-related: On Last Position Signaling for Lossless Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Marta Karczewicz, Alican Nalci, Hongtao Wang, Hilmi E. Egilmez, Muhammed Coban 5775 Morehouse Dr., San Diego, CA, 92121, USA | Tel: Email: | +1 (858) 845-6730 analci@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In VTM7, lossless coding is possible via existing syntax with transform skip (TS) residual coding. Core experiment (CE3) is investigating high-level (SPS/PPS) and low-level (CU/TU) alternatives for lossless coding for higher bit-rate savings. In this document, it is proposed to disable last position coding when lossless coding uses regular residual coding (RRC) instead of TS residual coding. Here, 3 variants are discussed with the following bit-rate savings:

- For variant 1, the simulation results show overall bit-rate savings of -0.39% AI, -0.43% RA, and -0.52% LD with Class F: 0.23% AI, 0.13% RA, and 0.19% LD and Class TGM: 1.32% AI, 1.08% RA, and 1.42% LD. Max encoder time 104%, max decoder time 101%.

- For variant 2, the simulation results show overall bit-rate savings of -0.39% AI, -0.43% RA, and -0.52% LD with Class F: -0.02% AI, -0.14% RA, and -0.18% LD and Class TGM: 0.36% AI, 0.34% RA, and 0.69% LD. Max encoder time 95%, max decoder time 88%.

- For variant 3, the simulation results show overall bit-rate savings of -0.39% AI, -0.43% RA, and -0.52% LD with Class F: 0.04% AI, 0.02% RA, and 0.09% LD and Class TGM: 0.80% AI, 0.89% RA, and 1.11% LD. Max encoder time 105%, max decoder time 100%.

# Introduction

In VTM-7.0, lossless coding is possible via already existing syntax with TS enabled both for luma and chroma [1]. This form of lossless coding can only use transform skip residual coding (TSRC) and is unable to switch between TSRC and regular residual coding (RRC). In the 16th JVET Geneva meeting, it was observed that RRC had more coding-gains as compared to doing only TSRC [2] and CE3 was established to investigate bit-rate savings for both residual coding methods for lossless coding.

In this contribution, it is proposed to disable last position signaling if regular residual coding is used for lossless coding. Three variants are presented, 1) always do RRC using a high-level flag (as in CE3-2.3 and CE3-2.4) and disabling last position coding, 2) use the TU level lossless coding flag from CE3-2.5 (subtest 2) to switch between TSRC and RRC and disabling last position coding, 3) use the CU level lossless coding flag from CE3-2.5 (subtest 3) to switch between TSRC and RRC and disabling last position coding.

# Proposed Method

In variant 1, a high-level flag (SPS/PPS) is used to switch between TSRC and RRC as in CE3-2.3 or CE3-2.4. For example, in CE3-2.3 and CE3-2.4 if the (SPS/PPS) config option “ForceRegularCoding=1” is true then RRC is always used for lossless coding. Last position signaling is disabled on top of this if this high-level flag is enabled and the last position location is implicitly set to bottom right coefficient of a coding block. In variant 2, TU level signaling from CE3-2.5 (subtest 2) is enabled and last position signaling is disabled whenever the TU level flag is non-zero. This TU level flag switches the used residual coding method between RRC and TSRC. In variant 3,  CU level signaling from CE3-2.5 (subtest 3) is enabled and last position signaling is disabled whenever the TU level flag is non-zero.

# Results

In variant 1, anchor is the alternative lossless anchor used in CE3 which always does RRC using an SPS level flag. This is the same code as in CE3-2.3 but BDPCM disabled for all sequences. Results for this anchor are versus the original lossless anchor are provided at the end of this document. In variant 2, CE3-2.5 subtest 2 results are used as the anchor. In variant 3, CE3-2.5 subtest 3 results are used as the anchor.

In all tests, “lossless.cfg” config file is used on top of the CTC configurations. BDPCM is disabled for both camera captured and screen content sequences both in the anchor and tests. As outlined in CE3 description, the experiments were run for 100 frames only for AI, RA and LDB configurations.

Proposal 1: Disable Last Position Signaling for Lossless with a high-level flag (e.g. from CE3-2.3)

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | Proposal 1 |  | Anchor | Proposal 1 |  | Anchor | Proposal 1 |  |
| Class A1 | 2.4 | 2.4 | -0.39% | 2.4 | 2.4 | -0.38% |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.34% | 1.9 | 1.9 | -0.33% |  |  |  |
| Class B | 2.3 | 2.3 | -0.41% | 2.4 | 2.4 | -0.47% | 2.4 | 2.4 | -0.46% |
| Class C | 2.0 | 2.0 | -0.38% | 2.6 | 2.6 | -0.50% | 2.6 | 2.6 | -0.51% |
| Class D | 2.0 | 2.0 | -0.36% | 2.9 | 2.9 | -0.56% | 2.9 | 2.9 | -0.57% |
| Class E | 2.9 | 2.9 | -0.44% |  |  |  | 3.3 | 3.4 | -0.62% |
| Class F | 5.3 | 5.3 | 0.23% | 33.8 | 33.5 | 0.13% | 50.2 | 49.6 | 0.19% |
| TGM | 11.6 | 11.4 | 1.32% | 106.1 | 105.1 | 1.08% | 122.6 | 120.9 | 1.42% |
| Overall | 2.2 | 2.3 | -0.39% | 2.3 | 2.4 | -0.43% | 2.7 | 2.7 | -0.52% |
| Enc Time[%] | 104% |  |  | 104% |  |  | 103% |  |  |
| Dec Time[%] | 100% |  |  | 101% |  |  | 98% |  |  |

Table 1. Results after disabling last position signaling for lossless with a high-level flag (SPS/PPS) from CE3-2.3 that controls the residual coding method between RRC and TSRC.

Proposal 2: Disable Last Position Signaling for Lossless on top of CE3-2.5 TU Level Signaling

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | CE3-2.5 (#2) | Proposal 2 |  | CE3-2.5 (#2) | Proposal 2 |  | CE3-2.5 (#2) | Proposal 2 |  |
| Class A1 | 2.4 | 2.4 | -0.39% | 2.4 | 2.4 | -0.38% |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.34% | 1.9 | 1.9 | -0.33% |  |  |  |
| Class B | 2.3 | 2.3 | -0.41% | 2.4 | 2.4 | -0.47% | 2.4 | 2.4 | -0.47% |
| Class C | 2.0 | 2.0 | -0.38% | 2.6 | 2.6 | -0.50% | 2.6 | 2.6 | -0.51% |
| Class D | 2.0 | 2.0 | -0.35% | 2.9 | 2.9 | -0.56% | 2.9 | 2.9 | -0.56% |
| Class E | 2.9 | 2.9 | -0.44% |  |  |  | 3.3 | 3.4 | -0.62% |
| Class F | 5.4 | 5.4 | -0.02% | 34.6 | 34.5 | -0.14% | 51.5 | 51.5 | -0.18% |
| TGM | 12.2 | 12.2 | 0.36% | 111.1 | 110.8 | 0.34% | 129.9 | 128.8 | 0.69% |
| Overall | 2.2 | 2.3 | -0.39% | 2.3 | 2.4 | -0.43% | 2.7 | 2.7 | -0.52% |
| Enc Time[%] | 95% |  |  | 94% |  |  | 94% |  |  |
| Dec Time[%] | 87% |  |  | 88% |  |  | 87% |  |  |

Table 2. Results after disabling last position signaling for lossless with a TU level flag (subtest #2) from CE3-2.5 that controls the residual coding method between RRC and TSRC.

Proposal 3: Disable Last Position Signaling for Lossless on top of CE3-2.5 CU Level Signaling

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | CE3-2.5 (#3) | Proposal 3 |  | CE3-2.5 (#3) | Proposal 3 |  | CE3-2.5 (#3) | Proposal 3 |  |
| Class A1 | 2.4 | 2.4 | -0.39% | 2.4 | 2.4 | -0.38% |  |  |  |
| Class A2 | 1.7 | 1.7 | -0.34% | 1.9 | 1.9 | -0.33% |  |  |  |
| Class B | 2.3 | 2.3 | -0.41% | 2.4 | 2.4 | -0.47% | 2.4 | 2.4 | -0.47% |
| Class C | 2.0 | 2.0 | -0.38% | 2.6 | 2.6 | -0.50% | 2.6 | 2.6 | -0.51% |
| Class D | 2.0 | 2.0 | -0.35% | 2.9 | 2.9 | -0.56% | 2.9 | 2.9 | -0.57% |
| Class E | 2.9 | 2.9 | -0.44% |  |  |  | 3.3 | 3.4 | -0.62% |
| Class F | 5.4 | 5.4 | 0.04% | 34.7 | 34.5 | 0.02% | 51.8 | 51.4 | 0.09% |
| TGM | 12.2 | 12.1 | 0.80% | 111.4 | 110.5 | 0.89% | 130.3 | 128.6 | 1.11% |
| Overall | 2.2 | 2.3 | -0.39% | 2.3 | 2.4 | -0.43% | 2.7 | 2.7 | -0.52% |
| Enc Time[%] | 103% |  |  | 104% |  |  | 105% |  |  |
| Dec Time[%] | 98% |  |  | 100% |  |  | 100% |  |  |

Table 3. Results after disabling last position signaling for lossless with a CU level flag from CE3-2.5 (subtest #3)  that controls the residual coding method between RRC and TSRC.

# Recommendation

It is recommended to disable signaling of last position for lossless coding in VVC. From the results, it appears that disabling last position works best (in variant 3) with TU-level signaling from CE3-2.5 with maximal bit-rate gains and reduced encoder/decoder time.

# Additional Results

Table below shows the results of CE3-2.3 with BDPCM disabled for both natural and screen content versus the lossless anchor used in CE3.

|  | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | Bit-rate savings | Compression Ratio |  | bit-rate savings |
|  | Anchor | CE3-2.3 (BDPCM Off) |  | Anchor | CE3-2.3 (BDPCM Off) |  | Anchor | CE3-2.3 (BDPCM Off) |  |
| Class A1 | 2.2 | 2.4 | -5.42% | 2.2 | 2.4 | -6.25% |  |  |  |
| Class A2 | 1.6 | 1.7 | -8.85% | 1.7 | 1.9 | -8.13% |  |  |  |
| Class B | 2.2 | 2.3 | -4.01% | 2.3 | 2.4 | -3.83% | 2.3 | 2.4 | -3.84% |
| Class C | 1.9 | 2.0 | -4.97% | 2.4 | 2.6 | -5.36% | 2.4 | 2.6 | -5.29% |
| Class D | 1.9 | 2.0 | -4.63% | 2.8 | 2.9 | -5.25% | 2.7 | 2.9 | -5.24% |
| Class E | 2.8 | 2.9 | -3.82% |  |  |  | 3.1 | 3.3 | -6.41% |
| Class F | 5.3 | 5.3 | -1.39% | 33.7 | 33.8 | -2.40% | 50.7 | 50.2 | -2.12% |
| TGM | 11.8 | 11.6 | 1.54% | 107.1 | 106.1 | 0.59% | 124.9 | 122.6 | 1.18% |
| Overall | 2.1 | 2.2 | -5.23% | 2.2 | 2.3 | -5.58% | 2.6 | 2.7 | -4.96% |
| Enc Time[%] | 92% |  |  | 98% |  |  | 96% |  |  |
| Dec Time[%] | 104% |  |  | 100% |  |  | 109% |  |  |

# Spec Changes

Changes are highlighted in yellow. SWITCH_FLAG can be the following depending on the proposal:

- Proposal 1 : sps_force_regular_coding, or pps_force_regular_coding (CE3-2.3 or lossless anchor)

- Proposal 2 : ! tu_tsrc_flag (CE3-2.5)

- Proposal 3 : ! cu_tsrc_flag (CE3-2.5)

#### 7.3.8.11 Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if ( SWITCH_FLAG ) { |  |  |
| LastSignificantCoeffX = ( 1 << log2TbWidth ) - 1 |  |  |
| LastSignificantCoeffY = ( 1 << log2TbHeight ) - 1 |  |  |
| } |  |  |
| else{ |  |  |
| if( log2TbWidth > 0 ) |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| } |  |
| … |  |
| QState = 0 |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |
| startQStateSb = QState |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |
| inferSbDcSigCoeffFlag = 0 |  |
| if( ( i < lastSubBlock ¦ ¦ SWITCH_FLAG ) && ( i > 0 ) ) |  |
| { |  |
| coded_sub_block_flag[ xS ][ yS ] | ae(v) |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| firstSigScanPosSb = numSbCoeff |  |
| lastSigScanPosSb = −1 |  |
| firstPosMode0 = ( i = = lastSubBlock ? lastScanPos : numSbCoeff − 1 ) |  |
| firstPosMode1 = −1 |  |
| for( n = firstPosMode0; n >= 0 && remBinsPass1 >= 4; n− − ) { |  |
| xC = ( xS << log2SbW ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 0 ] |  |
| yC = ( yS << log2SbH ) + DiagScanOrder[ log2SbW ][ log2SbH ][ n ][ 1 ] |  |
| if( coded_sub_block_flag[ xS ][ yS ] && ( n > 0 ¦ ¦ !inferSbDcSigCoeffFlag ) && ( xC != LastSignificantCoeffX ¦ ¦ yC != Last SignificantCoeffY ¦ ¦ SWITCH_FLAG ) ) { |  |
| sig_coeff_flag[ xC ][ yC ] | ae(v) |
| remBinsPass1− − |  |
| if( sig_coeff_flag[ xC ][ yC ] ) |  |
| inferSbDcSigCoeffFlag = 0 |  |
| } |  |
| … |  |

last_sig_coeff_x_prefix specifies the prefix of the column position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_x_prefix shall be in the range of 0 to ( log2ZoTbWidth  <<  1 ) − 1, inclusive.

When last_sig_coeff_x_prefix is not present, it is inferred as follows.

- If SWITCH_FLAG is equal to 1, it is inferred to be ( 1 << log2TbWidth )  - 1.

- Otherwise, it is inferred to be 0.

last_sig_coeff_y_prefix specifies the prefix of the row position of the last significant coefficient in scanning order within a transform block. The values of last_sig_coeff_y_prefix shall be in the range of 0 to ( log2ZoTbHeight  <<  1 ) − 1, inclusive.

When last_sig_coeff_y_prefix is not present, it is inferred as follows.

- If SWITCH_FLAG is equal to 1, it is inferred to be ( 1 << log2TbHeight ) - 1.

- Otherwise, it is inferred to be 0.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7)”, JVET-P2001, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 16th meeting, Geneva, CH, 1-11 Oct. 2019.

- F. Bossen, J. Boyce, X. Li, K. Sühring, V. Seregin, V. Seregin, “JVET common test conditions and software reference configurations for SDR video,” JVET-N1010, ISO/IEC JTC 1/SC 29/WG 11 Joint Video Exploration Team (JVET) 14th Meeting, Geneva, CH, 19–27 Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).