| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0115-v1 |
| --- | --- |

| Title: | AHG9: On AU and picture start detection |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following aspects related to AU and picture start detection:

- Keep not allowing PH repetition within a picture.

- Remove AUD.

- Move slice_pic_order_cnt_lsb from the SH to the PH before the POC related syntax elements.

# Introduction

## A discussion on PH repetition and pic_type in AUD

Currently, PH is not allowed to be repeated within a picture, thus can be relied upon as a good mechanism to detection of the start a picture (i.e., the first VCL NAL unit of a picture).

There are two potential usages of PH repetition within a picture, error resilience and systems usage.

The error resilience aspect is as follows. When allowed to be repeated within a picture, even when PHs are not repeated in the bitstream, the transport encapsulation module can include one PH in each of the packets containing one or more VCL NAL units of a picture, the receiver can just de-pack the NAL units and feed them to the decoder without worrying about repeated PHs. When a subset of the packets containing VCL NAL units of one picture gets lost, the rest can still be decoded, and the picture is error concealed and used for output. However, as agreed earlier, nowadays error concealed pictures are no longer displayed. Therefore, the error resilience purpose is not justifying for allowing PH repetition.

From systems usage point of view, it seems also OK to not allow PH repetition within a picture, as long as PHs are stored in separate file format tracks and separate DASH representations from the slices. For extraction based use cases, this certainly works, as the PH representation and the slice representation(s) can be separately requested. Similarly for merging use cases.

Therefore, it is actually a good design choice not to allow PH repetition within a picture.

As long as we keep not allowing PH repetition within a picture, AUD becomes not useful, as the usage was really to allow easy detection of the start of a picture.

The AUD includes a pic_type syntax element, which has the following semantics:

pic_type indicates that the slice_type values for all slices of the coded pictures in the AU containing the AUD NAL unit are members of the set listed in Table 7 for the given value of pic_type. The value of pic_type shall be equal to 0, 1 or 2 in bitstreams conforming to this version of this Specification. Other values of pic_type are reserved for future use by ITU-T | ISO/IEC. Decoders conforming to this version of this Specification shall ignore reserved values of pic_type.

Table 7 – Interpretation of pic_type

| pic_type | slice_type values that may be present in the AU |
| --- | --- |
| 0 | I |
| 1 | P, I |
| 2 | B, P, I |

Among the three values of pic_type, only the first value is useful for identifying intra-coded pictures in some trick mode usage, the other two values do not seem very useful. Therefore, the indication of an intra-coded picture can be moved to the PH, e.g., as proposed in JVET-Q0116.

## A discussion on POC related PH syntax elements

The PH syntax includes a few POC related syntax elements for which the semantics depend on the POC value of the current picture, while the POC value of the current picture can only be derived based on slice_pic_order_cnt_lsb, which was decided at the previous meeting to be put in the SH for picture start detection. These syntax elements include recovery_poc_cnt and some of the RPL syntax elements like pic_delta_poc_msb_present_flag[ i ][ j ] and pic_delta_poc_msb_cycle_lt[ i ][ j ]. Because of this, the decoder would not be able to use these syntax elements in the decoding process or check related conformance constraints until a slice is received. There was at least one bug ticket reporting such an issue.

Such issue would be resolved if slice_pic_order_cnt_lsb is included in the PH syntax before the POC related syntax elements.

# Proposal

Based on the above discussions, the following aspects are proposed:

- Keep not allowing PH repetition within a picture.

- Remove AUD.

- Move slice_pic_order_cnt_lsb from the SH to the PH before the POC related syntax elements.

# Patent rights declaration(s)

Bytedance Inc. does not have any current or pending patent rights relating to the technology proposed in this contribution (to the extent of the personal awareness of the author).