| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0117-v1 |
| --- | --- |

| Title: | AHG9: Cleanups on parameter sets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang Jizheng Xu Kai Zhang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com xujizheng@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes some cleanup changes on DPS, SPS, PPS, and APS, listed below.

On DPS:

- Add the following constraint: It is required that all DPS NAL units with a particular value of dps_decoding_parameter_set_id in a bitstream shall have the same content.

- Add the following constraint: When present in a bitstream, a DPS NAL unit shall be present in the first AU of the bitstream, and may be present in any AU that has at least one coded slice NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive (such a NAL unit is an IRAP or a GDR VCL NAL unit), and shall not be present in any other AU.

- To make sure that each profile_tier_level( ) syntax structure included in the DPS does not contain sublayer level information, add one more input argument for the PTL syntax structure to control whether there is sublayer level information signalled at all. This argument is 0 (i.e., no sublayer level information) for DPS and 1 for VPS and SPS.

- The following constraint is specified: Each OLS in a CVS in the bitstream shall confirm to at least one of the PTL syntax structures in the DPS (as part of the DPS semantics). Accordingly, a change is also made to the semantics of the PTL syntax structure regarding the OlsInScope: "When a profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope is one or more unidentified OLSs in one or more unidentified CVSs in the bitstream."

- The DPS ID (i.e., dps_decoding_parameter_set_id syntax element) is removed from the DPS syntax, and consequently the reference of the DPS ID in the SPS syntax (sps_decoding_parameter_set_id) is also removed. Effectively, the DPS then becomes a standalone NAL unit instead of being a parameter set. Therefore, the name of DPS can be changed to be "decoding parameters NAL unit". And the above items 1 and 2 apply to decoding parameters NAL units.

On SPS:

- It is proposed to move sps_seq_parameter_set_id to be the first syntax element in the SPS syntax now that it is u(4)-coded, to be aligned with the signalling position of the parameter IDs for all other parameter sets.

- Since sps_joint_cbcr_enabled_flag, which controls a chroma coding tool, is currently unnecessarily signalled when the color format is 4:0:0, it is proposed to be conditioned on "if(ChromaArrayType  !=  0 )".

On PPS: It is proposed to repeat ref_pic_resampling_enabled_flag and condition the picture width and height syntax elements as well as the scaling_window_flag based on the repeated PPS flag.

On APS: It is proposed to add the following rule regarding the APS ID value space: All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id. Consequently, the NOTE saying "Each type of APSs uses a separate value space for adaptation_parameter_set_id" can be removed.

# On DPS

## Issues with the current DPS design

The existing DPS design in VVC has the following issues:

- A DPS applies to the entire bitstream and all SPSs that are referred to by coded pictures in a bitstream shall refer to the same DPS. However, there lacks a constraint the requires that all DPS NAL units in a bitstream shall have the same content.

- In the latest VVC draft text, a DPS can be present in any AU. However, while it is useful to have a DPS at the beginning of the bitstream or in any AU that can be used as a random access point, e.g., AUs that contain IRAP pictures and GDR pictures, the presence of a DPS NAL unit in AUs that do not contain random accessible pictures or slices is useless. A random accessible slice is a slice that has one of the IRAP NAL unit types or has the GDR NAL unit type.

- Since the DPS indicates the highest the capability needed for decoding the entire bitstream, there is no need for a profile_tier_level( ) syntax structure included in the DPS to contain sublayer level information, which is carried by instances of the sublayer_level_idc[ i ] syntax elements.

- When a profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope (i.e., to OLS to which the DPS applies) is specified to be the OLS that includes all layers in the the entire bitstream that refers to the DPS. However, it is possible that there is no OLS that includs all layers in the entire bitstream.

- Since the entire bitstream refers to the same DPS, the DPS ID, which is referenced by SPSs, is actually useless.

## Proposed DPS changes

The address the above issues on DPS, the following is proposed:

- It is required that all DPS NAL units with a particular value of dps_decoding_parameter_set_id in a bitstream shall have the same content.

- The following constraint is specified: When present in a bitstream, a DPS NAL unit shall be present in the first AU of the bitstream, and may be present in any AU that has at least one coded slice NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive (such a NAL unit is an IRAP or a GDR VCL NAL unit), and shall not be present in any other AU.

- To make sure that each profile_tier_level( ) syntax structure included in the DPS does not contain sublayer level information, add one more input argument for the PTL syntax structure to control whether there is sublayer level information signalled at all. This argument is 0 (i.e., no sublayer level information) for DPS and 1 for VPS and SPS.

- The following constraint is specified: Each OLS in a CVS in the bitstream shall confirm to at least one of the PTL syntax structures in the DPS (as part of the DPS semantics). Accordingly, a change is also made to the semantics of the PTL syntax structure regarding the OlsInScope: "When a profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope is one or more unidentified OLSs in one or more unidentified CVSs in the bitstream."

- The DPS ID (i.e., dps_decoding_parameter_set_id syntax element) is removed from the DPS syntax, and consequently the reference of the DPS ID in the SPS syntax (sps_decoding_parameter_set_id) is also removed. Effectively, the DPS then becomes a standalone NAL unit instead of being a parameter set. Therefore, the name of DPS can be changed to be "decoding parameters NAL unit". And the above items 1 and 2 apply to decoding parameters NAL units.

Detailed spec text changes for items 1 to 4 are provided below. If the group agrees with item 5 or is interested in seeing the text before making a decision, we will provide the detailed spec text changes for item 5.

# On SPS

The following aspects are proposed for SPS:

- It is proposed to move sps_seq_parameter_set_id to be the first syntax element in the SPS syntax now that it is u(4)-coded, to be aligned with the signalling position of the parameter IDs for all other parameter sets.

- Since sps_joint_cbcr_enabled_flag, which controls a chroma coding tool, is currently unnecessarily signalled when the color format is 4:0:0, it is proposed to be conditioned on "if(ChromaArrayType  !=  0 )".

# On PPS

It is proposed to repeat ref_pic_resampling_enabled_flag and condition the picture width and height syntax elements as well as the scaling_window_flag based on the repeated PPS flag.

# On APS

It is proposed to add the following rule regarding the APS ID value space:

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id.

Consequently, the NOTE saying "Each type of APSs uses a separate value space for adaptation_parameter_set_id" can be removed.

# Proposed spec text changes

Spec text changes for all the above proposals, except for item 5 of the DPS changes, are provided below.

The proposed text changes are relative to the latest VVC draft text. The most relevant parts that have been added or modified are highlighted in yellow, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts.

#### 7.3.2.1 Decoding parameter set syntax

| decoding_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| dps_decoding_parameter_set_id | u(4) |
| dps_max_sublayers_minus1 | u(3) |
| dps_reserved_zero_5bits | u(5) |
| dps_num_ptls_minus1 | u(4) |
| for( i = 0; i <= dps_num_ptls_minus1; i++ ) |  |
| profile_tier_level( 1, 0, dps_max_sublayers_minus1 ) |  |
| dps_extension_flag | u(1) |
| if( dps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.4.3.1 Decoding parameter set RBSP semantics

A DPS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId equal to 0 or provided through external means.

NOTE 1 – DPS NAL units are required to be available (either in the bitstream or by external means) to the decoding process prior to their being referenced. However, the DPS RBSP contains information that is not necessary for operation of the decoding process specified in clauses 2 through 9 of this Specification.

All DPS NAL units with a particular value of dps_decoding_parameter_set_id in a bitstream shall have the same content.

When present in a bitstream, a DPS NAL unit shall be present in the first AU of the bitstream, and may be present in any AU that has at least one coded slice NAL unit having nal_unit_type in the range of IDR_W_RADL to GDR_NUT, inclusive (such a NAL unit is an IRAP or a GDR VCL NAL unit), and shall not be present in any other AU.

dps_decoding_parameter_set_id identifies the DPS for reference by other syntax elements. The value of dps_decoding_parameter_set_id shall be greater than 0.

dps_max_sublayers_minus1 plus 1 specifies the maximum number of temporal sublayers that may be present in a layer in the bitstream each CVS referring to the DPS. The value of dps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive.

dps_reserved_zero_5bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Other values for dps_reserved_zero_5bits are reserved for future use by ITU-T | ISO/IEC.

dps_num_ptls_minus1 plus 1 specifies the number of profile_tier_level( ) syntax structures in the DPS.

It is a requirement of bitstream conformance that each OLS in a CVS in the bitstream shall conform to at least one of the profile_tier_level( ) syntax structures in the DPS.

NOTE 2 – The DPS does not need to contain optimal PTL information for all the OLSs. For example, when there are OLSs that are of Levels 3, 4, and 5, assuming other PTL information of the OLSs is the same, it is good enough to include only one PTL syntax structure in the DPS that indicates Level 5, as it is correct to say that an OLS conforming to a particular level also conforms to any higher level and it is good enough to indicate the higest decoding capabiltiy needed by the DPS.

dps_extension_flag equal to 0 specifies that no dps_extension_data_flag syntax elements are present in the DPS RBSP syntax structure. dps_extension_flag equal to 1 specifies that there are dps_extension_data_flag syntax elements present in the DPS RBSP syntax structure.

dps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in Annex A. Decoders conforming to this version of this Specification shall ignore all dps_extension_data_flag syntax elements.

#### 7.3.2.2 Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| ... |  |  |
| for( i = 0; i < vps_num_ptls; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], 1, ptl_max_temporal_id[ i ] ) |  |
| ... |  |  |
| } |  |

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| sps_seq_parameter_set_id | u(4) |  |
| sps_decoding_parameter_set_id | u(4) |  |
| sps_video_parameter_set_id | u(4) |  |
| sps_max_sublayers_minus1 | u(3) |  |
| sps_reserved_zero_4bits | u(4) |  |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |  |
| profile_tier_level( 1, 1, sps_max_sublayers_minus1 ) |  |  |
| gdr_enabled_flag | u(1) |  |
| sps_seq_parameter_set_id | u(4) |  |
| chroma_format_idc | u(2) |  |
| if( chroma_format_idc = = 3 ) |  |  |
| separate_colour_plane_flag | u(1) |  |
| sps_ref_pic_resampling_enabled_flag | u(1) |  |
| pic_width_max_in_luma_samples | ue(v) |  |
| pic_height_max_in_luma_samples | ue(v) |  |
| ... | u(2) |  |
| sps_joint_cbcr_enabled_flag | u(1) |  |
| if( ChromaArrayType != 0 ) { |  |  |
| sps_joint_cbcr_enabled_flag | u(1) |  |
| same_qp_table_for_chroma | u(1) |  |
| ... |  |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

An SPS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId equal to 0 or provided through external means.

All SPS NAL units with a particular value of sps_seq_parameter_set_id in a CVS shall have the same content.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements.

SPS NAL units, regardless of the nuh_layer_id values, share the same value space of sps_seq_parameter_set_id.

...

gdr_enabled_flag equal to 1 specifies that GDR pictures may be present in CLVSs referring to the SPS. gdr_enabled_flag equal to 0 specifies that GDR pictures are not present in CLVSs referring to the SPS.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements.

SPS NAL units, regardless of the nuh_layer_id values, share the same value space of sps_seq_parameter_set_id.

chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2.

...

sps_ref_pic_resampling_enabled_flag equal to 1 specifies that reference picture resampling may be applied when decoding coded pictures in the CLVSs referring to the SPS. sps_ref_pic_resampling_enabled_flag equal to 0 specifies that reference picture resampling is not applied when decoding pictures in CLVSs referring to the SPS.

...

subpics_present_flag equal to 1 specifies that subpicture parameters are present in the SPS RBSP syntax. subpics_present_flag equal to 0 specifies that subpicture parameters are not present in the SPS RBSP syntax.

The values of sps_ref_pic_resampling_enabled_flag and subpics_present_flag shall not be both equal to 1.

...

sps_joint_cbcr_enabled_flag equal to 0 specifies that the joint coding of chroma residuals is disabled. sps_joint_cbcr_enabled_flag equal to 1 specifies that the joint coding of chroma residuals is enabled. When not present, the value of sps_joint_cbcr_enabled_flag is inferred to be equal to 0.

...

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| pps_pic_parameter_set_id | ue(v) |  |
| pps_seq_parameter_set_id | u(4) |  |
| pps_ref_pic_resampling_enabled_flag | u(1) |  |
| if( pps_ref_pic_resampling_enabled_flag ) { |  |  |
| pic_width_in_luma_samples | ue(v) |  |
| pic_height_in_luma_samples | ue(v) |  |
| } |  |  |
| conformance_window_flag | u(1) |  |
| if( conformance_window_flag ) { |  |  |
| conf_win_left_offset | ue(v) |  |
| conf_win_right_offset | ue(v) |  |
| conf_win_top_offset | ue(v) |  |
| conf_win_bottom_offset | ue(v) |  |
| } |  |  |
| if( pps_ref_pic_resampling_enabled_flag ) |  |  |
| scaling_window_flag | u(1) |  |
| if( scaling_window_flag ) { |  |  |
| scaling_win_left_offset | ue(v) |  |
| scaling_win_right_offset | ue(v) |  |
| scaling_win_top_offset | ue(v) |  |
| scaling_win_bottom_offset | ue(v) |  |
| } |  |  |
| ... |  |  |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

A PPS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId less than or equal to the TemporalId of the PPS NAL unit or provided through external means.

All PPS NAL units with a particular value of pps_pic_parameter_set_id within a PU shall have the same content.

pps_pic_parameter_set_id identifies the PPS for reference by other syntax elements. The value of pps_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

PPS NAL units, regardless of the nuh_layer_id values, share the same value space of pps_pic_parameter_set_id.

pps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the SPS. The value of pps_seq_parameter_set_id shall be in the range of 0 to 15, inclusive. The value of pps_seq_parameter_set_id shall be the same in all PPSs that are referred to by coded pictures in a CLVS.

pps_ref_pic_resampling_enabled_flag shall be equal to sps_ref_pic_resampling_enabled_flag.

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. When not present, the value of pic_width_in_luma_samples is inferred to be equal to pic_width_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_width_in_luma_samples shall be equal to pic_width_max_in_luma_samples.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. When not present, the value of pic_height_in_luma_samples is inferred to be equal to pic_height_max_in_luma_samples.

When subpics_present_flag is equal to 1 or ref_pic_resampling_enabled_flag equal to 0, the value of pic_height_in_luma_samples shall be equal to pic_height_max_in_luma_samples.

...

scaling_window_flag equal to 1 specifies that the scaling window offset parameters are present in the PPS. scaling_window_flag equal to 0 specifies that the scaling window offset parameters are not present in the PPS. When not present, the value of scaling_window_flag is inferred to be equal to 0.When ref_pic_resampling_enabled_flag is equal to 0, the value of scaling_window_flag shall be equal to 0.

...

#### 7.4.3.5 Adaptation parameter set semantics

Each APS RBSP shall be available to the decoding process prior to it being referenced, included in at least one AU with TemporalId less than or equal to the TemporalId of the coded slice NAL unit that refers it or provided through external means.

All APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

All APS NAL units with a particular value of aps_params_type, regardless of the nuh_layer_id values, share the same value space for adaptation_parameter_set_id. APS NAL units with different values of aps_params_type use separate values spaces for adaptation_parameter_set_id.

adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive.

When aps_params_type is equal to LMCS_APS, the value of adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive.

aps_params_type specifies the type of APS parameters carried in the APS as specified in Table 6.

Table 6 – APS parameters type codes and types of APS parameters

| aps_params_type | Name of aps_params_type | Type of APS parameters |
| --- | --- | --- |
| 0 | ALF_APS | ALF parameters |
| 1 | LMCS_APS | LMCS parameters |
| 2 | SCALING_APS | Scaling list parameters |
| 3..7 | Reserved | Reserved |

NOTE 1 – Each type of APSs uses a separate value space for adaptation_parameter_set_id.

NOTE 2 – An APS NAL unit (with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type) can be shared across pictures, and different slices within a picture can refer to different ALF APSs.

NOTE 3 – A suffix APS NAL unit associated with a particular VCL NAL unit (this VCL NAL unit precedes the suffix APS NAL unit in decoding order) is not for use by the particular VCL NAL unit, but for use by VCL NAL units following the suffix APS NAL unit in decoding order.

aps_extension_flag equal to 0 specifies that no aps_extension_data_flag syntax elements are present in the APS RBSP syntax structure. aps_extension_flag equal to 1 specifies that there are aps_extension_data_flag syntax elements present in the APS RBSP syntax structure.

aps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all aps_extension_data_flag syntax elements.

#### 7.3.3.1 General profile, tier, and level syntax

| profile_tier_level( profileTierPresentFlag, sublayerLevelFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| general_constraint_info( ) |  |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| num_sub_profiles | u(8) |
| for( i = 0; i < num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| if( sublayerLevelFlag ) { |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |
| } |  |

#### 7.4.4.1 General profile, tier, and level semantics

A profile_tier_level( ) syntax structure provides level information and, optionally, profile, tier, sub-profile, and general constraints information.

When a profile_tier_level( ) syntax structure is included in a DPS, the OlsInScope is one or more unidentified OLSs in one or more unidentified CVSs in the bitstream. When the profile_tier_level( ) syntax structure is included in a VPS, the OlsInScope is one or more OLSs specified by the VPS. When the profile_tier_level( ) syntax structure is included in an SPS, the OlsInScope is the OLS that includes only the layer that is the lowest layer among the layers that refer to the SPS, and this lowest layer is an independent layer.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).