| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0119-v1 |
| --- | --- |

| Title: | AHG12: Cleanups on signalling of subpictures, tiles, and rectangular slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following cleanup changes on signalling of subpictures, tiles, and rectangular slices:

- Change the coding of sps_num_subpics_minus1 from u(8) to ue(v), and the value of sps_num_subpics_minus1 is restricted to be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive.

- Condition sps_subpic_id_present_flag on "if( subpics_present_flag )", i.e., the syntax element sps_subpic_id_present_flag is not signalled when subpics_present_flag is equal to 0, and infer the value of sps_subpic_id_present_flag to be equal to 0 when it is not present. For easier understanding, the names of subpics_present_flag and sps_subpic_id_present_flag are suggested to be changed to subpic_info_present_flag and subpic_ids_explicitly_signalled_flag, respectively.

- Remove the signalling of subpicture IDs from the PH syntax.

- When subpicture IDs are indicated to be explicitly signalled, they are signalled either in the SPS or in the PPS, cannot be in both, and cannot be in neither. For easier understanding, the names of sps_subpic_id_signalling_present_flag and pps_subpic_id_signalling_present_flag are suggested to be changed to subpic_ids_in_sps_flag and subpic_ids_in_pps_flag, respectively.

- When subpics_present_flag is equal to 1, the length of subpicture IDs is explicitly signalled in the SPS regardless of whether subpicture IDs are explicitly signalled, and the length of slice_subpic_id is always specified by that length.

- When rect_slice_flag is equal to 1, the length of slice_address is specified to be Max( Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ), 1 ) bits, as opposed to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits.

# On sps_num_subpics_minus1

The coding of sps_num_subpics_minus1 is u(8), which disallows more than 256 subpictures per picture. However, in certain applications, the maximum number of subpictures per picture may need to be greater than 256.

It is therefore proposed to change the coding of sps_num_subpics_minus1 from u(8) to ue(v), and the value of sps_num_subpics_minus1 is restricted to be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive.

Note that the number of subpictures per picture is further restricted by the level limit on the number of slices, and it could be further restricted if needed.

# On subpics_present_flag and sps_subpic_id_present_flag

It is allowed to have subpics_present_flag equal to 0 and sps_subpic_id_present_flag equal to 1. However, this does not makes sense as subpics_present_flag equal to 0 means that the CLVS has no information on subpictures at all.

It is therefore proposed to condition the signalling of sps_subpic_id_present_flag on "if( subpics_present_flag )", i.e., the syntax element sps_subpic_id_present_flag is not signalled when subpics_present_flag is equal to 0, and infer the value of sps_subpic_id_present_flag to be equal to 0 when it is not present.

For easier understanding, the names of subpics_present_flag and sps_subpic_id_present_flag are suggested to be changed to subpic_info_present_flag and subpic_ids_explicitly_signalled_flag, respectively.

# On subpicture IDs in PHs

A list of subpicture IDs may be signalled in a PH, one for each of the subpictures. However, when the list of subpicture IDs is signalled in PHs, and when a subset of the subpictures is extracted from the bitstream, all the PHs would all need to be changed. This is undesirable.

It is therefore proposed that the signalling of subpicture IDs in the PH syntax is removed.

# On subpicture IDs in either SPS or PPS

Currently when subpicture IDs are indicated to be explicitly signalled, by sps_subpic_id_present_flag (suggested above to be changed to subpic_ids_explicitly_signalled_flag) equal to 1, subpicture IDs may be not signalled anywhere. However, subpicture IDs need to be explicitly signalled when so indicated.

It is therefore proposed that, on top of the removal of subpicture IDs in the PH, subpicture IDs are signalled either in the SPS or in the PPS, cannot be in both, and cannot be in neither but when subpicture IDs are indicated to be explicitly signalled.

For easier understanding, the names of sps_subpic_id_signalling_present_flag and pps_subpic_id_signalling_present_flag are suggested to be changed to subpic_ids_in_sps_flag and subpic_ids_in_pps_flag, respectively.

Consequently, the above proposal can be realized by specifying the following constraint: If subpic_ids_explicitly_signalled_flag is equal to 0 or subpic_ids_in_sps_flag is equal to 1, subpic_ids_in_pps_flag shall be equal to 0. Otherwise (subpic_ids_explicitly_signalled_flag is 1 and subpic_ids_in_sps_flag is equal to 0), subpic_ids_in_pps_flag shall be equal to 1.

# On the length of slice_subpic_id

When subpicture IDs are not explicitly signalled, the slice header syntax element slice_subpic_id still needs to be signalled as long as subpics_present_flag is equal to 1, including when sps_num_subpics_minus1 is equal to 0. However, subpicture IDs are not explicitly signalled, the subpicture length is not explicitly signalled, either, and the length of slice_subpic_id is currently specified as Ceil( Log2 ( sps_num_subpics_minus1 + 1 ) ) bits, which has two issues. The first issue is that the length would be 0 bits when sps_num_subpics_minus1 is equal to 0. The second issue is that, when the original bitstream of the CLVS has subpics_present_flag equal to 1 and sps_num_subpics_minus1 greater than 0, when the subpicture ID length is not explicitliy signalled, extraction of a subset of the subpictures would require changing of the slice headers.

Therefore, it is proposed that, when subpics_present_flag is equal to 1, the length of subpicture IDs is explicitly signalled in the SPS regardless of whether subpicture IDs are explicitly signalled, and the length of slice_subpic_id is always specified by that length.

# On the length of slice_address when rect_slice_flag is equal to 1

Whenever rect_slice_flag is equal to 1, the syntax element slice_address is signalled in the slice header and specifies the slice index within the subpicture containing the slice, including when the number of slices within the subpicture (i.e., NumSlicesInSubpic[ SubPicIdx ]) is equal to 1. However, currently, when rect_slice_flag is equal to 1, the length of slice_address is specified to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits, which would be 0 bits when NumSlicesInSubpic[ SubPicIdx ] is equal to 1. This is problematic, as the length of any present syntax elements cannot be 0 bits.

It is therefore proposed that, when rect_slice_flag is equal to 1, the length of slice_address is specified to be Max( Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ), 1 ) bits, as opposed to be Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits.

# Proposed spec text changes

Spec text changes for all the above proposed items are provided below.

The proposed text changes are relative to the latest VVC draft text. The most relevant parts that have been added or modified are highlighted in yellow, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts. Some of the changes that are purely editorial have not been highlighted.

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| sps_log2_ctu_size_minus5 | u(2) |
| subpic_info_present_flag | u(1) |
| if( subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| sps_subpic_id_len_minus1 | ue(v) |
| subpic_ids_explicitly_signalled_flag | u(1) |
| if( subpic_ids_explicitly_signalled_flag ) { |  |
| subpic_ids_in_sps_flag | u(1) |
| if( subpic_ids_in_sps_flag ) |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

...

subpic_info_present_flag equal to 1 specifies that subpicture information is present. subpic_info_present_flag equal to 0 specifies that subpicture information is not present. The values of sps_ref_pic_resampling_enabled_flag and subpic_info_present_flag shall not be both equal to 1.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpic_info_present_flag equal to 1 in the SPSs.

sps_num_subpics_minus1 plus 1 specifies the number of subpictures. The value of sps_num_subpics_minus1 shall be in the range of 0 to Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) * Ceil( pic_height_max_in_luma_samples ÷ CtbSizeY ) − 1, inclusive. Noet that the maximum number of subpictures can be further restricted in the level definitions. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the subpicture index of subpicA is less than that of subpicB, any coded slice NAL unit of subPicA shall precede any coded slice NAL unit of subPicB in decoding order.

- The shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

sps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the explicitly signalled subpicture IDs as well as the slice_subpic_id syntax element in the slice header. The value of sps_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

subpic_ids_explicitly_signalled_flag equal to 1 specifies that a set of subpicture ID, one for each subpicture, is explicitly signalled for each subpicture either in the SPS or the PPS. subpic_ids_explicitly_signalled_flag equal to 0 specifies that no subpicture IDs are explicitly signalled in the SPS or the PPS. When not present, the value of subpic_ids_explicitly_signalled_flag is inferred to be equal to 0.

subpic_ids_in_sps_flag equal to 1 specifies that a subpicture ID for each subpicture is explicitly signalled in the SPS. subpic_ids_in_sps_flag equal to 0 specifies that no subpicture IDs are signalled in the SPS. When not present, the value of subpic_ids_in_sps_flag is inferred to be equal to 0.

sps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits.

...

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| mixed_nalu_types_in_pic_flag | u(1) |
| subpic_ids_in_pps_flag | u(1) |
| if( subpic_ids_in_pps_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| for( j = 0; j < num_slices_in_tile_minus1[ i ]; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| ... |  |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

...

subpic_ids_in_pps_flag equal to 1 specifies that a subpicture ID for each subpicture is explicitly signalled in the PPS. subpic_ids_in_pps_flag equal to 0 specifies that no subpicture IDs are signalled in the PPS. If subpic_ids_explicitly_signalled_flag is 0 or subpic_ids_in_sps_flag is equal to 1, subpic_ids_in_pps_flag shall be equal to 0. Otherwise (subpic_ids_explicitly_signalled_flag is 1 and subpic_ids_in_sps_flag is equal to 0), subpic_ids_in_pps_flag shall be equal to 1.

pps_num_subpics_minus1 shall be equal to sps_num_subpics_minus1.

pps_subpic_id_len_minus1 shall be equal to sps_subpic_id_len_minus1.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

The list SubpicIdList[ i ], for i in the range of 0 to sps_num_subpics_minus1, inclusive, is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	if( subpic_ids_explicitly_signalled_flag )
		SubpicIdList[ i ] = subpic_ids_in_pps_flag ? pps_subpic_id[ i ] : sps_subpic_id[ i ]	(76)
	else
		SubpicIdList[ i ] = i

...

rect_slice_flag equal to 0 specifies that tiles within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that tiles within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When not present, rect_slice_flag is inferred to be equal to 1. When subpic_info_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1.

single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist one or more rectangular slices. When subpic_info_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0. When single_slice_per_subpic_flag is equal to 1, num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1.

...

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| ... |  |
| } |  |

#### 7.4.3.6 Picture header RBSP semantics

...

ph_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PH. ph_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PH.

ph_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element ph_subpic_id[ i ]. The value of pic_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of ph_subpic_id_len_minus1 shall be the same for all PHs that are referred to by coded pictures in a CLVS.

ph_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the ph_subpic_id[ i ] syntax element is ph_subpic_id_len_minus1 + 1 bits.

The list SubpicIdList[ i ] is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	SubpicIdList[ i ] = sps_subpic_id_present_flag ?	(76)
		( sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] :
		( ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ] ) ) : i

...

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( subpic_info_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| ... |  |
| } |  |

#### 7.4.8.1 General slice header semantics

...

slice_subpic_id specifies the subpicture identifier of the subpicture that contains the slice. If slice_subpic_id is present, the value of the variable SubPicIdx is derived to be such that SubpicIdList[ SubPicIdx ] is equal to slice_subpic_id. Otherwise (slice_subpic_id is not present), the variable SubPicIdx is derived to be equal to 0.

The length of slice_subpic_id is sps_subpic_id_len_minus1 + 1 bits.

The length of slice_subpic_id, in bits, is derived as follows:

- If sps_subpic_id_signalling_present_flag is equal to 1, the length of slice_subpic_id is equal to sps_subpic_id_len_minus1 + 1.

- Otherwise, if ph_subpic_id_signalling_present_flag is equal to 1, the length of slice_subpic_id is equal to ph_subpic_id_len_minus1 + 1.

- Otherwise, if pps_subpic_id_signalling_present_flag is equal to 1, the length of slice_subpic_id is equal to pps_subpic_id_len_minus1 + 1.

- Otherwise, the length of slice_subpic_id is equal to Ceil( Log2 ( sps_num_subpics_minus1 + 1 ) ).

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If rect_slice_flag is equal to 0, the following applies:

- The slice address is the raster scan tile index.

- The length of slice_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the subpicture-level slice index of the slice.

- The length of slice_address is Max( Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ), 1 ) bits.

- The value of slice_address shall be in the range of 0 to NumSlicesInSubpic[ SubPicIdx ] − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpic_info_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).