| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0120-v1 |
| --- | --- |

| Title: | AHG12: Control of loop filtering across subpicture/tile/slice boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Li Zhang Ye-Kui Wang Kai Zhang San Diego, CA, USA | Tel: Email: | lizhang.idm@bytedance.com yekui.wang@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following:

- The loop_filter_across_slices_enabled_flag is removed.

- The loop_filter_across_tiles_enabled_flag is made subpicture-specific to only control whether filtering across tile boundaries within a subpicture is on or off and is signalled in the SPS.

- The presence of the loop_filter_across_subpic_enabled_flag[ i ] is conditioned on "if( subpic_treated_as_pic_flag[ i ] )".

# Introduction

In the latest VVC draft text, in-loop filtering across subpicture, tile, and slice boundaries is controlled by the following syntax elements:

- loop_filter_across_subpic_enabled_flag[ i ]: for controlling of deblocking, SAO, and ALF across subpicture boundaries, signalled in the SPS, one for each subpicture.

- loop_filter_across_tiles_enabled_flag: for controlling of deblocking, SAO, and ALF across tile boundaries, signalled in the PPS, just one (thus applicable to all tiles in all pictures referring to the PPS).

- loop_filter_across_slices_enabled_flag: for controlling of deblocking, SAO, and ALF across slice boundaries, signalled in the PPS, just one (thus applicable to all slices in all pictures referring to the PPS).

They work as follows: for any particular boundary, whenever one of the above flags indicates that filtering across the boundary is turned off, regardless of the values of other flags, filtering across the boundary is turned off.

The following issues regarding in-loop filtering across subpicture, tile, and slice boundaries were overserved:

- The different flags for controlling of in-loop filtering across subpicture, tile, and slice boundaries can be somewhat conflicting with each other. For example, it is possible that the encoder chooses to turn on filtering across the boundaries of some independently coded subpictures as it found that otherwise those subpictures being presented together has very annoying artifacts. However, in this case, if the loop_filter_across_tiles_enabled_flag or loop_filter_across_slices_enabled_flag is equal to 0, then filtering across the subpicture boundaries that are also tile or slice boundaries would be turned off anyway.

- The subpicture feature introduced to VVC was mainly for support of region-of-interest and other extraction-based use cases. For AVC, such use cases were supported by using slices or slices groups. For HEVC, these use cases were supported by slices and MCTSs. Therefore, the ability to turn off filtering across slice boundaries was needed in AVC and HEVC. However, in VVC, such need is gone due to the support of subpictures.

- When subpic_treated_as_pic_flag[ i ] is equal to 0 for a subpicture, motion compensation is constrained for coding of the subpicture across all pictures in the CLVS, therefore it is not independently coded and not extractable. In this case, there is no good reason to have loop_filter_across_subpic_enabled_flag[ i ] equal to 0 (i.e., to turn off loop filtering across the boundaries of the subpicture). However, this is allowed by the current design.

# Proposal

To resolve the above issues, this contribution proposes the following summarized changes:

- To address the first and second issues, the loop_filter_across_slices_enabled_flag is removed and the loop_filter_across_tiles_enabled_flag is made subpicture-specific (i.e., signalled once for each subpicture) and only controls whether filtering across tile boundaries within a subpicture (excluding those tile boundaries that are also the boundaries of the subpicture) is on or off.

Furthermore, the subpicture-specific loop_filter_across_tiles_enabled_flag is signalled in the SPS instead of in the PPS. This is because the main (if not only) purpose of turning off filtering across tile boundaries is for reducing data exchange between different processing cores in parallel encoding. Such a purpose usually won't change across pictures, thus while it makes sense to signal tile layout in the PPS for load balancing purpose, signalling of the loop_filter_across_tiles_enabled_flag in the SPS makes more sense, particularly after the flag is made subpicture specific.

- To address the third issue, the presence of the syntax element loop_filter_across_subpic_enabled_flag[ i ] is conditioned on "if( subpic_treated_as_pic_flag[ i ] )". Note that currently when loop_filter_across_subpic_enabled_flag[ i ] is not present, the value of loop_filter_across_subpic_enabled_flag[ i ] is also already inferred to be equal to 1 This inference as it is still works with this syntax condition change.

# Proposed spec text changes

The proposed text changes on top of the latest VVC draft text are provided below. The most relevant parts that have been added or modified are highlighted in yellow, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts. There are some other changes that are editorial in nature and thus not highlighted.

7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| if( subpic_treated_as_pic_flag[ i ] ) |  |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| loop_filter_across_tiles_enabled_flag[ i ] | u(1) |
| ... |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

...

subpics_present_flag equal to 1 specifies that subpicture parameters are present in in the SPS RBSP syntax. subpics_present_flag equal to 0 specifies that subpicture parameters are not present in the SPS RBSP syntax.

NOTE 2 – When a bitstream is the result of a sub-bitstream extraction process and contains only a subset of the subpictures of the input bitstream to the sub-bitstream extraction process, it might be required to set the value of subpics_present_flag equal to 1 in the RBSP of the SPSs.

sps_num_subpics_minus1 plus 1 specifies the number of subpictures. sps_num_subpics_minus1 shall be in the range of 0 to 254. When not present, the value of sps_num_subpics_minus1 is inferred to be equal to 0.

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the subpicture index of subpicA is less than that of subpicB, any coded slice NAL unit of subPicA shall precede any coded slice NAL unit of subPicB in decoding order.

- The shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS.

sps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the SPS. sps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the SPS. When not present, the value of sps_subpic_id_signalling_present_flag is inferred to be equal to 0.

sps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sps_subpic_id[ i ]. The value of sps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

sps_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits. When not present, and when sps_subpic_id_present_flag equal to 0, the value of sps_subpic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to sps_num_subpics_minus1, inclusive

loop_filter_across_tiles_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries within the i-th subpicture in each coded picture in the CLVS. Herein tile boundaries within a subpicture do not includes tile boundaries that are also the boundaries of the subpicture. loop_filter_across_tiles_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries within the i-th subpicture in each coded picture in the CLVS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When there is no tile boundaries within the i-th subpicture, the value of loop_filter_across_tiles_enabled_flag[ i ] is ignored.

...

#### 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| ... | u(1) |
| } |  |

#### 7.4.3.4 Picture parameter set RBSP semantics

...

pps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PPS. pps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PPS. When sps_subpic_id_present_flag is 0 or sps_subpic_id_signalling_present_flag is equal to 1, pps_subpic_id_signalling_present_flag shall be equal to 0.

pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS.

It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to  sps_num_subpics_minus1.

pps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element pps_subpic_id[ i ]. The value of pps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of pps_subpic_id_len_minus1 shall be the same for all PPSs that are referred to by coded pictures in a CLVS.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

no_pic_partition_flag equal to 1 specifies that no picture partitioning applied to each picture referring to the PPS. no_pic_partition_flag equal to 0 specifies each picture referring to the PPS may be partitioned into more than one tile or slice.

It is a requirement of bitstream conformance that the value of no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

It is a requirement of bitstream conformance that the value of no_pic_partition_flag shall not be equal to 1 when the value of sps_num_subpics_minus1 + 1 is greater than 1.

pps_log2_ctu_size_minus5 plus 5 specifies the luma coding tree block size of each CTU. pps_log2_ctu_size_minus5 shall be equal to sps_log2_ctu_size_minus5.

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive. tile_column_width_minus1[ num_exp_tile_columns_minus1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 as specified in clause 6.5.1. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1.

rect_slice_flag equal to 0 specifies that tiles within each slice are in raster scan order and the slice information is not signalled in PPS. rect_slice_flag equal to 1 specifies that tiles within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When not present, rect_slice_flag is inferred to be equal to 1. When subpics_present_flag is equal to 1, the value of rect_slice_flag shall be equal to 1.

single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist one or more rectangular slices. When subpics_present_flag is equal to 0, single_slice_per_subpic_flag shall be equal to 0. When single_slice_per_subpic_flag is equal to 1, num_slices_in_pic_minus1 is inferred to be equal to sps_num_subpics_minus1.

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0.

tile_idx_delta_present_flag equal to 0 specifies that tile_idx_delta values are not present in the PPS and that all rectangular slices in pictures referring to the PPS are specified in raster order according to the process defined in clause 6.5.1. tile_idx_delta_present_flag equal to 1 specifies that tile_idx_delta values may be present in the PPS and that all rectangular slices in pictures referring to the PPS are specified in the order indicated by the values of tile_idx_delta.

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive. When not present, the value of slice_width_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive. When not present, the value of slice_height_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

num_slices_in_tile_minus1[ i ] plus 1 specifies the number of slices in the current tile for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of num_slices_in_tile_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice. When not present, the value of num_slices_in_tile_minus1[ i ] is inferred to be equal to 0.

slice_height_in_ctu_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of CTU rows for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of slice_height_in_ctu_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice.

tile_idx_delta[ i ] specifies the difference in tile index between the i-th rectangular slice and the ( i + 1 )-th rectangular slice. The value of tile_idx_delta[ i ] shall be in the range of –NumTilesInPic + 1 to NumTilesInPic − 1, inclusive. When not present, the value of tile_idx_delta[ i ] is inferred to be equal to 0. In all other cases, the value of tile_idx_delta[ i ] shall not be equal to 0.

loop_filter_across_tiles_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across tile boundaries in pictures referring to the PPS. loop_filter_across_tiles_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across tile boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_tiles_enabled_flag is inferred to be equal to 1.

loop_filter_across_slices_enabled_flag equal to 1 specifies that in-loop filtering operations may be performed across slice boundaries in pictures referring to the PPS. loop_filter_across_slice_enabled_flag equal to 0 specifies that in-loop filtering operations are not performed across slice boundaries in pictures referring to the PPS. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of loop_filter_across_slices_enabled_flag is inferred to be equal to 0.

...

### 8.8.3 Deblocking filter process

#### 8.8.3.1 General

...

The deblocking filter process is applied to all coding subblock edges and transform block edges of a picture, except the following types of edges:

- Edges that are at the boundary of the picture,

- Edges that coincide with the boundaries of a subpicture for which loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0,

- Edges that coincide with the virtual boundaries of the picture when VirtualBoundariesDisabledFlag is equal to 1,

- Edges that coincide with tile boundaries within a subpicture for which when loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0,

- Edges that coincide with slice boundaries when loop_filter_across_slices_enabled_flag is equal to 0,

- Edges that coincide with upper or left boundaries of slices with slice_deblocking_filter_disabled_flag equal to 1,

- Edges within slices with slice_deblocking_filter_disabled_flag equal to 1,

- Edges that do not correspond to 4x4 sample grid boundaries of the luma component,

- Edges that do not correspond to 8x8 sample grid boundaries of the chroma component,

- Edges within the luma component for which both sides of the edge have intra_bdpcm_luma_flag equal to 1,

- Edges within the chroma components for which both sides of the edge have intra_bdpcm_chroma_flag equal to 1,

- Edges of chroma subblocks that are not edges of the associated transform unit.

...

#### 8.8.3.2 Deblocking filter process for one direction

...

For each coding unit and each coding block per colour component of a coding unit indicated by the colour component index cIdx ranging from firstCompIdx to lastCompIdx, inclusive, with coding block width nCbW, coding block height nCbH and location of top-left sample of the coding block ( xCb, yCb ), when cIdx is equal to 0, or when cIdx is not equal to 0 and edgeType is equal to EDGE_VER and xCb % 8 is equal 0, or when cIdx is not equal to 0 and edgeType is equal to EDGE_HOR and yCb % 8 is equal to 0, the edges are filtered by the following ordered steps:

- The variable filterEdgeFlag is derived as follows:

- If edgeType is equal to EDGE_VER and one or more of the following conditions are true, filterEdgeFlag is set equal to 0:

- The left boundary of the current coding block is the left boundary of the picture.

- The left boundary of the current coding block is the left or right boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The left boundary of the current coding block is the left boundary of the tile but not a subpicture boundary and loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0.

- The left boundary of the current coding block is the left boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The left boundary of the current coding block is one of the vertical virtual boundaries of the picture and VirtualBoundariesDisabledFlag is equal to 1.

- Otherwise, if edgeType is equal to EDGE_HOR and one or more of the following conditions are true, the variable filterEdgeFlag is set equal to 0:

- The top boundary of the current luma coding block is the top boundary of the picture.

- The top boundary of the current coding block is the top or bottom boundary of the subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

- The top boundary of the current coding block is the top boundary of the tile but not a subpicture boundary and loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0.

- The top boundary of the current coding block is the top boundary of the slice and loop_filter_across_slices_enabled_flag is equal to 0.

- The top boundary of the current coding block is one of the horizontal virtual boundaries of the picture and VirtualBoundariesDisabledFlag is equal to 1.

- Otherwise, filterEdgeFlag is set equal to 1.

- ...

...

### 8.8.4 Sample adaptive offset process

#### 8.8.4.2 CTB modification process

...

For all sample locations ( xSi, ySj ) and ( xYi, yYj ) with i = 0..nCtbSw − 1 and j = 0..nCtbSh − 1, the following applies:

–	If one or more of the following conditions are true, saoPicture[ xSi ][ ySj ] is not modified:

- SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 0.

- VirtualBoundariesDisabledFlag is equal to 1 and xSj is equal to ( ( VirtualBoundariesPosX[ n ] / scaleWidth ) − 1 ) for any n = 0..VirtualBoundariesNumVer − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 1.

- VirtualBoundariesDisabledFlag is equal to 1 and xSj is equal to ( VirtualBoundariesPosX[ n ] / scaleWidth ) for any n = 0..VirtualBoundariesNumVer − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 1.

- VirtualBoundariesDisabledFlag is equal to 1 and ySj is equal to ( ( VirtualBoundariesPosY[ n ] / scaleHeight ) − 1 ) for any n = 0..VirtualBoundariesNumHor − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 0.

- VirtualBoundariesDisabledFlag is equal to 1 and ySj is equal to ( VirtualBoundariesPosY[ n ] / scaleHeight ) for any n = 0..VirtualBoundariesNumHor − 1 and SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2 and SaoEoClass[ cIdx ][ rx ][ ry ] is not equal to 0.

–	Otherwise, if SaoTypeIdx[ cIdx ][ rx ][ ry ] is equal to 2, the following ordered steps apply:

- The values of hPos[ k ] and vPos[ k ] for k = 0..1 are specified in Table 42 based on SaoEoClass[ cIdx ][ rx ][ ry ].

- The variable edgeIdx is derived as follows:

- The modified sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) are derived as follows:

( xSik′, ySjk′ ) = ( xSi + hPos[ k ], ySj + vPos[ k ] )	(1404)

( xYik′, yYjk′ ) = ( cIdx  = =  0 ) ? ( xSik′, ySjk′ ) : ( xSik′ * SubWidthC, ySjk′ * SubHeightC )	(1405)

- If one or more of the following conditions for all sample locations ( xSik′, ySjk′ ) and ( xYik′, yYjk′ ) with k = 0..1 are true, edgeIdx is set equal to 0:

- The sample at location ( xSik′, ySjk′ ) is outside the picture boundaries.

- The sample at location ( xSik′, ySjk′ ) belongs to a different subpicture and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] for the subpicture to which the sample recPicture[ xSi ][ ySj ] belongs to is equal to 0.

- loop_filter_across_slices_enabled_flag is equal to 0 and the sample at location ( xSik′, ySjk′ ) belongs to a different slice.

- loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0 and the sample at location ( xSik′, ySjk′ ) belongs to a different tile within the same subpicture.

- Otherwise, edgeIdx is derived as follows:

- The following applies:

edgeIdx = 2 + Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 0 ] ][ ySj + vPos[ 0 ] ] ) +
		Sign( recPicture[ xSi ][ ySj ] − recPicture[ xSi + hPos[ 1 ] ][ ySj + vPos[ 1 ] ] )	(1406)

- When edgeIdx is equal to 0, 1, or 2, edgeIdx is modified as follows:

edgeIdx = ( edgeIdx  = =  2 ) ? 0 : ( edgeIdx + 1 )	(1407)

- The modified picture sample array saoPicture[ xSi ][ ySj ] is derived as follows:

saoPicture[ xSi ][ ySj ] = Clip3( 0, ( 1  <<  BitDepth ) − 1, recPicture[ xSi ][ ySj ] +
		SaoOffsetVal[ cIdx ][ rx ][ ry ][ edgeIdx ] )	(1408)

...

### 8.8.5 Adaptive loop filter process

#### 8.8.5.5 ALF boundary position derivation process

...

The variable clipTopPos is modified as follows:

  - If y − ( CtbSizeY − 4 ) is greater than or equal to 0, the variable clipTopPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if VirtualBoundariesDisabledFlag is equal to 1, and yCtb + y − VirtualBoundariesPosY[ n ] is greater than or equal to 0 and less than 3 for any n = 0..VirtualBoundariesNumHor − 1, the following applies:

clipTopPos = VirtualBoundariesPosY[ n ]		(1468)

  - Otherwise, if y is less than 3 and one or more of the following conditions are true, the variable clipTopPos is set equal to yCtb:

    - The top boundary of the current coding tree block is the top boundary of the tile but not a subpicture boundary, and loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0.

    - The top boundary of the current coding tree block is the top boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The top boundary of the current coding tree block is the top boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipBottomPos is modified as follows:

  - If VirtualBoundariesDisabledFlag is equal to 1, VirtualBoundariesPosY[ n ] is not equal to pic_height_in_luma_samples − 1 or 0, and VirtualBoundariesPosY[ n ] − yCtb − y is greater than 0 and less than 5 for any n = 0..VirtualBoundariesNumHor − 1, the following applies:

clipBottomPos = VirtualBoundariesPosY[ n ]		(1469)

  - Otherwise, if CtbSizeY − 4 − y is greater than 0 and is less than 5, the variable clipBottomPos is set equal to yCtb + CtbSizeY − 4.

  - Otherwise, if CtbSizeY − y is less than 5, and one or more of the following conditions are true, the variable clipBottomPos is set equal to yCtb + CtbSizeY:

    - The bottom boundary of the current coding tree block is the bottom boundary of the tile but not a subpicture boundary, and loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0.

    - The bottom boundary of the current coding tree block is the bottom boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The bottom boundary of the current coding tree block is the bottom boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipLeftPos is modified as follows:

  - If VirtualBoundariesDisabledFlag is equal to 1, and xCtb + x − VirtualBoundariesPosX[ n ] is greater than or equal to 0 and less than 3 for any n = 0..VirtualBoundariesNumVer − 1, the following applies:

clipLeftPos = VirtualBoundariesPosX[ n ]		(1470)

  - Otherwise, if x is less than 3, and one or more of the following conditions are true, the variable clipLeftPos is set equal to xCtb:

    - The left boundary of the current coding tree block is the left boundary of the tile but not a subpicture boundary, and loop_filter_across_tiles_enabled_flag[ SubPicIdx ] is equal to 0.

    - The left boundary of the current coding tree block is the left boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The left boundary of the current coding tree block is the left boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipRightPos is modified as follows:

  - If VirtualBoundariesDisabledFlag is equal to 1, and VirtualBoundariesPosX[ n ]− xCtb − x is greater than 0 and less than 5 for any n = 0..VirtualBoundariesNumVer − 1, the following applies:

clipRightPos = VirtualBoundariesPosX[ n ]		(1471)

  - Otherwise, if CtbSizeY − x is less than 5, and one or more of the following conditions are true, the variable clipRightPos is set equal to xCtb + CtbSizeY:

    - The right boundary of the current coding tree block is the right boundary of the tile but not a subpicture boundary, and loop_filter_across_tiless_enabled_flag[ SubPicIdx ] is equal to 0.

    - The right boundary of the current coding tree block is the right boundary of the slice, and loop_filter_across_slices_enabled_flag is equal to 0.

    - The right boundary of the current coding tree block is the right boundary of the subpicture, and loop_filter_across_subpic_enabled_flag[ SubPicIdx ] is equal to 0.

The variable clipTopLeftFlag and clipBotRightFlag are modified as following:

  - If the coding tree block covering the luma position ( xCtb, yCtb ) and the coding tree block covering the luma position ( xCtb − CtbSizeY, yCtb − CtbSizeY) belong to different slices, and loop_filter_across_slices_enabled_flag is equal to 0, clipTopLeftFlag is set equal to 1.

  - If the coding tree block covering the luma position ( xCtb, yCtb ) and the coding tree block covering the luma position ( xCtb + CtbSizeY, yCtb + CtbSizeY) belong to different slices, and loop_filter_across_slices_enabled_flag is equal to 0, clipBotRightFlag is set equal to 1.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).