| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0121-v1 |
| --- | --- |

| Title: | Suggeted spec changes of JVET-Q0121 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu 8910 University Center Ln, San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution presents the suggested text changes for the extension of Tc and Beta settings used in the deblocking filter process.

# Text changes

The following text modifications are based on JVET-P2001-vE, with the changes marked in grey.

pic_beta_offset_div2 and pic_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the slices associated with the PH. The values of pic_beta_offset_div2 and pic_tc_offset_div2 shall both be in the range of −6 -12 to 12 6, inclusive. When not present, the values of pic_beta_offset_div2 and pic_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

slice_beta_offset_div2 and slice_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −6 -12 to 12 6, inclusive. When not present, the values of slice_beta_offset_div2 and slice_tc_offset_div2 are inferred to be equal to pic_beta_offset_div2 and pic_tc_offset_div2, respectively.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).