| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0121-v1 |
| --- | --- |

| Title: | AHG9: On deblocking control parameters |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jizheng Xu, Ye-Kui Wang, Li Zhang, Weijia Zhu, Kai Zhang, Hongbin Liu 8910 University Center Ln, San Diego, CA 92122, USA | Tel: Email: | +1-619-436-7216 xujizheng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to make three modifications to the current VVC draft text: 1) to remove picture header deblocking control to simplify the controlling logic; 2) to add deblocking strength control for Cb and Cr components; 3) to use accumulated deblocking strength control instead of using the overriding mechanism.

# Introduction

In the current draft, there are three levels control on deblocking strength control, shown in figure 1.

Figure 1. Deblocking strength controlling in the current draft.

The controlling logic is complicated, which should be simplified.

# Proposed methods

Three aspects are proposed in this document.

## Deblocking controlling logic simplification

It is proposed to simplify the logic from what is shown in figure 1 to figure 2. That is to remove related syntax elements in picture header but preserve those in PPS and slice header, which is the same as that in HEVC.

Figure 2. Proposed controlling logic.

## Deblocking strength control for different color components

Currently, filtering parameters for both SAO and ALF both separately controlled for luma and chroma components. This is also the case for QP, which is related to deblocking. It is therefore proposed to add beta_offset_div2 and tc_offset_div2 for Cb and Cr components to control the strength of different colour components.

## Offsets accumulations at different levels

It is proposed to accumulate beta_offset_div2 and tc_offset_div2 at different levels (PPS and SH). The proposed aspect does not the related syntax and the syntax element value ranges, while the represented offset range can be enlarged.

For example, with the current overriding mechanism, the final range of both beta_offset_div2 and tc_offset_div2 is [-6, 6]. While if two levels of syntax elements (without touching their current defined ranges) is accumulated, the final range is [-12, 12].

# Text changes

The following text modifications are based on JVET-P2001-vE, with the changes marked in grey.

## Aspect 1

7.3.2.6	Picture header RBSP syntax

| if( !pic_dep_quant_enabled_flag ) |  |
| --- | --- |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| pic_lmcs_enabled_flag | u(1) |

## 7.3.7.1 General slice header syntax

| if( deblocking_filter_override_enabled_flag && !pic_deblocking_filter_override_present_flag ) |  |
| --- | --- |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |

## Aspect 2

The modification is on top of aspect 1.

#### 7.3.2.4 Picture parameter set RBSP syntax

| if( deblocking_filter_control_present_flag ) { |  |
| --- | --- |
| deblocking_filter_override_enabled_flag | u(1) |
| pps_deblocking_filter_disabled_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |
| pps_beta_offset_div2 | se(v) |
| pps_tc_offset_div2 | se(v) |
| pps_cb_beta_offset_div2 | se(v) |
| pps_cb_tc_offset_div2 | se(v) |
| pps_cr_beta_offset_div2 | se(v) |
| pps_cr_tc_offset_div2 | se(v) |
| } |  |

#### 7.3.7.1 General slice header syntax

| if( slice_deblocking_filter_override_flag ) { |  |
| --- | --- |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| slice_cb_beta_offset_div2 | se(v) |
| slice_cb_tc_offset_div2 | se(v) |
| slice_cr_beta_offset_div2 | se(v) |
| slice_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |

7.4.3.4 Picture parameter set RBSP semantics

pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to Cb component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the slice headers of the slices referring to the PPS. The values of pps_beta_offset_div2 and pps_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the value of pps_beta_offset_div2 and pps_tc_offset_div2 are inferred to be equal to 0.

pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to Cr component for slices referring to the PPS, unless the default deblocking parameter offsets are overridden by the deblocking parameter offsets present in the slice headers of the slices referring to the PPS. The values of pps_beta_offset_div2 and pps_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the value of pps_beta_offset_div2 and pps_tc_offset_div2 are inferred to be equal to 0.

7.4.8.1 General slice header semantics

slice_cb_beta_offset_div2 and slice_ cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to Cb component for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the values of slice_beta_offset_div2 and slice_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to Cb component for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the values of slice_beta_offset_div2 and slice_tc_offset_div2 are inferred to be equal to pps_beta_offset_div2 and pps_tc_offset_div2, respectively.

8.8.3.6.3 Decision process for chroma block edges

…

The value of the variable β′ is determined as specified in Table 41 based on the quantization parameter Q derived as follows:

Q = Clip3( 0, 63, QpC + ( slice_beta_offset_div2  <<  1 ) )	(1322)

  - If cIdx is equal to 1

Q = Clip3( 0, 63, QpC + ( slice_cb_beta_offset_div2  <<  1 ) )

  - Otherwise

Q = Clip3( 0, 63, QpC + ( slice_cr_beta_offset_div2  <<  1 ) )

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as follows:

β = β′ * ( 1  <<  ( BitDepth − 8 ) )	(1323)

The value of the variable tC′ is determined as specified in Table 41 based on the chroma quantization parameter Q derived as follows:

Q = Clip3( 0, 65, QpC + 2 * ( bS − 1 ) + ( slice_tc_offset_div2  <<  1 ) )	(1324)

  - If cIdx is equal to 1

Q = Clip3( 0, 65, QpC + 2 * ( bS − 1 ) + ( slice_cb_tc_offset_div2  <<  1 ) )

  - Otherwise

Q = Clip3( 0, 65, QpC + 2 * ( bS − 1 ) + ( slice_cr_tc_offset_div2  <<  1 ) )

…

## Aspect 3

The modification is on top of aspect 2.

7.4.3.4 Picture parameter set RBSP semantics

pps_cb_beta_offset_div2 and pps_cb_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to Cb component for the current PPS. The values of pps_beta_offset_div2 and pps_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the value of pps_beta_offset_div2 and pps_tc_offset_div2 are inferred to be equal to 0.

pps_cr_beta_offset_div2 and pps_cr_tc_offset_div2 specify the default deblocking parameter offsets for β and tC (divided by 2) that are applied to Cr component for the current PPS. The values of pps_beta_offset_div2 and pps_tc_offset_div2 shall both be in the range of −6 to 6, inclusive. When not present, the value of pps_beta_offset_div2 and pps_tc_offset_div2 are inferred to be equal to 0.

7.4.8.1 General slice header semantics

slice_cb_beta_offset_div2 and slice_ cb_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to Cb component for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −6 to 6, inclusive.

slice_cr_beta_offset_div2 and slice_cr_tc_offset_div2 specify the deblocking parameter offsets for β and tC (divided by 2) that are applied to Cb component for the current slice. The values of slice_beta_offset_div2 and slice_tc_offset_div2 shall both be in the range of −6 to 6, inclusive.

8.8.3.6.1 Decision process for luma block edges

…

The value of the variable β′ is determined as specified in Table 41 based on the quantization parameter Q derived as follows:

Q = Clip3( 0, 63, qP + ( slice_beta_offset_div2  <<  1 ) )	(1262)

Q = Clip3( 0, 63, qP + ( ( pps_beta_offset_div2 + slice_beta_offset_div2 ) <<  1 ) )	(1262)

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as follows:

β = β′ * ( 1  <<  ( BitDepth − 8 ) )	(1263)

The value of the variable tC′ is determined as specified in Table 41 based on the quantization parameter Q derived as follows:

Q = Clip3( 0, 65, qP + 2 * ( bS − 1 ) + ( slice_tc_offset_div2  <<  1 ) )	(1264)

Q = Clip3( 0, 65, qP + 2 * ( bS − 1 ) + ( ( pps_tc_offset_div2 + slice_tc_offset_div2 ) <<  1 ) )	(1264)

…

8.8.3.6.3 Decision process for chroma block edges

…

The value of the variable β′ is determined as specified in Table 41 based on the quantization parameter Q derived as follows:

Q = Clip3( 0, 63, QpC + ( slice_beta_offset_div2  <<  1 ) )	(1322)

  - If cIdx is equal to 1

Q = Clip3( 0, 63, QpC + ( ( pps_cb_beta_offset_div2 + slice_cb_beta_offset_div2 )  <<  1 ) )

  - Otherwise

Q = Clip3( 0, 63, QpC + ( ( pps_cr_beta_offset_div2 + slice_cr_beta_offset_div2 )  <<  1 ) )

where slice_beta_offset_div2 is the value of the syntax element slice_beta_offset_div2 for the slice that contains sample q0,0.

The variable β is derived as follows:

β = β′ * ( 1  <<  ( BitDepth − 8 ) )	(1323)

The value of the variable tC′ is determined as specified in Table 41 based on the chroma quantization parameter Q derived as follows:

Q = Clip3( 0, 65, QpC + 2 * ( bS − 1 ) + ( slice_tc_offset_div2  <<  1 ) )	(1324)

  - If cIdx is equal to 1

Q = Clip3( 0, 65, QpC + 2 * ( bS − 1 ) + ( ( pps_cb_tc_offset_div2 + slice_cb_tc_offset_div2 )  <<  1 ) )

  - Otherwise

Q = Clip3( 0, 65, QpC + 2 * ( bS − 1 ) + ( ( pps_cr_tc_offset_div2 + slice_cr_tc_offset_div2 ) <<  1 ) )

…

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).