| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0122-v1 |
| --- | --- |

| Title: | AHG9/AHG8: Some HRD cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang San Diego, CA, USA | Tel: Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following cleanup changes on HRD:

- The vps_general_hrd_params_present_flag is not signalled when vps_general_hrd_params_present_flag is equal to 1, and when not present, the value of vps_general_hrd_params_present_flag is inferred to be equal to 0. Furthermore, the syntax condition for num_ols_hrd_params_minus1, i.e., "if( TotalNumOlss > 1 )", is removed.

- The HRD parameters for OLSs containing only one layer are only signalled in SPSs, not in the VPS.

# On vps_general_hrd_params_present_flag

The value of vps_general_hrd_params_present_flag can be equal to 1 when each_layer_is_an_ols_flag is equal to 1. However, when each_layer_is_an_ols_flag is equal to 1, the HRD parameters are only signalled in the SPSs, therefore the value of vps_general_hrd_params_present_flag shall not be equal to 1.

It is therefore proposed that vps_general_hrd_params_present_flag is not signalled when vps_general_hrd_params_present_flag is equal to 1, and when not present, the value of vps_general_hrd_params_present_flag is inferred to be equal to 0.

Furthermore, the syntax condition for num_ols_hrd_params_minus1, i.e., "if( TotalNumOlss > 1 )", is removed, as it is not needed. This is because when TotalNumOlss is equal to 1, the value of each_layer_is_an_ols_flag would be equal to 1 and then the value of vps_general_hrd_params_present_flag would equal to 0 then no HRD parameters would be signalled in the VPS at all.

# On HRD parameters for OLSs containing only one layer

The HRD parameters for OLSs containing only one layer are signalled in both the VPS and the SPSs. However, it is not useful to repeat the HRD parameters in the VPS for OLSs containing only one layer.

Therefore, it is proposed that the HRD parameters for OLSs containing only one layer are only signalled in SPSs, not in the VPS.

# Proposed spec text changes

Spec text changes for all the above proposed items are provided below.

The proposed text changes are relative to the latest VVC draft text. The most relevant parts that have been added or modified are highlighted in yellow, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts. Some of the changes that are purely editorial have not been highlighted.

#### 7.3.2.2 Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( !each_layer_is_an_ols_flag ) |  |
| vps_general_hrd_params_present_flag | u(1) |
| if( vps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_cpb_params_present_flag | u(1) |
| if( TotalNumOlss > 1 ) |  |
| num_ols_hrd_params_minus1 | ue(v) |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| hrd_max_tid[ i ] | u(3) |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |
| } |  |
| if( num_ols_hrd_params_minus1 > 0 ) |  |
| for( i = 1; i < TotalNumOlss && NumLayersInOls[ i ] > 1; i++ ) |  |
| ols_hrd_idx[ i ] | ue(v) |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

#### 7.4.3.2 Video parameter set RBSP semantics

...

vps_general_hrd_params_present_flag equal to 1 specifies that the syntax structure general_hrd_parameters( ) and other HRD parameters are present in the VPS RBSP syntax structure. vps_general_hrd_params_present_flag equal to 0 specifies that the syntax structure general_hrd_parameters( ) and other HRD parameters are not present in the VPS RBSP syntax structure. When not present, the value of vps_general_hrd_params_present_flag is inferred to be equal to 0.

vps_sublayer_cpb_params_present_flag equal to 1 specifies that the i-th ols_hrd_parameters( ) syntax structure in the VPS contains HRD parameters for the sublayer representations with TemporalId in the range of 0 to hrd_max_tid[ i ], inclusive. vps_sublayer_cpb_params_present_flag equal to 0 specifies that the i-th ols_hrd_parameters( ) syntax structure in the VPS contains HRD parameters for the sublayer representation with TemporalId equal to hrd_max_tid[ i ] only. When vps_max_sublayers_minus1 is equal to 0, the value of vps_sublayer_cpb_params_present_flag is inferred to be equal to 0.

When vps_sublayer_cpb_params_present_flag is equal to 0, the HRD parameters for the sublayer representations with TemporalId in the range of 0 to hrd_max_tid[ i ] − 1, inclusive, are inferred to be the same as that for the sublayer representation with TemporalId equal to hrd_max_tid[ i ]. These include the HRD parameters starting from the fixed_pic_rate_general_flag[ i ] syntax element till the sublayer_hrd_parameters( i ) syntax structure immediately under the condition "if( general_vcl_hrd_params_present_flag )" in the ols_hrd_parameters syntax structure.

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the general_hrd_parameters( ) syntax structure when vps_general_hrd_params_present_flag is equal to 1. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to 63, inclusive. When TotalNumOlss is equal to 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0.

hrd_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure. The value of hrd_max_tid[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of hrd_max_tid[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of hrd_max_tid[ i ] is inferred to be equal to vps_max_sublayers_minus1.

ols_hrd_idx[ i ] specifies the index, to the list of ols_hrd_parameters( ) syntax structures in the VPS, of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive. When NumLayersInOls[ i ] is greater than 1 and num_ols_hrd_params_minus1 is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

vps_extension_flag equal to 0 specifies that no vps_extension_data_flag syntax elements are present in the VPS RBSP syntax structure. vps_extension_flag equal to 1 specifies that there are vps_extension_data_flag syntax elements present in the VPS RBSP syntax structure.

vps_extension_data_flag may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all vps_extension_data_flag syntax elements.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).