| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0125-v1 |
| --- | --- |

| Title: | CE5-related: JC-CCALF with Alternative Filter Weight |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Che-Wei Kuo Jingya Li Chong Soon Lim | Tel: Email: | +65-6550-5423 chewei.kuo@sg.panasonic.com jingya.li@sg.panasonic.com chongsoon.lim@sg.panasoni.com |  |
| Source: | Panasonic Corporation |  |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to use individual JC-CCALF filter weight for each alternative filter. Specifically, instead of using various weight values as in JC-CCALF, only sign changes are applied for 4 alternative filters, e.g., +--+. In decoder, the current CTU selects the filter weight according to the parsed alternative filter index. Compared to the JC-CCALF in CE5-3.2, coding gain improvement and negligible runtime change are observed.

Simulation results with joint YUV performance (AHG13 8:1:1 metric) are summarized as follows.

|  | All Intra |  |  |  | Random Access |  |  |  | Low Delay B |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | YUV | Y | U | V | YUV | Y | U | V | YUV |
| CCALF (CE5 anchor) | 0.17% | -8.31% | -7.06% | -1.40% | 0.14% | -10.14% | -9.82% | -1.88% | 0.12% | -13.00% | -9.48% | -2.15% |
| JC-CCALF (CE5-3.2) | 0.10% | -5.84% | -4.87% | -0.99% | 0.16% | -8.63% | -8.27% | -1.56% | 0.12% | -12.71% | -9.47% | -2.12% |
| alternative filter weight | 0.10% | -6.65% | -5.56% | -1.14% | 0.15% | -9.39% | -9.16% | -1.74% | 0.11% | -13.03% | -10.11% | -2.22% |

# Introduction

JC-CCALF is proposed in JVET-P0372 and JVET-Q0229. It halves the CCALF filtering operation by reusing the Cb refinement value for Cr, with an additional weight multiplication in Cr refinement value derivation. JVET-Q0124 shows the slice level weight candidates can be reduced to ±1, so the filtering process is further simplified. However, using alternative filters for CCALF is proposed in JVET-P1008 and JVET-Q0058, to adapt different content in the same slice. It is noted that JC-CCALF weight is essentially part of filter coefficients, and hence alternative weights also need to be included in the alternative filter design.

# Proposal

In this contribution, it is proposed to use individual JC-CCALF filter weight for each alternative filter. A filter weight map is signaled in the slice header to indicate the 4 filter weights used for 4 alternative filters. In decoder, the current CTU selects the filter weight according to the parsed alternative filter index thus no further signaling overhead is required. The implementation is on top of JVET-Q0124 Method 1 (weight candidates reduced to ±1), so the map only includes sign flags, e.g., +--+.

In the CE5 CCALF anchor encoder, an alternative filter coefficients set is trained using a partial frame region and applied to the whole frame CTUs to select the best filter index. The proposed sign weight for each alternative filter is taken as part of filter coefficients and incorporated into the training and rate distortion decision process, to better adapt the filter set localization.

# Simulation results

Simulation results are reported as below. The JVET common test conditions defined in [1] are followed.

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.12% | -4.06% | -3.61% | -0.67% | 102% | 101% |
| Class A2 | 0.08% | -5.46% | -10.29% | -1.51% | 102% | 101% |
| Class B | 0.09% | -10.73% | -7.77% | -1.78% | 101% | 101% |
| Class C | 0.08% | -3.31% | -3.22% | -0.59% | 101% | 102% |
| Class E | 0.11% | -8.04% | -2.20% | -0.93% | 101% | 101% |
| Overall | 0.10% | -6.65% | -5.56% | -1.14% | 101% | 101% |
| Class D | 0.07% | -3.23% | -2.05% | -0.47% | 101% | 100% |
| Class F | 0.07% | -4.89% | -5.40% | -0.97% | 101% | 101% |
| TGM | 0.04% | -4.04% | -3.87% | -0.76% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | 0.14% | -5.37% | -5.22% | -0.95% | 101% | 100% |
| Class A2 | 0.15% | -9.47% | -14.53% | -2.28% | 101% | 101% |
| Class B | 0.20% | -16.01% | -12.84% | -2.72% | 101% | 101% |
| Class C | 0.08% | -4.08% | -3.49% | -0.69% | 101% | 101% |
| Class E |  |  |  |  |  |  |
| Overall | 0.15% | -9.39% | -9.16% | -1.74% | 101% | 101% |
| Class D | 0.05% | -4.37% | -3.41% | -0.73% | 101% | 100% |
| Class F | 0.11% | -3.68% | -3.76% | -0.66% | 101% | 101% |
| TGM | 0.27% | -4.78% | -4.94% | -0.76% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.15% | -19.05% | -17.17% | -3.50% | 101% | 101% |
| Class C | 0.15% | -5.42% | -4.50% | -0.87% | 101% | 100% |
| Class E | -0.01% | -13.14% | -5.83% | -1.90% | 102% | 100% |
| Overall | 0.11% | -13.03% | -10.11% | -2.22% | 101% | 101% |
| Class D | 0.08% | -5.99% | -4.32% | -0.97% | 100% | 100% |
| Class F | 0.06% | -8.02% | -6.15% | -1.37% | 101% | 100% |
| TGM | -0.15% | -6.45% | -6.02% | -1.37% | 101% | 101% |

# References

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, March 2019.

# Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).