| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0126 |
| --- | --- |

| Title: | Initializations and propagation of Chroma QP Offset |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kei Kawamura Sei Naito 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 80 5066 9075 ki-kawamura@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

Chroma QP offset-scheme was introduced in the 15th JVET meeting, which was also used for the decision of chroma deblocking filter from the previous meeting. This offset is signalled in the TU level in the working draft 6.0, which means that the offset is not signalled in the non-coded block where CBF is false. In this cases, an actual chroma QP values for the deblocking filter decision is ambiguous. In this contribution, initialization processes of offset values for some block levels are introduced. The most frequently case of the initialization is the unit of chroma quantization group while the least one case is the unit of slice header. At the same time, the offset values for the non-coded unit are propagated from that of the previous coding unit.

Introduction

Chroma QP offset-scheme was introduce in the 15th JVET meeting to handle a various content type like SDR, HDR, screen content, and so on [1]. In detail, chroma QP offset is signalled by a combination of offset list and index. In the original proposal, the index is initialized to 0 at the beginning of a slice header while the chroma QP values are independent to the decision of chroma deblocking filter.

From the previous meeting, the decision of chroma deblocking filter depends on the actual chroma QP values instead of luma based chroma values [2]. Since the index is signalled at TU level, offset values of chroma QP for non-coded unit becomes ambiguous.

To solve the above issue, we introduce an initialization process of the index for some block levels. At the same time, the index for the non-coded unit is propagated from that of the previous coded unit.

Proposed solutions

Initialization

Solution 1: chroma QP group

Solution 1 is a unit of a chroma QP group. An initialization timing is inserted at the coding tree table. An derivation process of actuable CuChromaQpOffsetCb/Cr/CbCr values is moved from the semantic paragraph of cu_chroma_qp_offset_idx to the decoding process section of “Derivation process for quantization parameters”

Solution 2: CTU

An initialization of CuChromaQpOffsetIdx is defined in the begging of each CTU. In this case, any parallel encoding/decoding is supported.

Solution 3: Tile/CTU row

An initialization of CuChromaQpOffsetIdx is defined in the beginning of tile. At the same time, each CTU row is another initialization timing when wavefront is enabled. In this case, VVC parallel encoding/decoding is supported.

Solution 4: Slice level

An initialization of CuChromaQpOffsetIdx is defined in the current specification. However, index propagation is not defined. Furthermore, parallel encoding/decoding with tile and/or wavefront is not realized.

Index propagation.

CuChromaQpOffsetIdx for non-coded unit is propagated from the previous coded unit. While the previous luma QP is defined as the value in the previous luma QP coding unit, the chroma index is define as the value of the previous chroma coding unit in decoding order for the consistency among the adjacent blocks.

Simulation results

Since the VTM7.0 with enabled CuChromaQpOffsetCb/Cr/CbCr contains mismatch of encoder and decoder due to the incorrect handling of CuChromaQpOffsetIdx. The solution 4 is implemented on the VTM-7.0. A test condition follows the common test condition with enabled cu_chroma_qp_offset [3]. We confirm the resolve of encoder mismatch. It is noted that an implementation is solution 4 is only bugfix of VTM-7.0 while missing parts are introduced into the working draft 7. Coding loss is caused by the chroma QP control not for quality/bit-allocation control but for an implementation check.

Table 1 Simulation results with Solution 4 (bugfix) vs VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.67% | 13.19% | 9.72% | 98% | 99% |
| Class A2 | -0.97% | 15.71% | 15.33% | 98% | 99% |
| Class B | -0.38% | 18.63% | 18.43% | 100% | 100% |
| Class C | -0.69% | 16.44% | 15.87% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | 16.37% | 15.38% | 99% | 100% |
| Class D | -0.65% | 18.13% | 17.95% | 99% | 99% |
| Class F | -0.97% | 12.70% | 12.84% | 99% | 99% |

Furthermore, solution 1 and 2 are also implemented on the top of solution 4. The summary of BD-rate are shown in the following tables. Since the chroma QP control does not consider the picture characteristics but assign mechanical values based on the block address, BD-rate values are not meaningful. It verify the whether the encoder and decoder is matched.

Table 1 Simulation results with Solution 1 vs VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.69% | 12.95% | 9.64% | 98% | 100% |
| Class A2 | -0.97% | 15.58% | 15.08% | 98% | 99% |
| Class B | -0.39% | 18.41% | 18.12% | 99% | 100% |
| Class C | -0.67% | 16.34% | 15.80% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | 16.20% | 15.20% | 99% | 100% |
| Class D | -0.67% | 17.85% | 17.82% | 100% | 99% |
| Class F | -1.03% | 12.46% | 12.57% | 99% | 99% |

Table 1 Simulation results with Solution 2 vs VTM7.0

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.71% | 12.98% | 9.72% | 98% | 99% |
| Class A2 | -0.96% | 15.75% | 15.36% | 98% | 99% |
| Class B | -0.40% | 18.59% | 18.53% | 99% | 100% |
| Class C | -0.65% | 16.32% | 15.79% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.64% | 16.30% | 15.40% | 99% | 100% |
| Class D | -0.63% | 18.23% | 17.96% | 99% | 99% |
| Class F | -1.01% | 12.73% | 12.75% | 99% | 99% |

Specification text changes for solution 3

#### 7.3.9.2	Coding tree unit syntax

| dual_tree_implicit_qt_split( x0, y0, cbSize, cqtDepth ) { | Descriptor |
| --- | --- |
| if( cbSize > 64 ) { |  |
| if( cu_chroma_qp_offset_enabled_flag && cbSubdiv <= CuChromaQpOffsetSubdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuChromaQpOffsetIdx = 0 |  |
| } /* Solution 1 as quantization group level */ |  |

#### 7.3.9.4	Coding tree syntax

| coding_tree( x0, y0, cbWidth, cbHeight, qgOnY, qgOnC, cbSubdiv, cqtDepth, mttDepth, depthOffset, partIdx, treeTypeCurr, modeTypeCurr ) { | Descriptor |
| --- | --- |
| if( cu_chroma_qp_offset_enabled_flag && qgOnC && cbSubdiv <= CuChromaQpOffsetSubdiv ) { |  |
| IsCuChromaQpOffsetCoded = 0 |  |
| CuChromaQpOffsetIdx = 0 |  |
| } /* Solution 1 as quantization group level */ |  |

cu_chroma_qp_offset_flag when present and equal to 1, specifies that an entry in the cb_qp_offset_list[ ] is used to determine the value of CuQpOffsetCb, a corresponding entry in the cr_qp_offset_list[ ] is used to determine the value of CuQpOffsetCr, and a corresponding entry in the joint_cbcr_qp_offset_list[ ] is used to determine the value of CuQpOffsetCbCr. cu_chroma_qp_offset_flag equal to 0 specifies that these lists are not used to determine the values of CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr.

When cu_chroma_qp_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

cu_chroma_qp_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ], cr_qp_offset_list[ ], and joint_cbcr_qp_offset_list[ ] that is used to determine the value of CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr. When present, the The value of cu_chroma_qp_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1, inclusive. When not present, the value of cu_chroma_qp_offset_idx is inferred to be equal to 0.

When cu_chroma_qp_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

- The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are derived as follows:

- If cu_chroma_qp_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ cu_chroma_qp_offset_idx ]	(186)

CuQpOffsetCr = cr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(187)

CuQpOffsetCbCr = joint_cbcr_qp_offset_list[ cu_chroma_qp_offset_idx ]	(188)

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are all set equal to 0.

8.7.1	Derivation process for quantization parameters

When ChromaArrayType is not equal to 0 and treeType is equal to SINGLE_TREE or DUAL_TREE_CHROMA, the following applies:

–	When treeType is equal to DUAL_TREE_CHROMA, the variable QpY is set equal to the luma quantization parameter QpY of the luma coding unit that covers the luma location ( xCb + cbWidth / 2, yCb + cbHeight / 2 ).

–	The variable CuChromaQpOffsetIdx is derived as follows:

- If cu_chroma_qp_offset_idx is present, CuChromaQpOffsetIdx is set equal to cu_chroma_qp_offset_idx.

- Otherwise, if one or more of the following conditions are true, CuChromaQpOffsetIdx is set equal to 0:

–	The current quantization group is the first quantization group in a slice.

–	The current quantization group is the first quantization group in a tile.

–	The current quantization group is the first quantization group in a CTB row of a tile and entropy_coding_sync_enabled_flag is equal to 1.

- Otherwise, CuChromaQpOffsetIdx is set equal to the chroma QP offset index CuChromaQpOffsetIdx of the last chroma coding unit in decoding order.

- The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are derived as follows:

- If cu_chroma_qp_offset_enabled_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ CuChromaQpOffsetIdx ]	(xxx)

CuQpOffsetCr = cr_qp_offset_list[ CuChromaQpOffsetIdx ]	(xxx)

CuQpOffsetCbCr = joint_cbcr_qp_offset_list[ CuChromaQpOffsetIdx ]	(xxx)

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are all set equal to 0.

- The variables qPCb, qPCr and qPCbCr are derived as follows:

Conclusion

In this contribution, we proposed cross-component residual prediction (CCRP) for 4:4:4 format. The CCRP is enabled only the single tree case with no-intra sub partition. We also propose the fast computation method to derive the scaling value as an encoder optimization. The simulation results show the improvement of both luma and chroma coding performance without sacrificing the encoding/decoding run-time. We recommend to adopt the proposal to the next working draft.

This work was conducted under the research project of Ministry of Internal Affairs and Communication.

References

- A. K. Ramasubramonian, G. Auwera, D. Rusanovskyy, M. Karczewicz, “AHG15: On signalling of chroma QP tables”, JVET-O0650, Gothenburg, SE, 3–12 July 2019.

- A. M. Kotra, S. Esenlik, B. Wang, H. Gao, E. Alshina, “Non-CE5:  Chroma QP derivation fix for deblocking filter (Combination of JVET-P0105 and JVET-P0539)”, JVET-P1001, Geneva, CH, 1–11 October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

Patent rights declaration

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).