| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0128-v2 |
| --- | --- |

| Title: | On clarification of applicable conditions of DMVR and BDOF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomonori Hashimoto Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

One of the applicable conditions of DMVR and BDOF is that luminance coefficients and offsets for weighted prediction do not exist for both L0 and L1 reference indexes. However, when any luminance coefficient does not exist and some chroma coefficients exist, the applicable conditions of DMVR and BDOF are not clear. There are bugs of both text and software, In this contribution, add a condition that chroma coefficients and offsets for weighted prediction do not exist for both L0 and L1 reference indexes. The changes are to add only one line to the applicable condition of both DMVR and BDOF. There is no change of experimental results on CTC. There were negligible differences for fade sequences when WeightedPredB on VTM-7.0 is set to 1. If the chroma weighted prediction is not constrained, the bugs of text and software should be fixed.

Problem Statement

One of applicable conditions of DMVR and BDOF is that luminance coefficients and offsets for weighted prediction are not exist for both L0 and L1 reference indexes. However, when any luminance coefficient is not existed and some chroma coefficients are existed, the applicable conditions of DMVR and BDOF are not clear. Currently, there are several confusions of both WD7 and VTM-7.0. In WD7, the variable weightedPredFlag is set equal to ( pps_weighted_bipred_flag && !dmvrFlag ), however this condition is not considered chroma weighted prediction. (It should be set equal to ( pps_weighted_bipred_flag && !(dmvrFlag && cIdx == 0) ),) In VTM-7.0, DMVR and BDOF support chroma weighted prediction. However, CIIP encoder does not consider chroma weighted prediction for DMVR. Therefore, the results between encoder and decoder are not identical when CIIP is on and chroma coefficients and offsets for weighted prediction are existed. (That bug has been reported as ticket #778)

Proposal changes of specification

The working draft [1] should be changed by the followings.

- 8.5 Decoding process for coding units coded in inter prediction mode

- 8.5.1 General decoding process for coding units coded in inter prediction mode

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0.

- Both chroma_weight_l0_flag[ refIdxL0 ] and chroma_weight_l1_flag[ refIdxL1 ] are equal to 0.

- 8.5.6Decoding process for inter blocks

- 8.5.6.1 General

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

      - chroma_weight_l0_flag[ refIdxL0 ] and chroma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

Experimental result

Since weighted prediction is not used on CTC [2], the experiments by using weighted prediction and fade-sequences have been conducted in this contribution.

Fade sequences are generated with a fading tool in JVET-L0201-v1 [3]. In our experiments, a linear fade is applied to the about first 2 seconds of the regular sequence set, and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs. The number of fade sequence frames is decided by following Table 1.

Table 1	The number of fade sequence frames against frame rate

| Frame Rate | The number of fade sequence frames |
| --- | --- |
| 20 | 33 |
| 30 | 65 |
| 50 | 97 |
| 60 | 129 |

The PC cluster was used for the simulation, but since the same CPUs are allocated to the same parameters of the same sequences, the execution time is almost accurate. In order to measure more accurate decoding times, the decoder was re-run all the bitstreams on single CPU.

Table 2 shows a summary of experimental results of proposed method. The parameters for weighted prediction was set WeightedPredP=1, WeightedPredB=1 and WeightedPredMethod=0. In VTM software, there are five modes to generate coefficients and offsets for weighted prediction. Those modes are defined as WeightedPredMethod. The results of the proposed method were identical with that of the anchor, because both luma and chroma weighted predictions are switched at the same time.

Table 3 shows a summary of experimental results of proposed method. The parameters for weighted prediction was set WeightedPredP=1, WeightedPredB=1 and WeightedPredMethod=4. In that case, there were very small differences between the results of proposed method and that of anchor, because sometimes chroma weighted prediction was used.

Those results show that proposed method has no negative effect.

Table 2  Summary of experimental results (compared to WeightedPredMethod = 0)

|  |  | Over VTM-7.0_WPM0+WPB+WPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |  |
| Proposed (RA) | Black fade sequences | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | White fade sequences | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 3  Summary of experimental results (compared WeightedPredMethod to = 4)

|  |  | Over VTM-7.0_WPM4+WPB+WPP |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT |  |
| Proposed (RA) | Black fade sequences | 0.00% | 0.01% | 0.01% | 100% | 100% |
|  | White fade sequences | 0.00% | -0.01% | 0.00% | 100% | 100% |

Conclusion

In this contribution, applicable conditions of DMVR and BDOF have been clarified. We would like to recommend that the working draft and reference software should support this specification. If not, the bugs of text and software should be fixed.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 7), Joint Video Experts Team Document, JVET-P2001-vE, Geneva, Nov. 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-O2010, Gothenburg, July 2019.

[3]	P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).