| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0129-v1 |
| --- | --- |

| Title: | On editorial improvements for specification of explicit weighted prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomonori Hashimoto Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In this contribution, an editorial problem with specification of explicit weighted prediction is pointed out and three options to solve it are proposed. In the current WD7, if weighting factors in explicit weighted prediction are not present, the default weighting factors of explicit weighted prediction are used even though it is possible to be performed in default weighted prediction. In order to solve it, it is necessary to improve the decision condition to switch between default weighted prediction and explicit weighted prediction correctly. Three different options to solve its are shown. First solution is to change the decoding process of explicit weighted prediction. These solutions are only editorial changes. Second solution is to change the specification of variable weightedPredFlag which switches the decoding process of default weighted prediction and explicit weighted prediction. Third solution is to define a new decision function for switching explicit weighted prediction. The bitstream and the results of encoding and decoding are not changed.

Problem Statement

In the current WD7, if weighting factors in explicit weighted prediction are not present, the default weighting factors of explicit weighted prediction are used even though it is possible to be performed in default weighted prediction. For example, if slice_type is equal to 0 (B) and the variable weightedPredFlag is equal to 1, then explicit weighted prediction is called. Here, the variable BitDepth is set to 10, luma_log2_weight_denom is set to 5, the variable refIdxL0 is set to 1, the variable refIdxL1 is set 0, luma_weight_l0_flag[1] is equal to 0 and luma_weight_l1_flag[0] is equal to 0.

When luma_weight_lX_flag[i] is equal to 0, LumaWeightLX[i] is inferred to be equal to 2luma_log2_weight_denom and luma_offset_lX[i] is inferred to be equal to 0. Therefore, the variable LumaWeightL0[ refIdxL0 ] is set to 32, LumaWeightL1[ refIdxL1 ] is set to 32, the variable luma_offset_l0[refIdxL0] is set to 0 and the variable luma_offset_l1[refIdxL1] is set to 0. As a result, the prediction sample is derived as follow:

pbSamples[x][y] = Clip3(0, 1023, (predSamplesL0[x][y]*32+predSamplesL1[x][y]*32 +(1<<9))>>10).

However, mathematically, the following formula should be derived.

pbSamples[x][y] = Clip3(0, 1023, (predSamplesL0[x][y]+predSamplesL1[x][y] +(1<<4))>>5).

Proposal changes of specification

In order to solve this problem, it is necessary to improve the decision condition to switch between default weighted prediction and explicit weighted prediction correctly. In this contribution, three different solutions have been proposed. Either method can solve this problem.

Solution 1

First solution is to change the decoding process of explicit weighted prediction is subsection 8.5.6.3.3.

- 8.5.6.6.3	Explicit weighted sample prediction process

Inputs to this process are:

- two variables nCbW and nCbH specifying the width and the height of the current coding block,

- two (nCbW)x(nCbH) arrays predSamplesL0 and predSamplesL1,

- the prediction list utilization flags, predFlagL0 and predFlagL1,

- the reference indices, refIdxL0 and refIdxL1,

- the variable cIdx specifying the colour component index,

- the sample bit depth, bitDepth.

Output of this process is the (nCbW)x(nCbH) array pbSamples of prediction sample values.

The variable shift1 is set equal to Max( 2, 14 − bitDepth ).

The variables log2Wd, o0, o1, w0 and w1 are derived as follows:

- If cIdx is equal to 0 for luma samples, the following applies:

log2Wd = luma_log2_weight_denom + shift1		(1010)

WeightL0Flag = luma_weight_l0_flag[ refIdxL0 ]

WeightL1Flag = luma_weight_l1_flag[ refIdxL1 ]

w0 = LumaWeightL0[ refIdxL0 ]		(1011)

w1 = LumaWeightL1[ refIdxL1 ]		(1012)

o0 = luma_offset_l0[ refIdxL0 ]  <<  ( bitDepth − 8 )	(1013)

o1 = luma_offset_l1[ refIdxL1 ]  <<  ( bitDepth − 8 )	(1014)

- Otherwise (cIdx is not equal to 0 for chroma samples), the following applies:

log2Wd = ChromaLog2WeightDenom + shift1		(1015)

WeightL0Flag = chroma_weight_l0_flag[ refIdxL0 ]

WeightL1Flag = chroma_weight_l1_flag[ refIdxL1 ]

w0 = ChromaWeightL0[ refIdxL0 ][ cIdx − 1 ]		(1016)

w1 = ChromaWeightL1[ refIdxL1 ][ cIdx − 1 ]		(1017)

o0 = ChromaOffsetL0[ refIdxL0 ][ cIdx − 1 ]  <<  ( bitDepth − 8 )	(1018)

o1 = ChromaOffsetL1[ refIdxL1 ][ cIdx − 1 ]  <<  ( bitDepth − 8 )	(1019)

The prediction sample pbSamples[ x ][ y ] with x = 0..nCbW − 1 and y = 0..nCbH − 1 are derived as follows:

- If predFlagL0 is equal to 1 and predFlagL1 is equal to 0, the prediction sample values are derived as follows:

if( WeightL0Flag ==  0 )
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
 			( predSamplesL0[ x ][ y ] + offset1 )  >>  shift1 )	
else if( log2Wd  >=  1 )
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
		( ( predSamplesL0[ x ][ y ] * w0 + 2log2Wd − 1 )  >>  log2Wd ) + o0 )	(1020)
else
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, predSamplesL0[ x ][ y ] * w0 + o0 )

- Otherwise, if predFlagL0 is equal to 0 and predFlagL1 is equal to 1, the prediction sample values are derived as follows:

if( WeightL1Flag ==  0 )
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
 			( predSamplesL1[ x ][ y ] + offset1 )  >>  shift1 )
else if( log2Wd  >=  1 )
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
		( ( predSamplesL1[ x ][ y ] * w1 + 2log2Wd − 1 )  >>  log2Wd ) + o1 )	(1021)
else
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, predSamplesL1[ x ][ y ] * w1 + o1 )

- Otherwise (predFlagL0 is equal to 1 and predFlagL1 is equal to 1), the prediction sample values are derived as follows:

if( WeightL0Flag ==  0 && WeightL1Flag ==  0 )
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1, 
		( predSamplesL0[ x ][ y ] + predSamplesL1[ x ][ y ] + offset2 )  >>  shift2 )
else
	pbSamples[ x ][ y ] = Clip3( 0, ( 1  <<  bitDepth ) − 1,
		( predSamplesL0[ x ][ y ] * w0 + predSamplesL1[ x ][ y ] * w1 +
			( ( o0 + o1 + 1 )  <<  log2Wd ) )  >>  ( log2Wd + 1 ) )	(1022)

Solution2

Second solution is to change the specification of variable weightedPredFlag which switches the decoding process of default weighted prediction and explicit weighted prediction in subsection 8.5.6.6.

- 8.5.6.6 Weighted sample prediction process

- 8.5.6.6.1 General

The variable weightedPredFlag is derived as follows:

- If slice_type is equal to P and pps_weighted_pred_flag is equal to 1, the variable weightedPredFlag is derived as follows:

weightedPredFlag = ( cIdx == 0 ) ? luma_weight_l0_flag [refIdxL0] : 
										chroma_weight_l0_flag[refIdxL0]

- Otherwise if slice_type is B and pps_weighted_bipred_flag is equal to 1, the following applies.

  - If refIdxL0 is greater than or equal to 0 and refIdxL1 is greater than or equal to 0, the variable weightedPredFlag is derived as follows:

weightedPredFlag = ( cIdx == 0 ) ? 
 				(luma_weight_l0_flag[refIdxL0] || luma_weight_l1_flag[refIdxL1]) :

(chroma_weight_l0_flag[refIdxL0] || chroma_weight_l1_flag[refIdxL1])

  - Otherwise, if refIdxL0 is greater than or equal to 0 and refIdxL1 is less than 0, the variable weightedPredFlag is derived as follows:

weightedPredFlag = ( cIdx == 0 ) ? luma_weight_l0_flag [refIdxL0] : 
										chroma_weight_l0_flag[refIdxL0]

  - Otherwise, if refIdxL0 is less than 0 and refIdxL1 is greater than or equal to 0, the variable weightedPredFlag is derived as follows:

weightedPredFlag = ( cIdx == 0 ) ? luma_weight_l1_flag [refIdxL1] : 
										chroma_weight_l0_flag[refIdxL1]

  - Otherwise,

weightedPredFlag = 0

- Otherwise

weightedPredFlag = 0

- If slice_type is equal to P, weightedPredFlag is set equal to pps_weighted_pred_flag.

- Otherwise (slice_type is equal to B), weightedPredFlag is set equal to ( pps_weighted_bipred_flag  &&  !dmvrFlag ).

Solution 3

Third solution is to define a new function WeightedPredFlag( slice_type, cIdx, refIdxL0, refIdxL1 ) for switching explicit weighted prediction correctly and use it for the condition of BCW, DMVR and BDOF and for the definition of the variable weightedPredFlag.

- 7.3.9.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| if( sps_bcw_enabled_flag && inter_pred_idc[ x0 ][ y0 ] = = PRED_BI && WeightedPredFlag( B, 0, ref_idx_l0, ref_idx_l1 ) && WeightedPredFlag( B, 1, ref_idx_l0, ref_idx_l1 ) && luma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && luma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l0_flag[ ref_idx_l0 [ x0 ][ y0 ] ] = = 0 && chroma_weight_l1_flag[ ref_idx_l1 [ x0 ][ y0 ] ] = = 0 && cbWidth * cbHeight >= 256 ) |  |
| bcw_idx[ x0 ][ y0 ] | ae(v) |

- 7.4.8.2 Weighted prediction parameters semantics

.

The function WeightedPredFlag( slice_type, cIdx, refIdxL0, refIdxL1 ) is specified as follows:

WeightedPredFlag( slice_type, cIdx, refIdxL0, refIdxL1 ) = 
( slice_type == P && pps_weighted_pred_flag ) ? 
((cIdx == 0) ? luma_weight_l0_flag [refIdxL0] : chroma_weight_l0_flag[refIdxL0]) : 
((slice_type == B && pps_weighted_bipred_flag ) ? 
((refIdxL0>=0 && refIdxL1>=0) ? 
((cIdx == 0) ? (luma_weight_l0_flag[refIdxL0] || luma_weight_l1_flag[refIdxL1]) : (chroma_weight_l0_flag[refIdxL0] || chroma_weight_l1_flag[refIdxL1])) : 
((refIdxL0>=0 && refIdxL1<0) ? 
((cIdx == 0) ? luma_weight_l0_flag [refIdxL0] : chroma_weight_l0_flag[refIdxL0]) : 
((refIdxL0<0 && refIdxL1>=0) ? 
((cIdx == 0) ? luma_weight_l1_flag [refIdxL1] : chroma_weight_l1_flag[refIdxL1]) : 0))) : 0)

- 8.5 Decoding process for coding units coded in inter prediction mode

- 8.5.1 General decoding process for coding units coded in inter prediction mode

- When all of the following conditions are true, dmvrFlag is set equal to 1:

- WeightedPredFlag( B, 0, refIdxL0, refIdxL1 ) is equal to 0

- Both luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are equal to 0

- 8.5.6Decoding process for inter blocks

- 8.5.6.1 General

    - If all of the following conditions are true, bdofFlag is set equal to TRUE.

      - WeightedPredFlag( B, 0, refIdxL0, refIdxL1 ) is equal to 0.

      - luma_weight_l0_flag[ refIdxL0 ] and luma_weight_l1_flag[ refIdxL1 ] are both equal to 0.

- 8.5.6.6 Weighted sample prediction process

- 8.5.6.6.1 General

The variable weightedPredFlag is set to WeightedPredFlag( slice_type, cIdx, refIdxL0. refIdxL1 ).

The variable weightedPredFlag is derived as follows:

- If slice_type is equal to P, weightedPredFlag is set equal to pps_weighted_pred_flag.

- Otherwise (slice_type is equal to B), weightedPredFlag is set equal to ( pps_weighted_bipred_flag  &&  !dmvrFlag ).

Conclusion

In this contribution, a problem with specification of explicit weighted prediction has been pointed out and three options to solve it have been proposed. We would like to recommend adopting either solution for the working draft.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 7), Joint Video Experts Team Document, JVET-P2001-vE, Geneva, Nov. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).