| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0130-v1 |
| --- | --- |

| Title: | On improvement of collocated_ref_idx |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Takeshi Chujoh Tomonori Hashimoto Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-70-1617-3470 chujoh.takeshi@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

In current Draft 7, the semantics of collocated_ref_idx which specifies the reference index of the collocated picture used for temporal motion vector prediction has some requirements on bitstream conformance. The decoder that stops to decode incorrect bitstreams is right. However, it may be possible to decode them. Since generally, codec users believe the decoder which can decode them is right, it is not good that the decoder satisfied with requirements on bit-stream conformance can not decode them. Therefore, from the view-point of standardization, the requirements on bitstream conformance should be reduced and it is desirable to be able to decode bitstreams as possible. In this contribution, an improvement of collocated_ref_idx has been proposed. it is a specification that when the bit-stream can not be satisfied requirements of bit-stream conformance, prohibit temporal motion vector prediction and they are decoded.

Problem Statement

In current Draft 7, the semantics of collocated_ref_idx which specifies the reference index of the collocated picture used for temporal motion vector prediction has some requirements on bit-stream conformance. From the view-point of standardization, the constraints of bitstream by encoder should be reduced and it is desirable to define the specification of the decoding process to avoid the constraints.

Proposal changes of specification

The working draft [1] should be changed by the followings.

- 7.4.8	Slice header semantics

- 7.4.8.1	General slice header semantics

collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When collocated_ref_idx is not present, the value of collocated_ref_idx is inferred to be equal to 0 and the variable SliceTemporalMvpEnabledFlag is set to pic_temporal_mvp_enabled_flag.

When all of the following conditions are true, the variable SliceTemporalMvpEnabledFlag is set to pic_temporal_mvp_enabled_flag.

- When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, the value of collocated_ref_idx is in the range of 0 to NumRefIdxActive[ 0 ] − 1.

- When slice_type is equal to B and collocated_from_l0_flag is equal to 0, the value of collocated_ref_idx is greater than NumRefIdxActive[ 1 ] − 1.

- RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx ] is equal to 0.

Otherwise, the variable SliceTemporalMvpEnabledFlag is set equal to 0.

When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, collocated_ref_idx refers to a picture in list 0, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and collocated_from_l0_flag is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When collocated_ref_idx is not present, the value of collocated_ref_idx is inferred to be equal to 0.

It is a requirement of bitstream conformance that the picture referred to by collocated_ref_idx shall be the same for all slices of a coded picture.

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture shall be the same and RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx  ] shall be equal to 0.

- 8.5.2.11 Derivation process for temporal luma motion vector prediction

The variables mvLXCol and availableFlagLXCol are derived as follows:

- If pic_temporal_mvp_enabled_flagSliceTemporalMvpEnabledFlag is equal to 0 or ( cbWidth * cbHeight ) is less than or equal to 32, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (pic_temporal_mvp_enabled_flagSliceTemporalMvpEnabledFlag is equal to 1), the following ordered steps apply:

- 8.5.5.3 Derivation process for subblock-based temporal merging candidates

The availability flag availableFlagSbCol is derived as follows.

- If one or more of the following conditions is true, availableFlagSbCol is set equal to 0.

- pic_temporal_mvp_enabled_flagSliceTemporalMvpEnabledFlag is equal to 0.

- 8.5.5.6 Derivation process for constructed affine control point motion vector merging candidates

- The variables mvLXCol and availableFlagLXCol, with X being 0 or 1, are derived as follows:

- If pic_temporal_mvp_enabled_flagSliceTemporalMvpEnabledFlag is equal to 0, both components of mvLXCol are set equal to 0 and availableFlagLXCol is set equal to 0.

- Otherwise (pic_temporal_mvp_enabled_flagSliceTemporalMvpEnabledFlag is equal to 1), the following applies:

Conclusion

In this contribution, an improvement of collocated_ref_idx has been proposed. We would like to recommend that the working draft and reference software should support this specification.

Reference

[1]	B. Bross, J. Chen and S. Liu, Versatile Video Coding (Draft 7), Joint Video Experts Team Document, JVET-P2001-vE, Geneva, Nov. 2019.

[2]	F. Bossen, J. Boyce, X. Li, V. Seregin and K. Sühring, JVET common test conditions and software reference configurations for SDR video, Joint Video Experts Team Document, JVET-N1010, Geneva, April. 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).