| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0131 |
| --- | --- |

| Title: | Non-CE4: GEO with mode reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Eiich Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 tomonori.hashimoto@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a mode reduction of geometric partitioning for merge mode (GEO). GEO extends triangle partitioning’ 2 angles and 1 distance into 24 angles and 4 distances to achieve better coding efficiency. However, it is asserted the burden of encoder search for GEO mode is not small and several angles in 24 angles would not be so effective. This contribution provides three options of mode reduction of 54 modes with 16 angles (option 1), 40 modes with 12 angles (option 2) and 26 modes with 8 angles (option 3). It is reported the bdrate change is -0.26 %, -0.24 % and -0.19 % in luma with EncT being 102 %, 101 % and 101 % in option 1, option 2 and option 3 respectively.

In addition, this contribution proposes a control syntax for maximum modes for GEO, which is signaled in a picture header. It can allow encoders to choose the number of modes for GEO so that it provides an encoder flexibility.

Introduction

Geometric partitioning for merge mode (GEO) [1] has been studied in CE4. GEO uses the similar predictors blending concept as TPM and extends the blending masks up to 82 different modes with 24 angles and 4 distances. Though it shows significant coding gain, it requires the burden of encoder search.

This contribution proposes a GEO with mode reduction. In our experiments, some angles of 24 angles seem redundant and we can reduce the number of angles with few losses. And if we switch the angle to be selected depending on block shape, more angles can be removed with few losses. In this contribution, there are three options, 54 modes with 16 angles, 40 modes with 12 angles and 26 modes with 8 angles. The reduction of modes for GEO eases the burden of GEO search in encoding process.

In addition, as a way to ease the burden, this contribution proposes to signal an additional parameter for maximum modes for GEO in picture header. It can control the number of modes for GEO adaptively and provides the flexibility for encoder.

Proposal

Method1: GEO with angle reduction

In method1, we propose to reduce the angles from original GEO.

Option1: 54 modes with 16 angles

Option 1 uses 16 angles for GEO. The angles where angleIdx is equal to 1, 5, 7, 11, 13, 17, 19 and 23 are removed from the original GEO as shown in Figure 1. There are no changes on distanceIdx; the number of distances remains 4. As a result, there are 54 modes for GEO.

Figure 1: 16 angles

Option2: 40 modes with 12 angles

Option 2 uses 12 angles for GEO. In this option, the angles are selected depending on block shape as shown in Figure 2. The number of distances remains 4. As a result, there are 40 modes for GEO.

Figure 2: 12 angles depending on block shape

Option3: 26 modes with 8 angles

Option 3 uses 8 angles for GEO. As the same as option 2, the angles are selected depending on block shape as shown in Figure 3. The number of distances remains 4. As a result, there are 26 modes for GEO.

Figure 3: 8 angles depending on block shape

Method2: Signal the maximum number of modes for GEO

In method2, we propose to select the number of modes for GEO by signaling an index of the maximum mode number. Specifically, if sps_geo_enabled_flag is on, newly introduced syntax element max_merge_geo_partition_idx is signaled in a picture header to indicate the maximum number of modes for GEO.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| … |  |
| if( sps_wedge_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_wedge_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_wedge_cand | ue(v) |
| if( sps_wedge_enabled_flag ) |  |
| max_merge_geo_partition_idx | u(2) |
| … |  |
| } |  |

The maximum number of modes for GEO is derived as Table 1.

Table 1. Specification of the maximum number of modes for GEO based on max_merge_geo_partition_idx value

| the value of max_merge_geo_partition_idx | the maximum number of modes for GEO |
| --- | --- |
| 0 | 82 |
| 1 | 54 |
| 2 | 40 |
| 3 | 26 |

Experiment

Encoder search

The actual encoding burden could depend on the numbers of candidates for SAD and SATD search which are used in encoding process.

It is more reasonable to reduce those candidates as the number of modes for GEO decreases. Hence, we conduct experiments with reduced numbers of candidates for SAD and SATD search as Table 2.

Table 2. Number of candidates for SAD and SATD in proposed VTM encoder configuration, which is based on the number of modes for GEO

| the number of modes for GEO | SAD candidates | SATD candidates |
| --- | --- | --- |
| 82 (original) | 40 | 5 |
| 54 | 30 | 5 |
| 40 | 20 | 4 |
| 28 | 15 | 3 |

Simulation

The proposed methods are implemented on top of the CE4 common base software. The simulations were conducted according to JVET-P2024 [2] and the above encoding settings.

The following tables show the summary of option 1, option 2 and option 3.

Compared to VTM-7.0

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Common base | -0.28% | -0.35% | -0.34% | 102% | 100% | -0.70% | -0.67% | -0.77% | 103% | 99% |
| Option 1 | -0.26% | -0.31% | -0.29% | 102% | 100% | -0.64% | -0.60% | -0.48% | 102% | 99% |
| Option 2 | -0.24% | -0.26% | -0.25% | 101% | 100% | -0.61% | -0.56% | -0.46% | 101% | 99% |
| Option 3 | -0.19% | -0.20% | -0.19% | 100% | 100% | -0.50% | -0.46% | -0.49% | 99% | 99% |

Compared to common base

|  | RA |  |  |  |  | LB |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Common base | - | - | - | - | - | - | - | - | - | - |
| Option 1 | 0.02% | 0.04% | 0.05% | 99% | 100% | 0.06% | 0.07% | 0.29% | 99% | 100% |
| Option 2 | 0.04% | 0.09% | 0.08% | 99% | 100% | 0.10% | 0.11% | 0.31% | 99% | 100% |
| Option 3 | 0.09% | 0.15% | 0.15% | 98% | 100% | 0.20% | 0.22% | 0.28% | 97% | 100% |

Compared with VTM-7.0

Results for 16 angles compared with VTM-7.0 (Option1)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.26% | -0.17% | 102% | 100% |
| Class A2 | -0.16% | -0.11% | -0.03% | 102% | 100% |
| Class B | -0.13% | -0.06% | -0.09% | 101% | 100% |
| Class C | -0.57% | -0.82% | -0.83% | 102% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.26% | -0.31% | -0.29% | 102% | 100% |
| Class D | -0.45% | -0.62% | -0.59% | 102% | 100% |
| Class F | -0.33% | -0.35% | -0.32% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.53% | -0.39% | 103% | 99% |
| Class C | -0.77% | -1.13% | -0.69% | 102% | 99% |
| Class E | -0.96% | -0.04% | -0.35% | 100% | 99% |
| Overall | -0.64% | -0.60% | -0.48% | 102% | 99% |
| Class D | -0.72% | -0.86% | -1.17% | 101% | 99% |
| Class F | -0.58% | -1.02% | -0.20% | 101% | 99% |

Results for 12 angles selected by depending on block shape compared with VTM-7.0 (Option2)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.13% | -0.11% | 102% | 100% |
| Class A2 | -0.16% | -0.06% | -0.05% | 101% | 100% |
| Class B | -0.13% | -0.10% | -0.10% | 101% | 100% |
| Class C | -0.51% | -0.72% | -0.72% | 101% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.24% | -0.26% | -0.25% | 101% | 100% |
| Class D | -0.39% | -0.62% | -0.46% | 102% | 100% |
| Class F | -0.33% | -0.34% | -0.32% | 100% | 99% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.42% | -0.46% | 102% | 99% |
| Class C | -0.69% | -0.96% | -0.65% | 102% | 99% |
| Class E | -0.96% | -0.27% | -0.20% | 99% | 99% |
| Overall | -0.61% | -0.56% | -0.46% | 101% | 99% |
| Class D | -0.61% | -0.95% | -1.05% | 101% | 99% |
| Class F | -0.62% | -1.16% | -0.11% | 101% | 99% |

Results for 8 angles selected by depending on block shape compared with VTM-7.0 (Option3)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | -0.12% | -0.10% | 101% | 100% |
| Class A2 | -0.15% | -0.11% | 0.02% | 100% | 100% |
| Class B | -0.09% | 0.01% | -0.07% | 100% | 100% |
| Class C | -0.44% | -0.59% | -0.55% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.19% | -0.20% | -0.19% | 100% | 100% |
| Class D | -0.32% | -0.31% | -0.25% | 100% | 100% |
| Class F | -0.27% | -0.29% | -0.19% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.35% | -0.48% | 100% | 99% |
| Class C | -0.49% | -0.71% | -0.47% | 99% | 99% |
| Class E | -0.88% | -0.32% | -0.53% | 98% | 99% |
| Overall | -0.50% | -0.46% | -0.49% | 99% | 99% |
| Class D | -0.58% | -0.36% | -0.56% | 99% | 99% |
| Class F | -0.49% | -1.00% | -0.35% | 99% | 99% |

Compared with the common base

Results for 16 angles compared with the common base (Option1)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.17% | 0.01% | 99% | 100% |
| Class A2 | 0.05% | 0.08% | -0.01% | 99% | 100% |
| Class B | 0.01% | 0.04% | 0.09% | 99% | 100% |
| Class C | 0.02% | 0.17% | 0.06% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.04% | 0.05% | 99% | 100% |
| Class D | -0.02% | 0.05% | 0.03% | 99% | 100% |
| Class F | -0.01% | 0.03% | 0.01% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.02% | 0.22% | 99% | 100% |
| Class C | 0.03% | 0.12% | 0.20% | 99% | 100% |
| Class E | 0.21% | 0.15% | 0.54% | 99% | 100% |
| Overall | 0.06% | 0.07% | 0.29% | 99% | 100% |
| Class D | -0.03% | -0.40% | -0.13% | 98% | 100% |
| Class F | -0.08% | -0.12% | 0.10% | 99% | 100% |

Results for 12 angles selected by depending on block shape compared with the common base (Option2)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.04% | 0.07% | 99% | 100% |
| Class A2 | 0.06% | 0.13% | -0.03% | 99% | 100% |
| Class B | 0.02% | 0.00% | 0.08% | 99% | 100% |
| Class C | 0.07% | 0.27% | 0.18% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.04% | 0.09% | 0.08% | 99% | 100% |
| Class D | 0.04% | 0.04% | 0.16% | 99% | 100% |
| Class F | -0.01% | 0.05% | 0.01% | 99% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | 0.08% | 0.14% | 99% | 100% |
| Class C | 0.10% | 0.30% | 0.24% | 99% | 100% |
| Class E | 0.21% | -0.08% | 0.69% | 99% | 100% |
| Overall | 0.10% | 0.11% | 0.31% | 99% | 100% |
| Class D | 0.07% | -0.49% | -0.01% | 98% | 100% |
| Class F | -0.11% | -0.26% | 0.19% | 99% | 100% |

Results for 8 angles selected by depending on block shape compared with the common base (Option3)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.08% | -0.03% | 0.08% | 98% | 100% |
| Class A2 | 0.07% | 0.08% | 0.04% | 98% | 100% |
| Class B | 0.05% | 0.11% | 0.10% | 98% | 100% |
| Class C | 0.15% | 0.40% | 0.35% | 97% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.15% | 0.15% | 98% | 100% |
| Class D | 0.11% | 0.36% | 0.37% | 97% | 100% |
| Class F | 0.04% | 0.09% | 0.14% | 98% | 100% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.07% | 0.15% | 0.12% | 97% | 100% |
| Class C | 0.31% | 0.55% | 0.42% | 96% | 100% |
| Class E | 0.29% | -0.13% | 0.35% | 97% | 100% |
| Overall | 0.20% | 0.22% | 0.28% | 97% | 100% |
| Class D | 0.10% | 0.07% | 0.48% | 96% | 100% |
| Class F | 0.01% | -0.09% | -0.05% | 97% | 100% |

Encoder only test (informative)

We provide the results of encoder only test proposed 16, 12 or 8 angles. The following tables show performances for RA with 16, 12 and 8 angles, respectively.

Encoder only results: 16 angles

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | -0.02% | 0.02% | 99% | 100% |
| Class A2 | 0.10% | 0.10% | 0.01% | 100% | 100% |
| Class B | 0.03% | -0.01% | 0.10% | 99% | 100% |
| Class C | 0.13% | 0.33% | 0.23% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 0.10% | 0.10% | 99% | 100% |
| Class D | 0.06% | 0.15% | 0.12% | 99% | 100% |
| Class F | 0.03% | 0.03% | 0.01% | 99% | 100% |

Encoder only results: 12 angles depending on block shape

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | -0.04% | 0.08% | 99% | 100% |
| Class A2 | 0.12% | 0.23% | 0.05% | 99% | 100% |
| Class B | 0.07% | -0.02% | 0.11% | 99% | 100% |
| Class C | 0.20% | 0.36% | 0.34% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.12% | 0.13% | 0.15% | 99% | 100% |
| Class D | 0.11% | 0.07% | 0.21% | 99% | 100% |
| Class F | 0.10% | 0.12% | 0.15% | 99% | 100% |

Encoder only results: 8 angles depending on block shape

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE4_common_base (sharp) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.15% | 0.11% | 0.21% | 99% | 100% |
| Class A2 | 0.23% | 0.25% | 0.18% | 99% | 100% |
| Class B | 0.15% | 0.19% | 0.22% | 99% | 100% |
| Class C | 0.40% | 0.67% | 0.66% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.23% | 0.32% | 0.33% | 99% | 100% |
| Class D | 0.29% | 0.20% | 0.44% | 99% | 100% |
| Class F | 0.21% | 0.24% | 0.17% | 99% | 100% |

These tables show unignorable losses when encoder searches only selected angles. Therefore, it is good to reduce the maximum number of modes for GEO when ineffective angles are removed.

Conclusion

This contribution proposes a GEO with mode reduction. It eases the burden for GEO search in encoding process with few losses. It is recommended to adopt this proposal in the next VVC WD and VTM software.

Reference

[1]	H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0108, JVET-P0264 and JVET-P0304),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P0884, 16th Meeting: Geneva, CH, 1–11 October 2019.

[2] C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang, “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning,” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2024, 16th Meeting: Geneva, CH, 1–11 October 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).