| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0133 |
| --- | --- |

| Title: | Fix on LFNST condition |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Tomonori Hashimoto Takeshi Chujoh Eiichi Sasaki Tomohiro Ikai 1-9-2 Nakase, Mihama-ku, Chiba, Japan | Tel: Email: | +81-50-5213-6337 tomonori.hashimoto@sharp.co.jp |
| Source: | Sharp Corporation |  |  |

_____________________________

- Abstract

This contribution proposes a fix related to LFNST condition. In the current specification, LFNST index and transform unit size are checked to decide if LFNST is applied or not to guarantee small TUs (i.e.chroma 2xN, Nx2 TUs) is not applied. However, in scaling list and implicit MTS process, transform unit size condition is missing (not checked). This contribution proposes a fix on WD and software. Specifically, the proposed WD uses the following variable lfnstEnabledFlag in whole WD to avoid mistakes and confusions

The variable lfnstEnabledFlag is set equal to lfnst_idx && nTbW >= 4 && nTbH >= 4 ? 1 : 0

Introduction

To decide if LFNST [1] is applied, lfnst_idx and transform unit size are needed to be checked to avoid small TU cases (2xN or Nx2 chroma TUs). The following table shows a case where lfnstIdx only check is not sufficient and TU size should be checked.

|  | cIdx | nTbW | nTbH | lfnstIdx | LFNST | Note |
| --- | --- | --- | --- | --- | --- | --- |
| SIGNLE_TREE | 0 | 4 | 4 | 1 | Y | - |
| SIGNLE_TREE | 1 | 2 | 2 |  | N | TU size should be checked |
| DUAL_TREE | 0 | 4 | 4 | 1 | Y |  |
| DUAL_TREE | 1 | 2 | 2 | - (inferred to be 0) | N |  |

The current specification in 8.7.4 Transformation process for scaled transform coefficients uses the following condition to apply LFNST.

When lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

However, in other places such as scaling list and implicit MTS process in WD and software, the above transform unit size condition is missing.

Proposal

This contribution proposes the same condition (lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4) using the following lfnstEnabledFlag for whole WD.

The variable lfnstEnabledFlag is set equal to lfnst_idx && nTbW >= 4 && nTbH >= 4 ? 1 : 0

Effect on scaling list process

The proposed change affects scaling list process when scaling_matrix_for_lfnst_disabled_flag is equal to 1. In the current specification and software, the 2xN and Nx2 chroma scaling lists are not applied even if LFNST is not applied when scaling_matrix_for_lfnst_disabled_flag is equal to 1. This bug is fixed with this proposal.

|  | cIdx | nTbW | nTbH | lfnstIdx | LFNST | Scaling matrix |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | scaling_matrix_for_ lfnst_disabled_flag = 0 | scaling_matrix_for_ lfnst_disabled_flag = 1 |
| SIGNLE_TREE | 0 | 4 | 4 | 1 | Y | Y | N |
| SIGNLE_TREE | 1 | 2 | 2 |  | N | Y | Y (proposal) N (VVC7) |
| DUAL_TREE | 0 | 4 | 4 | 1 | Y | Y | N |
| DUAL_TREE | 1 | 2 | 2 | - (inferred to be 0) | N | Y | Y |

Effect on Implicit MTS

The proposed change doesn’t affect implicit MTS process and results since trTypeHor and tyTypeVer is always 0 in chroma component (cIdx > 0) irrespective of lfnstIdx and transform unit size. Thus the proposal is just an editorial change to avoid confusions.

Proposed text changes

We propose the following text changes. The added texts are highlighted in yellow, and deleted texts are marked by red strikethrough.

- 8.7.3			Scaling process for transform coefficients

…

The (nTbW)x(nTbH) array dz is set equal to the (nTbW)x(nTbH) array TransCoeffLevel[ xTbY ][ yTbY ][ cIdx ].

The variable lfnstEnabledFlag is set equal to lfnst_idx && nTbW >= 4 && nTbH >= 4 ? 1 : 0.

For the derivation of the scaled transform coefficients d[ x ][ y ] with x = 0..nTbW − 1, y = 0..nTbH − 1, the following applies:

- The intermediate scaling factor m[ x ][ y ] is derived as follows:

- If one or more of the following conditions are true, m[ x ][ y ] is set equal to 16:

- sps_scaling_list_enabled_flag is equal to 0.

- pic_scaling_list_present_flag is equal to 0.

- transform_skip_flag[ xTbY ][ yTbY ][ cIdx ] is equal to 1.

- scaling_matrix_for_lfnst_disabled_flag is equal to 1 and lfnstEnabledFlaglfnst_idx[ xTbY ][ yTbY ] is not equal to 0.

- Otherwise, the following applies:

…

- 8.7.4			Transformation process for scaled transform coefficients

- 8.7.4.1		General

Inputs to this process are:

- a luma location ( xTbY, yTbY ) specifying the top-left sample of the current luma transform block relative to the top-left luma sample of the current picture,

- a variable nTbW specifying the width of the current transform block,

- a variable nTbH specifying the height of the current transform block,

- a variable cIdx specifying the colour component of the current block,

- an (nTbW)x(nTbH) array d[ x ][ y ] of scaled transform coefficients with x = 0..nTbW − 1, y = 0..nTbH − 1.

Output of this process is the (nTbW)x(nTbH) array res[ x ][ y ] of residual samples with x = 0..nTbW − 1, y = 0..nTbH − 1.

The variable lfnstEnabledFlag is set equal to lfnst_idx && nTbW >= 4 && nTbH >= 4 ? 1 : 0.

When lfnstEnabledFlag is not equal to 0 lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4, the following applies:

- The variables predModeIntra, nLfnstOutSize, log2LfnstSize, nLfnstSize, and nonZeroSize are derived as follows:

predModeIntra = ( cIdx = = 0 )  ?  IntraPredModeY[ xTbY ][ yTbY ]  :  IntraPredModeC[ xTbY ][ yTbY ]	(1149)

nLfnstOutSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  48  :  16	(1150)

log2LfnstSize = ( nTbW >= 8  &&  nTbH  >= 8 )  ?  3  :  2	(1151)

nLfnstSize = 1  <<  log2LfnstSize		(1152)

nonZeroSize = ( ( nTbW = = 4  &&  nTbH  = = 4 )  | |  ( nTbW = = 8  &&  nTbH  = = 8 ) )  ?  8  :  16	(1153)

- When intra_mip_flag[ xTbY ][ yTbY ] is equal to 1 and cIdx is equal to 0, predModeIntra is set equal to INTRA_PLANAR.

- When predModeIntra is equal to either INTRA_LT_CCLM, INTRA_L_CCLM, or INTRA_T_CCLM, predModeIntra is derived as follows:

- If intra_mip_flag[ xTbY + nTbW * SubWidthC / 2 ][ yTbY + nTbH * SubHeightC / 2 ] is equal to 1, predModeIntra is set equal to INTRA_PLANAR.

- Otherwise, if CuPredMode[ 0 ][ xTbY + nTbW * SubWidthC / 2 ][ yTbY + nTbH * SubHeightC / 2 ] is equal to MODE_IBC or MODE_PLT, predModeIntra is set equal to INTRA_DC.

- Otherwise, predModeIntra is set equal to IntraPredModeY[ xTbY + nTbW * SubWidthC / 2 ][ yTbY + nTbH * SubHeightC / 2 ].

- The wide angle intra prediction mode mapping process as specified in clause 8.4.5.2.6 is invoked with predModeIntra, nTbW, nTbH and cIdx as inputs, and the modified predModeIntra as output.

- The values of the list u[ x ] with x = 0..nonZeroSize − 1 are derived as follows:

xC = DiagScanOrder[ 2 ][ 2 ][ x ][ 0 ]		(1154)

yC = DiagScanOrder[ 2 ][ 2 ][ x ][ 1 ]		(1155)

u[ x ] = d[ xC ][ yC ] 			(1156)

- The one-dimensional low frequency non-separable transformation process as specified in clause 8.7.4.2 is invoked with the input length of the scaled transform coefficients nonZeroSize, the transform output length nTrS set equal to nLfnstOutSize, the list of scaled non-zero transform coefficients u[ x ] with x = 0..nonZeroSize − 1, and the intra prediction mode for LFNST set selection predModeIntra as inputs, and the list v[ x ] with x = 0..nLfnstOutSize − 1 as output.

- The array d[ x ][ y ] with x = 0..nLfnstSize − 1, y = 0..nLfnstSize − 1 is derived as follows:

- If predModeIntra is less than or equal to 34, the following applies:

d[ x ][ y ] =  ( y < 4 )  ?  v[ x + ( y << log2LfnstSize ) ]  : 	(1157)
	( ( x < 4 )  ?  v[ 32 + x + ( ( y − 4 ) << 2 ) ]  :  d[ x ][ y ] )

- Otherwise, the following applies:

d[ x ][ y ] = ( x < 4 )  ?  v[ y + ( x << log2LfnstSize ) ]  : 	(1158)
	( ( y < 4 )  ?  v[ 32 + y + ( ( x − 4 ) << 2 ) ]  :  d[ x ][ y ] )

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnstEnabledFlaglfnst_idx[ x0 ][ y0 ] is equal to 0 and intra_mip_flag[ x0 ][ y0 ] is equal to 0

- Otherwise, implicitMtsEnabled is set equal to 0.

The variable trTypeHor specifying the horizontal transform kernel and the variable trTypeVer specifying the vertical transform kernel are derived as follows:

- If one or more of the following conditions is true, trTypeHor and trTypeVer are set equal to 0.

- cIdx is greater than 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT and lfnstEnabledFlag lfnst_idx is not equal to 0

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 38 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise (cu_sbt_flag is equal to 0), trTypeHor and trTypeVer are derived as follows:

trTypeHor = ( nTbW >= 4  &&  nTbW <= 16 ) ? 1 : 0	(1159)

trTypeVer = ( nTbH >= 4  &&  nTbH <= 16 ) ? 1 : 0	(1160)

- Otherwise, trTypeHor and trTypeVer are specified in Table 37 depending on mts_idx.

The variables nonZeroW and nonZeroH are derived as follows:

- If lfnstEnabledFlag lfnst_idx[ xTbY ][ yTbY ] is not equal to 0 and both nTbW and nTbH are greater than or equal to 4 is not equal to 0, the following applies:

nonZeroW = ( nTbW = = 4  | |  nTbH = = 4 )  ?  4  :  8	(1161)

nonZeroH = ( nTbW = = 4  | |  nTbH = = 4 )  ?  4  :  8	(1162)

- Otherwise, the following applies:

nonZeroW = Min( nTbW, ( trTypeHor > 0 )  ?  16  :  32 )	(1163)

nonZeroH = Min( nTbH, ( trTypeVer > 0 )  ?  16  :  32 )	(1164)

The (nTbW)x(nTbH) array r of residual samples is derived as follows:

…

Conclusion

This contribution proposes a fix related to LFNST condition. It is recommended to adopt this fix in the next VVC WD.

Reference

[1]	M. Koo, M. Salehifar, J. Lim, S. Kim, “CE6: Reduced Secondary Transform (RST) (CE6-3.1),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-N0193, 14th Meeting: Geneva, CH, 19–27 March 2019.

[2]	B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7),” Joint Video Expert Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, 16th Meeting: Geneva, CH, 1–11 October 2019.

Patent rights declaration(s)

Sharp Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).