| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0140-v1 |
| --- | --- |

| Title: | On chroma processing simplification for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alexey Filippov, Vasily Rufitskiy, Elena Alshina | Email: | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com elena.alshina@huawei.com |
| Source: | Huawei Technologies Co., LTD |  |  |

_____________________________

# Abstract

This contribution proposes to simplify chroma processing in intra prediction by disabling padding. This proposal targets to improve implementation freedom of intra prediction process. As compared to the VTM- 7.0 software, the simulation results for this method reportedly show no BD-rate changes for AI, RA and LB configurations.

# Introduction and problem statement

As prescribed Clause 8.4.5.2.12 of the current VVC specification draft, padding of main reference side samples is performed even for chroma components, although linear (i.e. 2-tap) interpolation filter is applied to chroma components. Hence, these padded samples are never referenced. Thus, the current formulation in Clause 8.4.5.2.12 restricts implementation freedom that harms one of the basic principles behind the VVC design.

# Proposed method

It is proposed to perform padding of reference samples only for luma component that 4-tap interpolation filter is applied to. To interpolate chrominance components, linear (i.e. 2-tap) filter is used in intra prediction process, and, therefore, padded samples are not required to get the values of predicted samples.

The need for padding is caused by removal of the check that a sample being predicted is obtained from a reference sample using an integer subsample position. In this case, interpolation filter would overlap the boundary of reference samples (which is a doubled width or doubled height). However, the samples that are further multiplied by interpolation filter coefficients do not affect the obtained result since coefficients that correspond to these samples are equal to zero when fractional position is zero.

Comparison of sample interpolation processes defined in [1] and in [3] (see Table 1) shows that according to both of these documents for each predicted chroma sample a check is performed on whether a subsample position is integer or fractional (“If iFact is not equal to 0”). When this position is non-fractional (integer), reference sample is copied to predicted sample position with no interpolation filter applied (“predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]”)

Table 1. Comparison of interpolation processes for HEVC and VVC

| HEVC [3], p. 159 | VVC, JVET-P2001-v14 [1] |
| --- | --- |
| … - Depending on the value of iFact, the following applies: – If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows: predSamples[ x ][ y ] = ( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 ) >> 5 (8-66) – Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows: predSamples[ x ][ y ] = ref[ y + iIdx + 1 ] (8-67) | - Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies: - If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows: predSamples[ x ][ y ] = ( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 ) >> 5 (344) - Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows: predSamples[ x ][ y ] = ref[ y + iIdx + 1 ] (345) |

However, luminance processing described in the HEVC specification is different from the one in the current VVC specification draft. In HEVC, luminance samples are also checked if their position corresponds to non-fractional subsample position in reference sample array, and interpolation filter is also not applied when subsample position is non-fractional.

In the current VVC specification, such a check is performed just for chrominance samples but not for luminance. Instead, luminance reference array is extended by samples’ padding. It could be considered to remove padding and to perform the same checks in luma and chroma in order to make the current VVC specification more consistent and HEVC-compliant.

It is worth noting that earlier versions of VTM software used a check on whether position of a sample is non-fractional. VTM software prior to [4] (commit 199c7dd), used the following lines:

p[0] = refMain[refMainIndex - 1];

p[1] = refMain[refMainIndex];

p[2] = refMain[refMainIndex + 1];

p[3] = f[3] != 0 ? refMain[refMainIndex + 2] : 0;

In [4], these were further changed to

p[0] = refMain[deltaInt + x];

p[1] = refMain[deltaInt + x + 1];

p[2] = refMain[deltaInt + x + 2];

p[3] = refMain[deltaInt + x + 3];

This change [4] removes the check of filter coefficient being equal to 0 (f[3] != 0 ) , and instead, it introduces additional reference sample padding.

# Specification text changes

The specification text changes are provided on top of the VVC specification draft version 7 text provided in [1].

The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- If predModeIntra is greater than or equal to 34, the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = 0..nTbW + refIdx + 1	(326)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbH ) ],
	with x = −nTbH..−1	(327)

- Otherwise,

ref[ x ] = p[ −1 − refIdx + x ][ −1 − refIdx ], with x = nTbW + 2 + refIdx..refW + refIdx	(328)

- If cIdx is equal to 0, the additional samples ref[ refW + refIdx +x ] with x = 1..( Max( 1, nTbW / nTbH ) * refIdx + 1 ) are derived as follows:

ref[ refW + refIdx + x ] = p[ −1 + refW ][ −1 − refIdx ]	(329)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( y + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(330)

iFact = ( ( y + 1 + refIdx ) * intraPredAngle ) & 31	(331)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(332)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(333)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(334)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(335)

- Otherwise (predModeIntra is less than 34), the following ordered steps apply:

- The reference sample array ref[ x ] is specified as follows:

- The following applies:

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = 0..nTbH + refIdx + 1	(336)

- If intraPredAngle is less than 0, the main reference sample array is extended as follows:

ref[ x ] = p[ −1 − refIdx + Min( ( x * invAngle + 256 )  >>  9, nTbW ) ][ −1 − refIdx ],
	with x = −nTbW..−1	(337)

- Otherwise,

ref[ x ] = p[ −1 − refIdx ][ −1 − refIdx + x ], with x = nTbH + 2 + refIdx..refH + refIdx	(338)

- If cIdx is equal to 0, the additional samples ref[ refH + refIdx +x ] with x = 1..( Max( 1, nTbH / nTbW ) * refIdx + 1) are derived as follows:

ref[ refH + refIdx +x ] = p[ −1 + refH ][ −1 − refIdx ]	(339)

- The values of the prediction samples predSamples[ x ][ y ], with x = 0..nTbW − 1, y = 0..nTbH − 1 are derived as follows:

- The index variable iIdx and the multiplication factor iFact are derived as follows:

iIdx = ( ( ( x + 1 + refIdx ) * intraPredAngle )  >>  5 ) + refIdx	(340)

iFact = ( ( x + 1 + refIdx ) * intraPredAngle ) & 31	(341)

- If cIdx is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = filterFlag  ?  fG[ iFact ][ j ]  :  fC[ iFact ][ j ]	(342)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 )  >>  6 )	(343)

- Otherwise (cIdx is not equal to 0), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(344)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(345)

# Experimental results

Test were carried out in accordance with [2]. The obtained results reportedly show no changes of BD-rate as compared with the anchor, which is VTM7.0 software (see Table 2). No memory issues related to this modifications have been reported during valgrind check (see Table 3).

Table 2. Simulation results for AI, RA and LDB

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 3. Results of valgrind check

started @ Tue Dec 31 22:39:59 2019 ==104033== Warning: set address range perms: large range [0x395d8040, 0x6809d840) (undefined) ==104033== Warning: set address range perms: large range [0x15659040, 0x2cbbbc40) (undefined) POC 0 TId: 0 ( I-SLICE, QP 37 ) 51008 bits [Y 30.5492 dB U 38.4659 dB V 38.6348 dB] [ET 1191 ] [L0 ] [L1 ] [MD5:f94d751b58aad2ba92399e884059cfa5,999f5b0462d5c69c2f781b179b71de53,26cce1ae66b48cbf3abb5769622b92a1] Total Frames ¦ Bitrate Y-PSNR U-PSNR V-PSNR YUV-PSNR 1 a 382.5600 30.5492 38.4659 38.6348 31.9790 ==104033== Warning: set address range perms: large range [0x395d8028, 0x6809d858) (noaccess) ==104033== Warning: set address range perms: large range [0x15659028, 0x2cbbbc58) (noaccess) finished @ Tue Dec 31 22:59:52 2019 Total Time: 1192.860 sec. [user] 1192.912 sec. [elapsed] ==104033== ==104033== HEAP SUMMARY: ==104033== in use at exit: 113,696 bytes in 3 blocks ==104033== total heap usage: 3,172,778 allocs, 3,172,775 frees, 1,795,197,899 bytes allocated ==104033== ==104033== LEAK SUMMARY: ==104033== definitely lost: 40,960 bytes in 1 blocks ==104033== indirectly lost: 0 bytes in 0 blocks ==104033== possibly lost: 0 bytes in 0 blocks ==104033== still reachable: 72,736 bytes in 2 blocks ==104033== suppressed: 0 bytes in 0 blocks ==104033== Rerun with --leak-check=full to see details of leaked memory ==104033== ==104033== For counts of detected and suppressed errors, rerun with: -v ==104033== ERROR SUMMARY: 0 errors from 0 contexts (suppressed: 0 from 0)

# Conclusion

It is proposed to adopt the proposed modification to the VVC 8.0 software and to the VVC specification draft 8.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, 16th JVET meeting, Geneva, October 2019.

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET Meeting, Geneva, Switzerland, March 2019.

- ISO/IEC 23008-2. ITU-T Recommendation H.265.2: Reference software for ITU-T H.265 high efficiency video coding.

- Merge request 787 of the VVC software https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/787.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).