| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0146 |
| --- | --- |

| Title: | Simplified bypass coding of transform regular residual |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yusuke Kato Kiyofumi Abe Tadamasa Toma | Tel: Email: | +81-80-3083-1171 kato.yusuke003@jp.panasonic.com abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution proposes a simplified bypass coding of transform residual coding. Update of QState in bypass coding is removed and derivation of ZeroPos[n] is changed not to use QState. Two methods are proposed.

- Method 1: ZeroPos[n] is defined to (1 << rice).

- Method 2: On top of Method 1, ZeroPos[n] is changed depending on inter block or not.

Method 1 reportedly provides 0.01%, 0.01%, and 0.01% luma BD-rates for the AI, RA, and LB settings, respectively, over the VTM-7.0 anchor under CTC, and Method 2 reportedly provides 0.00%, 0.00%, and -0.01% luma BD-rates for the AI, RA, and LB settings, respectively.

Introduction

In VTM7.0, the transform residual levels of a coefficient group (CG) are coded in the following three passes:

- Pass 1: for (n=firstPosMode0; n >= 0 && CCB>=4; n--)

sig_coeff_flag[n]

Gt 1 flag[n] (abs_level_gtx_flag[0])

parity flag[n] (par_level_flag)

Gt 3 flag[n] (abs_level_gtx_flag[1])

QState = QStateTransTable[ QState ][ AbsLevelPass1[ xC ][ yC ] & 1 ]

firstPosMode1 = n

- Pass 2(a): for (n=firstPosMode0; n > firstPosMode1; n--)

abs_remainder[n]

- Pass 2(b): for (n=firstPosMode0; n > firstPosMode1; n--)

dec_abs_level[ n ]

QState = QStateTransTable[ QState ][ AbsLevel[ xC ][ yC ] & 1 ]

- Pass 3: for (n = numSbCoeff-1; n >=0; n--)

coeff_sign_flag[n]

CCB means the number of context coded bins. The dependent scalar quantizer state QState is updated for each coefficient before and after CCB exceeds the limit. Before CCB exceeds the limit, QState is used to derive ctxInc of sig_coeff_flag. After CCB exceeds the limit, QState is used to derive ZeroPos[n] as follows:

ZeroPos[ n ] = ( QState < 2 ? 1 : 2 ) << cRiceParam.

cRiceParam is the Rice parameter associated with the current coefficient.

It is noted that in Pass1, Qstate can be updated using only the parity flag, whereas in Pass2(b), all the  coefficients need to be decoded to update Qstate.

Proposal

This contribution proposes to remove QState update in bypass coding pass, that is after CCB reaches the limit. Two methods of deriving ZeroPos are proposed that do not use Qstate.

Method 1:

ZeroPos[n] = 1 << cRiceParam

Method 2:

ZeroPos[n] =(CuPredMode[ chType ][ x0 ][ y0 ] == MODE_INTER ? 1 : 2) << cRiceParam

Removing the Qstate update for each coefficient in bypass coding reduces hardware complexity.

Simulation results

Table 1 and 2 show the CTC results of proposed method 1 and method 2, respectively. Table 3 and 4 show the LowQP results of proposed method 1 and method 2, respectively.

Table 1: Simulation results of proposed method 1 in CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.01% | 0.07% | 100% | 100% |
| Class A2 | 0.01% | 0.02% | -0.01% | 100% | 100% |
| Class B | 0.00% | 0.04% | 0.00% | 100% | 100% |
| Class C | 0.04% | 0.14% | 0.12% | 100% | 100% |
| Class E | 0.00% | 0.03% | -0.02% | 101% | 100% |
| Overall | 0.01% | 0.05% | 0.03% | 100% | 100% |
| Class D | 0.04% | -0.03% | 0.05% | 101% | 99% |
| Class F | 0.01% | 0.01% | 0.07% | 100% | 100% |
| TGM | -0.01% | 0.01% | 0.01% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.01% | 101% | 101% |
| Class A2 | -0.01% | 0.10% | 0.02% | 101% | 101% |
| Class B | 0.02% | 0.05% | 0.15% | 101% | 101% |
| Class C | 0.01% | 0.09% | 0.13% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.09% | 101% | 101% |
| Class D | 0.02% | -0.06% | -0.17% | 100% | 101% |
| Class F | -0.04% | 0.02% | 0.09% | 100% | 100% |
| TGM | 0.00% | -0.01% | 0.02% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.22% | -0.22% | 99% | 99% |
| Class C | 0.01% | -0.53% | -0.27% | 104% | 98% |
| Class E | 0.05% | 0.41% | 0.01% | 101% | 98% |
| Overall | 0.01% | -0.17% | -0.18% | 101% | 99% |
| Class D | -0.05% | 0.25% | -0.43% | 105% | 103% |
| Class F | -0.07% | -0.12% | 0.66% | 100% | 99% |
| TGM | 0.01% | -0.06% | -0.09% | 101% | 100% |

Table 2: Simulation results of proposed method 2 in CTC

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.05% | 100% | 100% |
| Class A2 | 0.00% | 0.03% | -0.02% | 100% | 100% |
| Class B | 0.00% | 0.02% | -0.09% | 101% | 100% |
| Class C | 0.01% | 0.06% | -0.04% | 100% | 100% |
| Class E | 0.00% | -0.06% | -0.02% | 100% | 100% |
| Overall | 0.00% | 0.01% | -0.03% | 100% | 100% |
| Class D | -0.01% | 0.04% | 0.01% | 100% | 100% |
| Class F | 0.00% | 0.01% | 0.03% | 100% | 100% |
| TGM | 0.00% | 0.02% | 0.02% | 100% | 101% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.02% | 0.01% | 100% | 100% |
| Class A2 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class B | 0.00% | -0.02% | -0.01% | 100% | 100% |
| Class C | 0.01% | -0.03% | -0.17% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.02% | -0.05% | 100% | 100% |
| Class D | 0.03% | 0.03% | -0.14% | 100% | 100% |
| Class F | -0.02% | 0.07% | -0.05% | 100% | 100% |
| TGM | 0.00% | -0.01% | -0.01% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.17% | -0.11% | 99% | 99% |
| Class C | 0.00% | -0.19% | 0.08% | 104% | 98% |
| Class E | -0.02% | 0.39% | -0.39% | 100% | 98% |
| Overall | -0.01% | -0.03% | -0.12% | 101% | 98% |
| Class D | -0.03% | -0.11% | -0.29% | 104% | 102% |
| Class F | -0.07% | -0.29% | 0.53% | 100% | 99% |
| TGM | -0.01% | 0.00% | -0.09% | 101% | 100% |

Table 3: Simulation results of proposed method 1 in LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.14% | 0.16% | 0.17% | 99% | 98% |
| Class A2 | 0.16% | 0.18% | 0.19% | 99% | 97% |
| Class B | 0.11% | 0.14% | 0.16% | 99% | 98% |
| Class C | 0.16% | 0.20% | 0.21% | 99% | 99% |
| Class E | 0.07% | 0.13% | 0.10% | 99% | 98% |
| Overall | 0.13% | 0.16% | 0.17% | 99% | 98% |
| Class D | 0.16% | 0.15% | 0.21% | 98% | 94% |
| Class F | 0.11% | 0.08% | 0.11% | 99% | 99% |
| TGM | 0.08% | 0.03% | 0.02% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.09% | 0.14% | 0.10% | 100% | 99% |
| Class A2 | 0.10% | 0.11% | 0.11% | 100% | 98% |
| Class B | 0.06% | 0.07% | 0.10% | 100% | 99% |
| Class C | 0.08% | 0.12% | 0.10% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.08% | 0.11% | 0.10% | 100% | 99% |
| Class D | 0.08% | 0.19% | 0.07% | 99% | 98% |
| Class F | 0.11% | 0.17% | 0.19% | 100% | 99% |
| TGM | 0.09% | 0.06% | 0.06% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.07% | 0.10% | 100% | 98% |
| Class C | 0.09% | 0.10% | 0.16% | 99% | 102% |
| Class E | 0.03% | 0.10% | 0.04% | 100% | 99% |
| Overall | 0.06% | 0.09% | 0.10% | 100% | 100% |
| Class D | 0.08% | 0.18% | 0.00% | 100% | 101% |
| Class F | -0.05% | -0.01% | -0.08% | 100% | 99% |
| TGM | -0.13% | -0.27% | -0.23% | 99% | 99% |

Table 4: Simulation resutls of proposed method 2 in LowQP

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.03% | 0.03% | 0.04% | 100% | 99% |
| Class A2 | 0.08% | 0.07% | 0.07% | 99% | 98% |
| Class B | 0.02% | 0.03% | 0.03% | 99% | 99% |
| Class C | -0.02% | -0.04% | -0.04% | 99% | 102% |
| Class E | -0.02% | 0.01% | -0.02% | 99% | 99% |
| Overall | 0.01% | 0.02% | 0.02% | 99% | 100% |
| Class D | -0.03% | -0.08% | -0.04% | 99% | 94% |
| Class F | 0.04% | -0.04% | 0.01% | 100% | 101% |
| TGM | 0.11% | 0.04% | 0.04% | 99% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.09% | 0.05% | 100% | 99% |
| Class A2 | 0.09% | 0.13% | 0.12% | 100% | 99% |
| Class B | 0.04% | 0.07% | 0.06% | 99% | 99% |
| Class C | 0.05% | 0.04% | 0.05% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.06% | 0.08% | 0.07% | 100% | 100% |
| Class D | 0.05% | 0.10% | -0.02% | 99% | 99% |
| Class F | -0.04% | 0.08% | 0.08% | 100% | 98% |
| TGM | 0.08% | 0.03% | 0.03% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0(100Frm,4K,LowQP) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.08% | 0.10% | 99% | 98% |
| Class C | 0.08% | 0.09% | 0.09% | 99% | 98% |
| Class E | 0.00% | 0.05% | 0.02% | 99% | 99% |
| Overall | 0.05% | 0.08% | 0.08% | 99% | 98% |
| Class D | 0.05% | 0.08% | -0.01% | 100% | 98% |
| Class F | 0.03% | 0.05% | -0.09% | 100% | 99% |
| TGM | 0.00% | -0.10% | -0.09% | 99% | 99% |

Conclusion

This contribution proposes to remove QState update from bypass coding pass. To fulfill this aspect, two methods of deriving ZeroPos, not using QState are proposed. Method 2 has almost no coding loss in CTC and lowQP. We suggest to adopt method 2 into the test model.

References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).