| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0147-v1 |
| --- | --- |

| Title: | AHG9: On JCCR signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Tadamasa Toma 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com toma.tadamasa@jp.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

This contribution proposes to skip signaling of sps_joint_cbcr_enabled_flag when ChromaArrayType is equal to 0. With this modification, all SPS chroma related syntaxes can avoid signaling in the format that does not use chroma blocks.

Introduction

If the chroma format is monochrome format or 4:4:4 with separate colour plane format, all chroma related tools are not applied. So, the header layer syntaxes of chroma tools should follow the following rule in this case.

SPS 	: Skip signaling based on ChromaArrayType (or chroma_format_idc)

PPS 	: Signal all syntaxes but prohibit to enable it

APS	: Signal all syntaxes

PH 	: Skip signaling based on SPS and PPS parameters

SH 	: Skip signaling based on SPS, PPS and PH parameters

However, only sps_joint_cbcr_enabled_flag breaks this rule and signaled in SPS even for the format that does not use chroma blocks.

This contribution proposes to skip signaling of sps_joint_cbcr_enabled_flag when ChromaArrayType is equal to 0, and achieve the above rule in all syntaxes.

Proposed changes

Proposed changes over the draft specification [1] are presented below.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| chroma_format_idc | u(2) |
| if( chroma_format_idc == 3 ) |  |
| separate_colour_plane_flag | u(1) |
| … |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) { |  |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

sps_joint_cbcr_enabled_flag equal to 0 specifies that the joint coding of chroma residuals is disabled. sps_joint_cbcr_enabled_flag equal to 1 specifies that the joint coding of chroma residuals is enabled. When sps_joint_cbcr_enabled_flag is not present in the bitstream, the value of sps_joint_cbcr_enabled_flag is inferred to be equal to 0.

Conclusion

This contribution proposes to skip signaling of sps_joint_cbcr_enabled_flag when ChromaArrayType is equal to 0. With this modification, all SPS chroma related syntaxes can avoid signaling in the format that does not use chroma blocks. We propose to adopt it into VVC.

References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, JVET 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).