| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0150 |
| --- | --- |

| Title: | Fix for ALF virtual boundary processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, J. Ström, Z. Zhang, J. Enhorn | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

ALF virtual boundary processing avoids increasing the size of the line buffers for adaptive loop filtering. This contribution claims that this processing, in some cases, can introduce visual artifacts due to the use of extreme padding for the closest row on each side of the virtual horizontal CTU boundary without compensating the filter strength for that. In this contribution, two alternative solutions are proposed. Alternative 1 moves the virtual boundary one row upwards and reduce the amount of padding such that always at least the neighbouring rows to the center row are available. Alternative 2 reduce the filter strength by increasing the right shift for ALF filtering of the rows adjacent to the virtual horizontal CTU boundary. Alternative 1 increase the line buffers by two lines for luma and two lines for chroma. Alternative 2 has similar complexity as VTM.

The BD rate impact for luma for alternative 1:

- AI -0.01%, RA 0.01%, LDB -0.02%

The BD rate impact for luma for alternative 2:

- AI 0.00%, RA 0.00%, LDB 0.02%

Similar encoding and decoding time.

# Introduction

ALF virtual boundary processing in VVC defines a virtual horizontal CTU boundary 4 samples above the top row of the below CTU for luma and 2 samples above the top row of the below CTU for chroma. ALF symmetry padding is used to avoid using samples below the virtual boundary when filtering above it, and to avoid using samples above the virtual boundary when filtering below it.

# Problem

When filtering samples immediately adjacent to the virtual horizontal boundary, ALF virtual boundary processing can give rise to visual artifacts, as exemplified in the frame from BasketballDrill shown below:

Figure 1: Artifacts from ALF virtual boundary processing

The reason for these artifacts is that the contribution of the filtering from neighboring samples of the current row is amplified up to 5 times for luma and 3 times for chroma due to padding of above and below samples with the samples of the current row which is illustrated for luma below:

Figure 2: Illustration of the effect of the contribution from neighbouring samples when filtering samples adjacent to the virtual horizontal CTU boundary for luma.

The padding for other rows further away from the virtual horizontal CTU boundary has not shown to produce any artifacts. This is likely due to the fact that contributions from neighbouring samples above and below the current sample can be used which preserves most of the intended effect of the filter.

# Alternative solution 1

In this alternative the extreme padding when only samples from the center row is available is avoided by allowing to use one row on the other side of the virtual boundary in addition to the center row thus having one row above and one row below the center row when filtering a row adjacent to the virtual boundary. The virtual boundary is moved one step upwards such that the virtual boundary is 5 rows above the top row of the CTU for luma and 3 rows above the top row of the CTU for chroma. One drawback of this solution is that it increases the line buffers by two lines for luma and two lines for chroma but still significantly less than the amount of line buffers that would be required without ALF virtual boundary processing.  It also requires storage of classifications from a row above the virtual boundary for luma when filtering below the virtual boundary but the amount of storage for that is relatively small (about 800 bytes for 4K).

Figure 3: Illustration of the effect of the contribution from neighbouring samples when filtering samples adjacent to the virtual horizontal CTU boundary for luma for alternative 1.

Figure 4: BasketballDrill LDB at QP 37. Left: VTM-7.0, Right: Alternative 1

# Alternative solution 2

The proposed solution is to reduce the filter strength for both luma and chroma when filtering rows adjacent to the ALF virtual boundary.

Figure 5: BasketballDrill LDB at QP 37. Left: VTM-7.0, Right: Alternative 2

# Text Alternative solution 1

The proposed changes to the VVC specification [1] are highlighted with strikethrough and in yellow below:

Table 43 – Specification of y1, y2 and y3 according to the vertical luma sample position y and applyAlfLineBufBoundary

| Condition | y1 | y2 | y3 |
| --- | --- | --- | --- |
| ( y = = CtbSizeY − 5 ¦ ¦ y = = CtbSizeY − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 0 | 0 | 0 |
| ( y = = CtbSizeY − 6 ¦ ¦ y = = CtbSizeY − 5 ) && ( applyAlfLineBufBoundary = = 1 ) | 1 | 1 | 1 |
| ( y = = CtbSizeY − 7 ¦ ¦ y = = CtbSizeY − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 1 | 2 | 2 |
| Otherwise | 1 | 2 | 3 |

Table 44 – Specification of y1 and y2 according to the vertical chroma sample position y and applyAlfLineBufBoundary

| Condition | y1 | y2 |
| --- | --- | --- |
| ( y = = ctbHeightC − 2 ¦ ¦ y = = ctbHeightC − 3 ) && ( applyAlfLineBufBoundary = = 1 ) | 0 | 0 |
| ( y = = ctbHeightC − 3 ¦ ¦ y = = ctbHeightC − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 1 | 1 |
| Otherwise | 1 | 2 |

8.8.5.3 Derivation process for ALF transpose and filter index for luma samples

..

- The variables applyAlfLineBufBoundary, minY, maxY, and ac[ x ][ y ] are derived as follows:

- If y4 is equal to ( CtbSizeY − 8 ) and one of the following condition is true, minY is set equal to −2, maxY is set equal to 3, applyAlfLineBufBoundary is set equal to 1, and ac[ x ][ y ] is set equal to 96.

  - The bottom boundary of the current coding tree block is the bottom boundary of the picture and pic_height_in_luma_samples − yCtb > CtbSizeY − 4.

  - The bottom boundary of the current coding tree block is not the bottom boudary of the picture.

- Otherwise, if y4 is equal to ( CtbSizeY − 4 ) and one of the following condition is true, minY is set equal to 0, maxY is set equal to 5, applyAlfLineBufBoundary is set equal to 1, and ac[ x ][ y ] is set equal to 96.

  - The bottom boundary of the current coding tree block is the bottom boundary of the picture and pic_height_in_luma_samples − yCtb > CtbSizeY − 4.

  - The bottom boundary of the current coding tree block is not the bottom boudary of the picture

- Otherwise, minY is set equal to −2 and maxY is set equal to 5, applyAlfLineBufBoundary is set equal to 0 and ac[ x ][ y ] is set equal to 64.

# Text Alternative solution 2

The proposed changes to the VVC specification [1] are highlighted in yellow below:

sum = curr + ( ( sum + 64 ) >> alfShiftY )		(1425)

Table 43 – Specification of y1, y2, y3 and alfShiftY according to the vertical luma sample position y and applyAlfLineBufBoundary

| Condition | alfShiftY | y1 | y2 | y3 |
| --- | --- | --- | --- | --- |
| ( y = = CtbSizeY − 5 ¦ ¦ y = = CtbSizeY − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 10 | 0 | 0 | 0 |
| ( y = = CtbSizeY − 6 ¦ ¦ y = = CtbSizeY − 3 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 1 | 1 |
| ( y = = CtbSizeY − 7 ¦ ¦ y = = CtbSizeY − 2 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 2 | 2 |
| Otherwise | 7 | 1 | 2 | 3 |

sum = curr + ( ( sum + 64 ) >> alfShiftC )		(1466)

Table 44 – Specification of y1, y2 and alfShiftC according to the vertical chroma sample position y and applyAlfLineBufBoundary

| Condition | alfShiftC | y1 | y2 |
| --- | --- | --- | --- |
| ( y = = ctbHeightC − 2 ¦ ¦ y = = ctbHeightC − 3 ) && ( applyAlfLineBufBoundary = = 1 ) | 10 | 0 | 0 |
| ( y = = ctbHeightC − 1 ¦ ¦ y = = ctbHeightC − 4 ) && ( applyAlfLineBufBoundary = = 1 ) | 7 | 1 | 1 |
| Otherwise | 7 | 1 | 2 |

# Results

The objective performance for the proposal is shown below in comparison to VTM-7.0 [2] using CTC [3]. The objective results shows a very small impact on coding efficiency. Encoding time is measured on simulations run on a cluster with machines with different capability and is not reliable. Decoding time is measured by running test and anchor on same machine without yuv output. Decoding time is similar as for the anchor.

## Alternative 1

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,14% | -0,14% | 97% | 100% |
| Class A2 | 0,04% | -0,04% | -0,05% | 96% | 99% |
| Class B | 0,00% | -0,06% | -0,05% | 97% | 99% |
| Class C | -0,03% | -0,04% | -0,06% | 100% | 100% |
| Class E | -0,03% | 0,00% | -0,05% | 97% | 98% |
| Overall | -0,01% | -0,06% | -0,07% | 98% | 99% |
| Class D | -0,01% | -0,01% | -0,03% | 97% | 100% |
| Class F | -0,03% | -0,06% | -0,06% | 97% | 100% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,01% | -0,07% | -0,06% | 98% | 99% |
| Class A2 | 0,06% | 0,10% | 0,00% | 99% | 100% |
| Class B | 0,01% | 0,00% | 0,07% | 98% | 101% |
| Class C | -0,02% | -0,07% | -0,07% | 98% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0,01% | -0,02% | -0,01% | 98% | 100% |
| Class D | -0,07% | 0,14% | -0,20% | 98% | 101% |
| Class F | -0,07% | -0,01% | -0,09% | 98% | 99% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,02% | -0,09% | 0,13% | 96% | 101% |
| Class C | -0,07% | -0,32% | -0,12% | 98% | 99% |
| Class E | 0,00% | 0,41% | 0,13% | 100% | 101% |
| Overall | -0,02% | -0,04% | 0,04% | 98% | 100% |
| Class D | -0,08% | 0,24% | 0,36% | 100% | 100% |
| Class F | -0,08% | 0,03% | 0,45% | 98% | 99% |

## Alternative 2

|  | All intra Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,01% | 0,04% | 0,07% | 96% | 97% |
| Class A2 | 0,01% | 0,02% | 0,03% | 96% | 100% |
| Class B | 0,01% | 0,02% | 0,03% | 98% | 101% |
| Class C | -0,01% | 0,03% | 0,02% | 99% | 100% |
| Class E | 0,01% | 0,05% | 0,06% | 95% | 100% |
| Overall | 0,00% | 0,03% | 0,04% | 97% | 100% |
| Class D | 0,00% | 0,01% | 0,02% | 95% | 100% |
| Class F | -0,01% | 0,03% | 0,02% | 98% | 100% |

|  | Random access Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0,02% | -0,01% | 0,03% | 95% | 101% |
| Class A2 | 0,00% | 0,10% | 0,09% | 97% | 99% |
| Class B | 0,01% | 0,13% | 0,11% | 98% | 100% |
| Class C | -0,01% | -0,03% | -0,06% | 96% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0,00% | 0,05% | 0,04% | 97% | 100% |
| Class D | 0,00% | 0,07% | 0,06% | 97% | 101% |
| Class F | -0,04% | -0,01% | -0,01% | 98% | 99% |

|  | Low-delay B Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0,01% | -0,24% | 0,32% | 95% | 99% |
| Class C | 0,04% | -0,15% | 0,02% | 95% | 100% |
| Class E | 0,01% | 0,72% | -0,03% | 73% | 99% |
| Overall | 0,02% | 0,03% | 0,13% | 89% | 99% |
| Class D | 0,00% | -0,18% | -0,15% | 99% | 100% |
| Class F | 0,05% | 0,07% | 0,24% | 95% | 99% |

# Conclusion

It is proposed to adopt one of the proposed alternatives to the VVC draft and VTM software.

# References

- B. Bross, J. Chen, and S. Liu, "Versatile Video Coding (Draft 7)", Document of Joint Video Experts Team, JVET-P2001.

- VTM-7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, JVET-N1010

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).