| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0154 |
| --- | --- |

| Title: | AHG9: On picture header IRAP/GDR signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Muhammed Coban Vadim Seregin Adarsh K. Ramasubramonian Yao-Jen Chang Marta Karczewicz | Tel: Email: | +1 858 658 3973 mcoban@qti.qualcomm.com vseregin@qti.qualcomm.com martak@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

In this contribution the following methods are proposed to signal IRAP or GDR at picture header level:

- IRAP or GDR picture indicator in at the beginning of picture header.

- PH_IRAP_GDR_NUT indicating an IRAP or GDR picture.

- Signalling slice layer NUTs as PH_NUTs at picture header. Slice layer NUTs are replaced by SLICE_NUT.

- Recovery picture poc count signalling for mixed mixed nal unit type pictures.

# Introduction

In VVC (Draft 7) picture header (PH) does not contain IRAP indicator. IRAP or GDR detection requires locating an IRAP or GDR NUT at slice layer and locating its picture header that is preceding the slice layer data. Indicating IRAP at PH would simplify IRAP detection by directly identifying it at the beginning of the associated picture data. PH level indication can be achieved by using IRAP or GDR specific PH NUTs or indication in picture header.

# Proposal

## IRAP/GDR signalling in picture header

A flag indicating IRAP or GDR picture is indicated at the start of picture header.

==========================VVC draft 7 changes BEGIN============================

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| irap_or_gdr_pic_flag | u(1) |
| non_reference_picture_flag | u(1) |
| if( irap_or_gdr_pic_flag ) |  |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| … |  |

#### 7.4.3.6 Picture header RBSP semantics

The PH contains information that is common for all slices of the coded picture associated with the PH.

irap_or_gdr_pic_flag equal to 1 specifies the picture associated with the PH is an IRAP or GDR. irap_or_gdr_pic_flag equal to 0 specifies the picture associated with the PH is neither IRAP nor GDR picture.

non_reference_picture_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. non_reference_picture_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture.

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, it is inferred to be equal to 0.

==========================VVC draft 7 changes END==============================

Alternatively, u(2) field indicating IDR, CRA, GDR IRAP types is included in the picture header.

==========================VVC draft 7 changes BEGIN============================

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| irap_gdr_idc | u(2) |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| if(irap_gdr_idc = = 3) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| … |  |

#### 7.4.3.6 Picture header RBSP semantics

The PH contains information that is common for all slices of the coded picture associated with the PH.

irap_gdr_idc equal to 0 indicates a picture that is not IRAP or GDR. irap_gdr_pic_idc equal to 1 indicates IDR picture, irap_gdr_idc equal to 2 indicates CRA picture. irap_gdr_idc equal to 3 indicates GDR picture.

non_reference_picture_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. non_reference_picture_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture.

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture.

==========================VVC draft 7 changes END==============================

## PH_IRAP_NUT for indicating IRAP pictures

An additional PH_IRAP_GDR_NUT is introduced to indicate IRAP or GDR picture type.

==========================VVC draft 7 changes BEGIN============================

#### 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| if(nal_unit_type == PH_IRAP_GDR_NUT) |  |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| … |  |

#### 7.4.3.6 Picture header RBSP semantics

The PH contains information that is common for all slices of the coded picture associated with the PH.

non_reference_picture_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. non_reference_picture_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture.

gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, it is inferred to be equal to 0.

Table 5 – NAL unit type codes and NAL unit type classes

| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| --- | --- | --- | --- |
| … | … | … |  |
| 19 | PH_NUT | Picture header picture_header_rbsp( ) | non-VCL |
| 20 | PH_IRAP_GDR_NUT | Picture header for IRAP or GDR picture_header_rbsp( ) | non-VCL |
| … | … | .. |  |

==========================VVC draft 7 changes END==============================

## Signalling slice layer NUTs types in picture header

PHs are mandatory for each picture. Slice layer specific NUTs can be signalled at PH as PH NUT types and the slice layer NUTs can be replaced by SLICE_NUT indication with IRAP or GDR indication signalled in the slice header when mixed_nalu_types_in_pic_flag is equal to 1. The slice header level NAL unit types are derived from associated PH_NUT types except for mixed_nal_unit_case.

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if (mixed_nalu_types_in_pic_flag == 1 ) |  |
| irap_gdr_idc | u(2) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |

irap_gdr_idc equal to 0 indicates IDR_W_RADL picture. irap_gdr_pic_idc equal to 1 indicates IDR_N_LP, picture, irap_gdr_idc equal to 2 indicates CRA picture. irap_gdr_idc equal to 3 indicates GDR picture.

## recovery_poc_cnt signalling in slice header

In VVC (Draft 7) specification recovery_poc_cnt is signalled in PH if the picture is a GDR picture. For mixed NAL unit case where a GDR slice can be combined with IRAP slices, resulting in a non-GDR picture. The recovery_poc_cnt of the GDR slice needs to be signalled. It is proposed to signal the recovery_poc_cnt at subpicture/slice level or change the definition of the gdr_pic_flag to include mixed nal unit type in picture that has GDR slices.

#### 7.3.7.1 General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if (mixed_nalu_types_in_pic_flag == 1 && nal_unit_type == GDR_NUT ) |  |
| recovery_poc_cnt | ue(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |

  - gradual decoding refresh (GDR) picture: A picture for which each VCL NAL unit has nal_unit_type equal to GDR_NUT.

#### 7.4.3.6 Picture header RBSP semantics

The PH contains information that is common for all slices of the coded picture associated with the PH.

non_reference_picture_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. non_reference_picture_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture.

gdr_pic_flag equal to 1 specifies the picture associated with the PH is contains GDR picture slices. gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture.

# References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, 16th Meeting: Geneva, CH, 1–11 Oct. 2019.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).