| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0155 |
| --- | --- |

| Title: | AHG9: On separate colour plane coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Bappaditya Ray Adarsh K. Ramasubramonian Yao-Jen Chang Muhammed Coban Geert Van der Auwera Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com bray@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the VVC draft 7, colour_plane_id is signaled in a picture header, so when separate_colour_plane_flag is equal to 1 three colour planes are decoded as monochrome pictures. In the current design there is no possibility to signal various colour_plane_id for a picture. Moreover, slices of colour component planes can be signalled in an interleaved manner, but slices do not have any color plane indication. In this contribution, it is proposed to move colour_plane_id to slice header.

Secondly, since colour planes are coded as monochrome pictures without decoding dependencies between colour planes, it is proposed to modify signalling of enable flags for cross-components tools such as joint CbCr, adaptive color transform, palette and chroma BDPCM.

# Introduction

In VVC draft 7, colour_plane_id is signaled in a picture header. When separate colour plane coding is enabled by setting separate_colour_plane_flag equal to 1, colour planes are coded as monochrome pictures, where slices of colour planes can be signalled in an interleaved manner.

separate_colour_plane_flag equal to 1 specifies that the three colour components of the 4:4:4 chroma format are coded separately. separate_colour_plane_flag equal to 0 specifies that the colour components are not coded separately. When separate_colour_plane_flag is not present, it is inferred to be equal to 0. When separate_colour_plane_flag is equal to 1, the coded picture consists of three separate components, each of which consists of coded samples of one colour plane (Y, Cb, or Cr) and uses the monochrome coding syntax. In this case, each colour plane is associated with a specific colour_plane_id value.

NOTE 1 – There is no dependency in decoding processes between the colour planes having different colour_plane_id values. For example, the decoding process of a monochrome picture with one value of colour_plane_id does not use any data from monochrome pictures having different values of colour_plane_id for inter prediction.

In the current design, one picture header is signalled per picture, so it is not possible to have 3 picture headers with different colour_plane_id.

Secondly, there can be multiples slices belonging to diffident colour planes but according to the current constraint slice_address should be unique within a picture:

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpics_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

# Proposal

The proposal consists of two aspects.

Aspect 1

It is proposed to move colour_plane_id to slice header.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) |  |
| num_tiles_in_slice_minus1 | ue(v) |
| slice_type | ue(v) |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |

It is also proposed to allow having the same slice_address for different colour planes of the same picture, so the constraints are modified to exclude slices of the different colour planes:

It is a requirement of bitstream conformance that the following constraints apply for all slices of the same colour plane of a picture when separate_colour_plane_flag is equal to 1 or for all slices of a picture when separate_colour_plane_flag is equal to 0:

- If rect_slice_flag is equal to 0 or subpics_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

Aspect 2

It is proposed to modify the signaling condition of SPS level enabling flags for joint CbCr, adaptive color transform, chroma BDPCM, and palette.

For joint CbCr, the SPS control flag is proposed to only be signaled when ChromaArrayType! = 0, as it should be inherently disabled for monochrome or when separate color plane is enabled.

For adaptive color transform, and chroma BDPCM, the signaling is enabled when ChromaArrayType = =3 instead of chroma_format_idc = = 3, as chroma_format_idc = = 3 includes separate color plane case where these tools should be disabled. Note, although palette can be enabled for separate colour planes and monochrome, the proposal only considers the case with ChromaArrayType = =3 i.e., 4:4:4 content coding with separate color plane disabled.

| sps_joint_cbcr_enabled_flag | u(1) |
| --- | --- |
| if( ChromaArrayType != 0 ) { |  |
| sps_joint_cbcr_enabled_flag | u(1) |
| same_qp_table_for_chroma | u(1) |
| numQpTables = same_qp_table_for_chroma ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |
| for( i = 0; i < numQpTables; i++ ) { |  |
| qp_table_start_minus26[ i ] | se(v) |
| num_points_in_qp_table_minus1[ i ] | ue(v) |
| for( j = 0; j <= num_points_in_qp_table_minus1[ i ]; j++ ) { |  |
| delta_qp_in_val_minus1[ i ][ j ] | ue(v) |
| delta_qp_diff_val[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

| if( ChromaArrayType = = 3 chroma_format_idc = = 3 ) { |  |
| --- | --- |
| sps_palette_enabled_flag | u(1) |
| sps_act_enabled_flag | u(1) |
| } |  |

| if( sps_bdpcm_enabled_flag && ChromaArrayType = = 3 chroma_format_idc = = 3 ) |  |
| --- | --- |
| sps_bdpcm_chroma_enabled_flag | u(1) |

sps_joint_cbcr_enabled_flag equal to 0 specifies that the joint coding of chroma residuals is disabled. sps_joint_cbcr_enabled_flag equal to 1 specifies that the joint coding of chroma residuals is enabled. When sps_joint_cbcr_enabled_flag is not present, the value of sps_joint_cbcr_enabled_flag is inferred to be equal to 0.

pps_joint_cbcr_qp_offset_present_flag equal to 1 specifies that pps_joint_cbcr_qp_offset_value and joint_cbcr_qp_offset_list[ i ] are present in the PPS RBSP syntax structure. pps_joint_cbcr_qp_offset_present_flag equal to 0 specifies that pps_joint_cbcr_qp_offset_value and joint_cbcr_qp_offset_list[ i ] are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0 or sps_joint_cbcr_enabled_flag is equal to 0, the value of pps_joint_cbcr_qp_offset_present_flag shall be equal to 0.

pps_joint_cbcr_qp_offset_value specifies the offset to the luma quantization parameter Qp′Y used for deriving Qp′CbCr. The value of pps_joint_cbcr_qp_offset_value shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0 or sps_joint_cbcr_enabled_flag is equal to 0, pps_joint_cbcr_qp_offset_value is not used in the decoding process and decoders shall ignore its value. When pps_joint_cbcr_qp_offset_present_flag is equal to 0, pps_joint_cbcr_qp_offset_value is not present and is inferred to be equal to 0.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).