| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0157 |
| --- | --- |

| Title: | AHG12: On motion compensation for sub-pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Yao-Jen Chang Muhammed Coban Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In the VVC draft 7, a picture can be partitioned in one or more sub-pictures. A sub-picture can be treated as a picture by setting a flag subpic_treated_as_pic_flag[ i ] equal to 1. In this case, a sub-picture can be decoded as a separate picture, i.e. being extracted.

Number of sub-pictures in a picture stays the same, while the sub-picture reordering can happen per picture. When sub-picture reordering happens, then it is asserted that the decoding results of the reordered sub-pictures is different when the sub-picture is decoded as a part of the whole picture or as a separate picture.

In this contribution, it is proposed to fix the decoding problem of the reordered sub-pictures by considering boundaries of a reference sub-picture in the motion compensation process and TMVP derivation. Additionally, it is proposed to replace a current picture size with a reference picture size in the clipping (motion padding) since the reference picture may have different size than the current picture when reference picture resampling is enabled.

# Problem Statement

In the VVC draft 7, a picture can be partitioned in one or more sub-pictures. The sub-picture partitioning layout is signaled in SPS, the picture sub-picture layout mapping to SPS layout can be signaled in PPS or picture header (PH), while slices have slice_subpic_id signaled in a slice header (SH) to identify which sub-picture they belong to.

A sub-picture can be treated as a picture by setting a flag subpic_treated_as_pic_flag[ i ] equal to 1. In this case, a sub-picture can be decoded as a separate picture, i.e. being extracted.

Number of sub-pictures in a picture stays the same, while the sub-picture reordering can happen per picture. When sub-picture reordering happens, then it is asserted that the decoding results of the reordered sub-pictures is different when the sub-picture is decoded as a part of the whole picture or as a separate picture.

An example of this case, where MV points to different reference blocks, is shown on the Figure 1 (sub-picture is decoded as a part of the entire picture) and on the Figure 2 (the only extracted sub-picture is decoded)

Figure 1. Reordering of sub-pictures.

Figure 2. Extracted sub-picture after sub-pictures reordering.

Additionally, a current picture size is used in the reference sample clipping (padding) process, for example, in the following equations.

The variable picW is set equal to pic_width_in_luma_samples and the variable picH is set equal to pic_height_in_luma_samples.

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(957) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(958)

The reference sample coordinates are clipped by the current picture width and height, while it should be the reference picture width and height, since it may have a different size when reference picture resampling is used.

# Proposal

It is proposed to derive reference sub-picture boundaries in addition to the current sub-picture boundaries and use the reference sub-picture boundaries for the clipping and the difference between the current and reference boundary positions is added in the motion compensation.

In a similar way, a collocated block position for TMVP derivation is modified by adding a difference between the left boundary positions of the reference and current sub-pictures to the horizontal collocated block position, and by adding a difference between the top boundary positions of the reference and current sub-pictures to the vertical collocated block position.

It is also proposed to replace the current picture width and height with the reference picture width and height in the reference sample clipping.

An example of such changes is provided below, and the entire specification modification is attached to this contribution.

8.5.6.3.2 	Luma sample interpolation filtering process

...

The variable picW is set equal to pic_width_in_luma_samples of the referenfce picture refPicLX and the variable picH is set equal to pic_height_in_luma_samples of the reference picture refPicLX.

The luma interpolation filter coefficients fbL[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are specified in Table 28.

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

RefSubPicIdx is derived to be such that SubpicIdList[ RefSubPicIdx ] of the reference picture is equal to slice_subpic_id.

The variables RefSubPicLeftBoundaryPos, RefSubPicTopBoundaryPos, RefSubPicRightBoundaryPos, and RefSubPicBotBoundaryPos are derived as follows:

RefSubPicLeftBoundaryPos = subpic_ctu_top_left_x[ RefSubPicIdx ] * CtbSizeY
	RefSubPicRightBoundaryPos = Min( picW − 1,
		( subpic_ctu_top_left_x[ RefSubPicIdx ] + subpic_width_minus1[ RefSubPicIdx ] + 1 ) * CtbSizeY − 1)
	RefSubPicTopBoundaryPos = subpic_ctu_top_left_y[ RefSubPicIdx ] *CtbSizeY
	RefSubPicBotBoundaryPos = Min( picH − 1,		( subpic_ctu_top_left_y[ RefSubPicIdx ] + subpic_height_minus1[ RefSubPicIdx ] + 1 ) * CtbSizeY − 1)

xInti = Clip3( RefSubPicLeftBoundaryPos, RefSubPicRightBoundaryPos, xIntL + i )	(642)

yInti = Clip3( RefSubPicTopBoundaryPos, RefSubPicBotBoundaryPos, yIntL + i )	(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(645)

…

8.5.6.3	Fractional sample interpolation process

8.5.6.3.1	General

…

The variables leftBoundaryOffset is set equal to scaling_win_left_offset, topBoundaryOffset is set equal to scaling_win_top_offset, refSubPicLeftBoundaryPos, and refSubPicTopBoundaryPos are set equal to 0.

If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

leftBoundaryOffset += SubPicLeftBoundaryPos

topBoundaryOffset += SubPicTopBoundaryPos

RefSubPicIdx is derived to be such that SubpicIdList[ RefSubPicIdx ] of the reference picture is equal to slice_subpic_id.

refSubPicLeftBoundaryPos = subpic_ctu_top_left_x[ RefSubPicIdx ] * CtbSizeY

refSubPicTopBoundaryPos = subpic_ctu_top_left_y[ RefSubPicIdx ] *CtbSizeY

The variable fRefLeftOffset is set equal to ( ( scaling_win_left_offset + refSubPicLeftBoundaryPos )  <<  10 ), where scaling_win_left_offset is the scaling_win_left_offset for the reference picture.

The variable fRefTopOffset is set equal to ( ( scaling_win_top_offset + refSubPicTopBoundaryPos )  <<  10 ), where scaling_win_top_offset is the scaling_win_top_offset for the reference picture.

…

- Let ( refxSbL, refySbL ) and ( refxL, refyL ) be luma locations pointed to by a motion vector ( refMvLX[0], refMvLX[1] ) given in 1/16-sample units. The variables refxSbL, refxL, refySbL, and refyL are derived as follows:

refxSbL = ( ( ( xSb – leftBoundaryOffset scaling_win_left_offset )  <<  4 ) + refMvLX[ 0 ] ) * scalingRatio[ 0 ]	(937)

refxL = ( ( Sign( refxSbL ) * ( ( Abs( refxSbL ) + 128 )  >>  8 )
		+ xL * ( ( scalingRatio[ 0 ] + 8 )  >>  4 ) ) + fRefLeftOffset + 32 )  >>  6	(938)

refySbL = ( ( ( ySb – topBoundaryOffset scaling_win_top_offset ) <<  4 ) + refMvLX[ 1 ] ) * scalingRatio[ 1 ]	(939)

refyL = ( ( Sign( refySbL ) * ( ( Abs( refySbL ) + 128 )  >>  8 ) + yL *
		( ( scalingRatio[ 1 ] + 8 )  >>  4 ) ) + fRefTopOffset + 32 )  >>  6	(940)

- The variables xIntL, yIntL, xFracL and yFracL are derived as follows:

xIntL = refxL  >>  4		(941)

yIntL = refyL  >>  4		(942)

xFracL = refxL & 15		(943)

yFracL = refyL & 15		(944)

…

- Let (refxSbC, refySbC) and ( refxC, refyC ) be chroma locations pointed to by a motion vector (refMvLX[ 0 ], refMvLX[ 1 ]) given in 1/32-sample units. The variables refxSbC, refySbC, refxC and refyC are derived as follows:

addX = sps_chroma_horizontal_collocated_flag ? 0 : 8 * ( scalingRatio[ 0 ] − ( 1  <<  14 ) )	(945)

addY = sps_chroma_vertical_collocated_flag ? 0 : 8 * ( scalingRatio[ 1 ] − ( 1  <<  14 ) ) 	(946)

refxSbC = ( ( ( xSb – leftBoundaryOffset scaling_win_left_offset  ) / SubWidthC  <<  5 ) + refMvLX[ 0 ] ) *
		scalingRatio[ 0 ] + addX	(947)

refxC = ( ( Sign( refxSbC ) * ( ( Abs( refxSbC ) + 256 )  >>  9 )
		+ xC * ( ( scalingRatio[ 0 ] + 8 )  >>  4 ) )   + fRefLeftOffset / SubWidthC + 16 )  >>  5	(948)

refySbC =  ( ( ( ySb – topBoundaryOffset scaling_win_top_offset  ) / SubHeightC  <<  5 ) + refMvLX[ 1 ] ) *
		scalingRatio[ 1 ] + addY	(949)

refyC = ( ( Sign( refySbC ) * ( ( Abs( refySbC ) + 256 )  >>  9 )
		+ yC* ( ( scalingRatio[ 1 ] + 8 )  >>  4 ) ) + fRefTopOffset / SubHeightC + 16 )  >>  5	(950)

- The variables xIntC, yIntC, xFracC and yFracC are derived as follows:

xIntC = refxC  >>  5		(951)

yIntC = refyC  >>  5		(952)

xFracC = refxC & 31		(953)

yFracC = refyC & 31		(954)

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).