| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0160-v1 |
| --- | --- |

| Title: | CE4-related: CE4-1 spec text with suggested fixes |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Minhua Zhou | Email: | minhua.zhou@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

The GEO spec text provided for the CE4 common code base (JVET-O0079) and CE4-1 (JVET-O0059) is broken in several places. To avoid the erroneous text populated into other CE or non-CE GEO-related contributions, it is recommended to consider this version as the base for spec text preparation.

Suggested normative changes to match the code:

- In 7.3.9.7 Merge data syntax, fixed condition for the regular_merge_flag[ x0 ][ y0 ] signalling when the GEO is present.

- Under semantics of ciip_flag[ x0 ][ y0 ], fixed the condition for MergeGeoFlag[ x0 ][ y0 ]. Unlike the TPM, the GEO only applies to CUs with both width and height larger than or equal to 8.

- In Table 36, geo_partition_idx is undefined, replaced it with merge_geo_partition_idx.

- Added needed parentheses to the equations for offsetX and offsetY derivation (discussed on the CE email reflector already, w/o those parentheses the equations do not make sense).

Suggested editorial changes:

- In semantics of sps_geo_enabled_flag, replace “triangular” with “wedge”. The GEO does not only produce triangular shapes.

- In semantics of pic_max_num_merge_cand_minus_max_num_geo_cand and CIIP_flag[x0][y0], replace “triangular” with “GEO”.  This has been overseen by proponents.

- In semantics of merge_geo_idx0[ x0 ][ y0 ] and merge_geo_idx1[ x0 ][ y0 ], replace “non-rectangular” with “wedge”. I believe some of GEO partitions are still rectangular.

- Deleted the definition of rho and motionOffset to make the spec text more compact (discussed on the CE email reflector already).

In general, it might be better to replace “GEO” with “wedge” across the board.

Please refer to the attached document for detail.