| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0162-v3 |
| --- | --- |

| Title: | A memory issue on adaptive color transform |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kenji Kondo, Masaru Ikeda, Teruhiko Suzuki Sony City Osaki. 2-10-1 Osaki Shinagawa-ku, Tokyo, 141-8610, Japan | Tel: Email: | +81 50 3750 2782 kenji.kondo@sony.com Masaru.Ikeda@sony.com Teruhiko.S@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to restrict the large transform such 64-points when an adaptive color transform (ACT) is used. Also, it is introduced that ACT needs large buffering because the inverse-ACT needs three color components after inverse transform. To reduce maximum buffer size, it is proposed that sps_act_enabled_flag is implicitly set equal to zero when sps_max_luma_transform_size_64_flag is true. The test is carried out and report the impact for coding efficiency. Based on the same test as JVET-P0517, simulation results reportedly show that when the separate-tree partition is off, the average {G, B, R} BD-rate losses {0.3%, 0.1%, 0.2%}, {0.4%, 0.1%, 0.3%} and { 0.5%, 0.2%, 0.3%} for AI, RA and LD configurations, respectively.

Introduction

In the last meeting, ACT was proposed in [1], and it was adopted. As introduced in [1], HEVC already supported ACT tool in the screen content coding (SCC) extension. The contribution proposed to include the ACT tool into VVC.

The block diagram when inverse-ACT at a point of decoder is applied, is shown in Figure 1. The inverse-ACT is located after the inverse transform.

Figure 1: Block diagram with ACT

The memory is required to buffer the values between inverse transform and inverse-ACT because the inverse-ACT needs the all three components. In the HEVC standard, the maximum size of inverse transform is 32-points. On the other hand, VVC supports 64-points transform. The inverse transform can’t be processed only for one value because it processes a bunch of vectors. Thus, the inverse ACT must wait for all components of transform size that is 64x64x3 in the worst case.

Proposals

As introduced in above, ACT tool may increase the memory size although it depends on hardware architecture. To reduce the maximum memory size, this contribution proposes to restrict the transform size when ACT is used.

In this proposal, a signal sps_max_luma_transform_size_64_flag is used to restrict the size. The sps_max_luma_transform_size_64_flag equal to 0 specifies that the maximum transform size in luma samples is equal to 32. If the sps_max_luma_transform_size_64_flag is set equal to 0, the memory size can be restricted as same size as HEVC even if ACT is used.

The syntax change is described in the following subclause. The sps_act_enabled_flag is inferred to be equal to 0 if the sps_max_luma_transform_size_64_flag is equal to 1.

Syntax change

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_max_luma_transform_size_64_flag | u(1) |
| … |  |
| if( chroma_format_idc = = 3 ) { |  |
| if(!sps_max_luma_transform_size_64_flag) |  |
| sps_act_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

sps_act_enabled_flag equal to 1 specifies that adaptive colour transform may be used and the cu_act_enabled_flag may be present in the coding unit syntax. sps_act_enabled_flag equal to 0 speifies that adaptive colour transform is not used and cu_act_enabled_flag is not present in the coding unit syntax. When sps_act_enabled_flag is not present, it is inferred to be equal to 0.

Experiments

The introduced techniques had been implemented based on a reference software VTM-7.1 [2]. For experimental simulations, the conditions and the test sequences were used as same as JVET-P0517.

In the simulation, IBC:1, PLT:1, BDPCM:2 and inter HashME are enabled for both anchor and test. Also, results with separate-tree both enabled and disabled are provided.

Results in a case of separate tree OFF

In a case of separate tree off, the results of BD-rate and runtime are shown in Table 1.

Table 1: Test results for separate tree OFF

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | -0.1% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.1% | 0.3% | 0.2% |
| RGB, Animation, 720p | 0.2% | 0.1% | 0.1% |
| RGB, camera captured, 1080p | 1.1% | 0.0% | 0.7% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.0% | 0.1% | 0.1% |
| YUV, Animation, 720p | 0.0% | 0.0% | 0.0% |
| YUV, camera captured, 1080p | 0.8% | -0.1% | 0.7% |
| Enc Time[%] | 99% |  |  |
| Dec Time[%] | 101% |  |  |
|  | Random Access |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | 0.0% | -0.1% |
| RGB, mixed content, 1440p & 1080p | 0.3% | 0.4% | 0.3% |
| RGB, Animation, 720p | 0.0% | 0.0% | 0.0% |
| RGB, camera captured, 1080p | 1.3% | 0.1% | 1.0% |
| YUV, text & graphics with motion, 1080p & 720p | 0.1% | 0.1% | 0.1% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.5% | 0.5% |
| YUV, Animation, 720p | 0.2% | 0.1% | -0.1% |
| YUV, camera captured, 1080p | 0.8% | -0.1% | 0.6% |
| Enc Time[%] | 104% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.0% | 0.0% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.8% | 0.6% | 0.6% |
| RGB, Animation, 720p | -0.3% | 0.1% | 0.2% |
| RGB, camera captured, 1080p | 1.7% | 0.0% | 0.6% |
| YUV, text & graphics with motion, 1080p & 720p | 0.4% | 0.4% | 0.5% |
| YUV, mixed content, 1440p & 1080p | 1.0% | 0.9% | 1.1% |
| YUV, Animation, 720p | 0.5% | 0.2% | 0.3% |
| YUV, camera captured, 1080p | 0.8% | 0.4% | 0.7% |
| Enc Time[%] | 106% |  |  |
| Dec Time[%] | 100% |  |  |

Results in a case of separate tree ON

In a case of separate tree ON, the results of BD-rate and runtime are shown in Table 2.

Table 2: Test results for separate tree ON

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.1% | 0.1% | 0.0% |
| RGB, mixed content, 1440p & 1080p | 0.4% | 0.4% | 0.3% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.0% |
| RGB, camera captured, 1080p | 1.8% | 0.0% | 1.4% |
| YUV, text & graphics with motion, 1080p & 720p | 0.2% | 0.3% | 0.4% |
| YUV, mixed content, 1440p & 1080p | 0.4% | 0.7% | 0.5% |
| YUV, Animation, 720p | 0.3% | 0.3% | 0.4% |
| YUV, camera captured, 1080p | 0.3% | 0.0% | 0.3% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 99% |  |  |
|  | Low delay B |  |  |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.2% | -0.1% | -0.2% |
| RGB, mixed content, 1440p & 1080p | 0.7% | 0.5% | 0.2% |
| RGB, Animation, 720p | -0.1% | 0.1% | 0.3% |
| RGB, camera captured, 1080p | 1.6% | 0.0% | 0.6% |
| YUV, text & graphics with motion, 1080p & 720p | 0.3% | 0.5% | 0.3% |
| YUV, mixed content, 1440p & 1080p | 0.9% | 0.9% | 1.1% |
| YUV, Animation, 720p | 0.3% | 0.2% | 0.2% |
| YUV, camera captured, 1080p | 0.7% | 0.4% | 0.8% |
| Enc Time[%] | 105% |  |  |
| Dec Time[%] | 100% |  |  |

Conclusion

This contribution mentioned an issue that ACT supporting in current design may increase the memory size in worst case. To reduce the memory size, a restriction was proposed that ACT can be used when sps_max_luma_transform_size_64_flag is equal to 0. The test reportedly showed the impact for coding efficiency when sps_max_luma_transform_size_64_flag is set to 0. The simulation results showed that when the separate-tree partition is off, the average {G, B, R} BD-rate losses {0.3%, 0.1%, 0.2%}, {0.4%, 0.1%, 0.3%} and {0.5%, 0.2%, 0.3%} for AI, RA and LD configurations, respectively.

References

- Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Hong-Jheng Jhu, Xianglin Wang, “Support of adaptive color transform for 444 video coding in VVC”, Document of JVET-P0517, Geneva, Switzerland, October 2019

- VTM-7.1 software, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.1

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).