| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0165-v1 |
| --- | --- |

| Title: | CE5-related: On the CC-ALF filtering process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Z. Zhang, J. Ström, K. Andersson | Tel: Email: | +46761499428 {zhi.a.zhang, jacob.strom, kenneth.r.andersson} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

A common feature to all tests in CE5 is that CC-ALF is not DC neutral which means that it does not guarantee that chroma is not changed if luma is flat. The main aspect with this contribution is to fix that problem. A common feature of CE5-1.1 and CE5-2.2 is to remove the multiplications of CC-ALF filtering process. This contribution presents results of a DC neutral design on top of CE5 anchor, CE5-2.1 and CE5-2.2.

Compared to the CE5-CC-ALF anchor, the modified version of CE5-2.1 has a Y/U/V BD-rate impact of
 -0.02%/-0.02%/-0.19% (AI), 0.00%/-0.28%/-0.31% (RA) and 0.03%/-0.51%/-0.63% (LDB).

Compared to CE5-2.1, the modified version of CE5-2.1 has a Y/U/V BD-rate impact of 
0.00%/-0.55%/-0.54% (AI), -0.02%/-0.36%/-0.37% (RA) and 0.03%/-0.27%/-0.50% (LDB).

Compared to the CE5-CC-ALF anchor, the modified version of CE5-2.2 has a Y/U/V BD-rate impact of
-0.03%	/0.15%/-0.06% (AI), -0.01%/-0.13%/-0.13% (RA) and 0.00%/-0.55%/-0.63% (LDB).

Compared to CE5-2.2, the modified version of CE5-2.2 has a Y/U/V BD-rate impact of
0.00%	/-0.58%/-0.65% (AI), -0.01%/-0.47%/-0.57% (RA) and -0.01%/-0.47%/-0.53% (LDB).

# Introduction

In the 16th JVET meeting, CE5 was defined to evaluate different design aspects of the cross component adaptive loop filter CC-ALF. The different design aspects would be evaluated relative to a “CE anchor”. The CE5-2 sub-tests of CE5 targeted complexity reduction of the filtering process. In CE5-2.1, it was proposed to restrict the CC-ALF coefficients to have values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}. In CE5-2.2, it was proposed to restrict the CC-ALF coefficients to have values: {-8, -4, -2, -1, 0, 1, 2, 4, 8}. In both two tests in CE5-2, the multiplication was removed from the CC-ALF filtering process.

The CC-ALF method derives a residual correction ΔIx,y by applying a 3 × 4 diamond-shape filter with 8 unique coefficients to the co-located luma samples centered at the chroma sample to be refined. The filter coefficient values range between [-32, 31], inclusive. The residual correction ΔIx,y is calculated in the following way:

ΔIx,y=(CL0*RLxL, yL-1+
                      CL1*RLxL-1, yL+
                      CL2*RLxL, yL+
                      CL3*RLxL+1, yL+
                      CL4*RLxL-1, yL+1+
                      CL5*RLxL, yL+1+
                      CL6*RLxL+1, yL+1+
                      CL7*RLxL, yL+2+2shiftFactor-1)≫shiftFactor                      (Eqn 1)

where CLi specifies the CC-ALF filter coefficients, i ranges from 0 to 7.

- In the CE5 anchor, each coefficient but CL2 is estimated in the encoder side. The coefficient CL2 is calculated as CL2=(-1)*(CL0+CL1+CL3+CL4+CL5+CL6+CL7) and then clipped to the range [-32, 31], inclusive.

- In the CE5-2.1 and CE5-2.2, each coefficient is estimated in the encoder side.

The sum of 8 unique filter coefficients values may be not equal to 0 due to the clipping of the coefficient values. The situation when the coefficients do not sum to zero is clearly undesirable: This means that the filter is not DC-neutral, i.e., it modifies chroma even if luma is flat. However, from the statistic of the sum values of 8 unique filter coefficients in decoder side, it shows that very few signalled CC-ALF filter sets have a coefficient sum value i=07CLi that is not equal to 0:

- CE5 anchor (AI configuration) 16 out of 27636 coefficient sets has 0≠i=07CLi

- CE5-2.1 (AI configuration) 10 out of 28231 coefficient sets has 0≠i=07CLi

- CE5-2.2 (AI configuration) 108 out of 29207 coefficient sets has 0≠i=07CLi

This makes sense since the encoder coefficient optimization likely gives a strong penalty when the coefficients do not sum to zero.

We argue that it is better to modify the residual correction ΔIx,y derivation process to assure that filter is DC neutral. The equation 1 is then modified as follows:

From i=07CLi=0, we have CL2=-CL0-CL1-CL3-CL4-CL5-CL6-CL7. Inserting this value of CL2  into Equation 1, we get:

ΔIx,y=(CL0*(RLxL, yL-1-RLxL, yL)+
                      CL1*(RLxL-1, yL-RLxL, yL)+
                      CL3*(RLxL+1, yL-RLxL, yL)+
                      CL4*(RLxL-1, yL+1-RLxL, yL)+
                      CL5*(RLxL, yL+1-RLxL, yL)+
                      CL6*(RLxL+1, yL+1-RLxL, yL)+
                      CL7*(RLxL, yL+2-RLxL, yL)+2shiftFactor-1)≫shiftFactor               (Eqn 2)

In Equation 2, there are 7 unique coefficients CLi to be applied to the delta values ΔRLxi, yi=(RLxi, yi-RLxL, yL). The resulting filter is guaranteed to be DC-neutral.

# Proposed method

In this proposal, we propose to signal 7 unique CC-ALF coefficients to be applied to the delta values of the co-located luma samples centered at the chroma sample to be refined. The residual correction ΔIx,y is calculated as described in Equation 2.

The proposed method is applied to the CE5 anchor, CE5-2.1 and CE5-2.2 as test 1, test 2 and test 3.

The proposed method does not need to apply a coefficient to the luma sample RLxL, yL in the CC-ALF filtering process. However, the weight that is actually applied to the RLxL, yL can be calculated as sum of the signalled 7 unique coefficients: Csum=(-CL0-CL1-CL3-CL4-CL5-CL6-CL7). One advantage compared to the CE5 anchor or the CE5-2 tests is that this sum value Csum does not need to be restricted to be in the range of [-32, 31] or have a value of power-of-two. In other words, the proposed method allows the encoder to select a CC-ALF coefficient set such that Csum can be any value. It is speculated that the BD-rate improvement comes from the fact that the actual weight that is applied to RLxL, yL is allowed to be any value. In other words, the encoder does not have to discard filter candidates that are not DC-neutral but are good in every other respect. Figure 1 shows a distribution of Csum in test 1, test 2 and test 3 compared to a distribution of CL2 in CE5, CE5-2.1 and CE5-2.2 respectively.

Fig 1. The distribution of Csum compared to the distribution of CL2.

In CE5-2.1 which has 8 unique coefficients and each coefficient is restricted to have a value from {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, the total number of coefficients combinations is 158=2 562 890 625. However, the number of combinations that guarantee DC-neutrality is 16 089 375. In the modified version of CE5-2.1 which has 7 unique coefficients and each coefficient is restricted to have a value from {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64}, the total number of coefficients combinations is 157=170 859 375, where all combinations guarantee DC-neutrality. If we exclude the combinations that gives a sum value of all 7 coefficients greater than 128 or smaller than 128, the number of combinations that guarantee DC-neutral is 157 657 497. So even though we send fewer coefficients, we still have ten times more useful (DC-neutral) combinations available. Table 1 shows an analysis of the total number of coefficients combinations and the number of combinations that guarantee DC-neutral. The modified version of complexity reduction of CC-ALF filtering process has 9.8× and 3.4× combinations that guarantee DC-neutral compared to signalling 8 CC-ALF coefficients in CE5-2.1 and CE5-2.2 respectively.

|  | Nr. Combinations Total | Nr. Combinations DC-neutral | Rate |
| --- | --- | --- | --- |
| CE5-2.1 | 2 562 890 625 | 16 089 375 ( A ) | B ∕ A = 9.8 |
| Test 2 | 170 859 375 | 157 657 497 ( B ) |  |
| CE5-2.2 | 43 046 721 | 1 402 689 ( C ) | D ∕ C = 3.4 |
| Test 3 | 4 782 969 | 4 782 969 ( D ) |  |

Table 1. The analysis of number of coefficients combinations that guarantee DC-neutral

# Simulation results

## Test 1

In test 1, we implement the proposed method on top of the CE5-Anchor, where 7 CC-ALF coefficients are signaled in the bitstream. The dynamic range of the CC-ALF coefficients is [-32, 31], inclusive (same as for the CE5-Anchor). The CC-ALF coefficient coding is 6-bits FLC (same as for the CE5-Anchor).

Simulation results compared to the CE5-Anchor:

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | -0.17% | 102% | 100% |
| Class A2 | 0.00% | -0.51% | -0.36% | 102% | 102% |
| Class B | -0.01% | -0.02% | -0.04% | 102% | 100% |
| Class C | -0.01% | -0.04% | -0.07% | 94% | 101% |
| Class E | -0.01% | -0.01% | -0.19% | 96% | 98% |
| Overall | 0.00% | -0.11% | -0.15% | 99% | 100% |
| Class D | 0.00% | -0.05% | -0.09% | 100% | 99% |
| Class F | -0.01% | -0.02% | -0.02% | 101% | 99% |

|  | Random Access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.37% | -0.17% | 102% | 100% |
| Class A2 | 0.01% | -0.25% | -0.09% | 104% | 98% |
| Class B | 0.01% | -0.11% | -0.18% | 100% | 101% |
| Class C | -0.01% | -0.08% | -0.02% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | -0.18% | -0.11% | 101% | 100% |
| Class D | -0.02% | -0.12% | -0.02% | 101% | 99% |
| Class F | 0.00% | -0.02% | -0.04% | 108% | 100% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.55% | -0.21% | 101% | 99% |
| Class C | 0.03% | -0.07% | -0.41% | 102% | 99% |
| Class E | -0.16% | -0.03% | -0.49% | 99% | 102% |
| Overall | -0.04% | -0.26% | -0.35% | 101% | 100% |
| Class D | -0.05% | -0.32% | -0.54% | 97% | 101% |
| Class F | -0.02% | -0.01% | 0.72% | 101% | 100% |

## Test 2

In test 2, we implement the proposed method on top of CE5-2.1, where 7 CC-ALF coefficients are signaled in the bitstream. The CC-ALF coefficients are restricted to have the values: {-64, -32, -16, -8, -4, -2, -1, 0, 1, 2, 4, 8, 16, 32, 64} (same as in CE5-2.1). The CC-ALF coefficient coding is 3-bits FLC to signal the absolute values of the coefficients followed by a sign bit for non-zero coefficients (same as in CE5-2.1).

Simulation results compared to the CE5-Anchor and compared to CE5-2.1:

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  | Over CE5-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.07% | 101% | 100% | 0.00% | -0.47% | -0.51% | 100% | 100% |
| Class A2 | 0.00% | -0.46% | -0.26% | 100% | 101% | 0.00% | -0.67% | -1.06% | 100% | 100% |
| Class B | -0.02% | 0.10% | 0.03% | 100% | 100% | 0.00% | -0.79% | -0.58% | 99% | 100% |
| Class C | -0.05% | -0.10% | -0.18% | 94% | 100% | 0.00% | -0.18% | -0.16% | 99% | 100% |
| Class E | -0.03% | 0.27% | -0.62% | 97% | 98% | 0.01% | -0.58% | -0.51% | 102% | 99% |
| Overall | -0.02% | -0.02% | -0.19% | 99% | 100% | 0.00% | -0.55% | -0.54% | 100% | 100% |
| Class D | -0.01% | -0.23% | -0.56% | 94% | 99% | 0.00% | -0.13% | -0.12% | 96% | 101% |
| Class F | -0.04% | 0.04% | 0.04% | 101% | 101% | 0.00% | -0.27% | -0.30% | 100% | 100% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  | Over CE5-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.21% | -0.16% | 102% | 103% | 0.00% | -0.34% | -0.38% | 102% | 100% |
| Class A2 | 0.01% | -0.21% | -0.23% | 103% | 103% | 0.00% | -0.53% | -0.65% | 103% | 100% |
| Class B | -0.01% | -0.31% | -0.35% | 99% | 99% | -0.02% | -0.48% | -0.27% | 100% | 99% |
| Class C | -0.03% | -0.34% | -0.44% | 100% | 100% | -0.03% | -0.08% | -0.27% | 101% | 99% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.28% | -0.31% | 101% | 101% | -0.02% | -0.36% | -0.37% | 101% | 100% |
| Class D | -0.02% | -0.33% | -0.34% | 101% | 100% | 0.00% | 0.01% | -0.18% | 101% | 101% |
| Class F | -0.04% | -0.13% | -0.22% | 108% | 100% | -0.01% | -0.22% | -0.13% | 101% | 100% |

|  | Low delay B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  | Over CE5-2.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | 0.01% | -0.67% | -0.11% | 101% | 100% | 0.02% | -0.53% | -0.20% | 99% | 100% |
| Class C | 0.03% | -0.18% | -0.59% | 102% | 101% | 0.04% | -0.01% | -0.21% | 100% | 99% |
| Class E | 0.04% | -0.67% | -1.53% | 99% | 99% | 0.03% | -0.17% | -1.38% | 100% | 99% |
| Overall | 0.03% | -0.51% | -0.63% | 101% | 100% | 0.03% | -0.27% | -0.50% | 100% | 99% |
| Class D | -0.04% | -1.41% | -0.84% | 96% | 99% | -0.04% | -0.60% | -0.17% | 100% | 101% |
| Class F | 0.02% | -0.26% | 0.45% | 101% | 100% | -0.01% | -0.11% | 0.05% | 101% | 101% |

## Test 3

In test 3, we implement the proposed method on top of CE5-2.2, where 7 CC-ALF coefficients are signaled in the bitstream. The CC-ALF coefficients are restricted to have the values: {-8, -4, -2, -1, 0, 1, 2, 4, 8} (same as in CE5-2.2). The CC-ALF coefficient coding is truncated binary coding with a maximum value 5 to signal the absolute values of the coefficients followed by a sign bit for non-zero coefficients.

Simulation results compared to the CE5-Anchor and compared to CE5-2.2:

|  | All Intra |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  | Over CE5-2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.13% | 0.20% | 100% | 101% | 0.01% | -0.45% | -0.59% | 100% | 102% |
| Class A2 | -0.01% | 0.36% | 0.48% | 100% | 100% | 0.00% | -0.85% | -1.38% | 100% | 101% |
| Class B | -0.03% | 0.20% | 0.02% | 102% | 102% | 0.00% | -0.83% | -0.61% | 102% | 101% |
| Class C | -0.06% | -0.12% | -0.22% | 96% | 100% | -0.01% | -0.18% | -0.18% | 100% | 99% |
| Class E | -0.04% | 0.23% | -0.77% | 98% | 101% | 0.01% | -0.58% | -0.67% | 102% | 99% |
| Overall | -0.03% | 0.15% | -0.06% | 99% | 101% | 0.00% | -0.58% | -0.65% | 101% | 100% |
| Class D | 0.00% | -0.38% | -0.84% | 96% | 101% | 0.01% | -0.22% | -0.31% | 99% | 101% |
| Class F | -0.06% | 0.01% | 0.02% | 101% | 100% | 0.00% | -0.27% | -0.33% | 100% | 100% |

|  | Random Access |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  | Over CE5-2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.04% | 0.26% | 103% | 100% | 0.01% | -0.26% | -0.86% | 100% | 104% |
| Class A2 | 0.01% | 0.41% | 0.51% | 103% | 100% | -0.02% | -0.97% | -1.05% | 100% | 98% |
| Class B | -0.01% | -0.25% | -0.42% | 100% | 101% | -0.03% | -0.56% | -0.37% | 101% | 98% |
| Class C | -0.02% | -0.45% | -0.55% | 101% | 98% | 0.00% | -0.15% | -0.26% | 102% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.13% | -0.13% | 101% | 100% | -0.01% | -0.47% | -0.57% | 101% | 100% |
| Class D | -0.04% | -0.65% | -0.66% | 101% | 101% | -0.02% | -0.15% | -0.18% | 101% | 100% |
| Class F | -0.03% | -0.17% | -0.30% | 107% | 100% | 0.00% | -0.26% | -0.23% | 99% | 101% |

|  | Low delay B |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over CE5-Anchor |  |  |  |  | Over CE5-2.2 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.01% | -0.37% | -0.08% | 102% | 100% | -0.01% | -0.68% | -0.56% | 101% | 98% |
| Class C | 0.01% | -0.46% | -0.66% | 103% | 99% | -0.04% | -0.32% | -0.24% | 102% | 101% |
| Class E | 0.01% | -0.97% | -1.52% | 99% | 100% | 0.01% | -0.32% | -0.88% | 100% | 101% |
| Overall | 0.00% | -0.55% | -0.63% | 101% | 100% | -0.01% | -0.47% | -0.53% | 101% | 100% |
| Class D | -0.02% | -1.44% | -1.13% | 96% | 101% | -0.03% | -0.18% | -0.23% | 101% | 99% |
| Class F | 0.05% | -0.18% | 0.53% | 101% | 100% | 0.01% | -0.42% | 0.15% | 100% | 101% |

# Conclusion

This contribution proposes a DC-neutral CC-ALF filtering process. The proposed method modifies the residual correction ΔIx,y derivation by using 7 unique coefficients and the delta values ΔRLxi, yi=(RLxi, yi-RLxL, yL). The proposed method has a BD-rate improvement when applied to CE5 anchor, CE5-2.1 and CE5-2.2.

We propose to adopt the DC-neutral CC-ALF filtering process method to VVC and VTM.

# References

[1]. JVET-P2025: Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).