| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0167 |
| --- | --- |

| Title: | Non-CE5: Multiplication simplification for ALF and CC-ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jacob Ström, Zhi Zhang, Kenneth Andersson Torshamnsgatan 23 164 83 Stockholm Sweden | Tel: Email: | +46702672192 {jacob.strom, zhi.a.zhang, kenneth.r.andersson} @ericsson.com |
| Source: | Ericsson Research |  |  |

_____________________________

# Abstract

This document proposes a restriction of ALF coefficients. Instead of allowing all coefficient values from -127 to 127, only values that are pure powers of two +/-{0, 1, 2, 4, 8, …} and values that are 3 times pure powers of two +/-{3, 6, 12, 24, …} are proposed to be allowed. It is claimed that this gives a substantial complexity reduction in terms of surface area of hardware implementations of ALF since every per-sample multiplication can be then replaced by an addition and shifts. Reported BDR figures are:

Test 1:	Y-BDR: 	 0.02% (AI), 0.02% (RA), 0.00% (LDB)
		YUV-BDR: 0.01% (AI), 0.01% (RA) -0.07% (LDB)

A variant (test 2) which, in addition to allowing the values of test 1, also allows coefficient values that are 5 times a pure power of two +/-{5, 10, 20, 40, …} is also presented, and it is claimed that this can also be implemented in hardware with the same number of additions as test 1. Reported BDR figures are:

Test 2:	Y-BDR:	  0.00% (AI), 0.00% (RA), -0.04% (LDB) 
YUV-BDR: -0.01% (AI) -0.01% (RA) -0.08% (LDB).

Combinations with CC-ALF enabled are also presented for information.

# Introduction

Adaptive loop filtering (ALF) is a tool that gives a substantial BDR improvement. However, it is also quite expensive in the number of multiplications that are needed per sample. For every sample value, the following sum value must be calculated:

sum = f[ idx[ 0 ] ]   * (  Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy + y3 ] − curr ) + 
		Clip3( −c[ idx[ 0 ] ], c[ idx[ 0 ] ],     recPictureL[ hx, vy − y3 ] − curr ) ) + 
	f[ idx[ 1 ] ]   * (  Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx + 1, vy + y2 ] − curr ) + 
		Clip3( −c[ idx[ 1 ] ], c[ idx[ 1 ] ],     recPictureL[ hx − 1, vy − y2 ] − curr ) ) + 
	f[ idx[ 2 ] ]   * (  Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy + y2 ] − curr ) + 
		Clip3( −c[ idx[ 2 ] ], c[ idx[ 2 ] ],     recPictureL[ hx, vy − y2 ] − curr ) ) + 
	f[ idx[ 3 ] ]   * (  Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx − 1, vy + y2 ] − curr ) + 
		Clip3( −c[ idx[ 3 ] ], c[ idx[ 3 ] ],     recPictureL[ hx + 1, vy − y2 ] − curr ) ) + 
	f[ idx[ 4 ] ]   * (  Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx + 2, vy + y1 ] − curr ) + 
		Clip3( −c[ idx[ 4 ] ], c[ idx[ 4 ] ],     recPictureL[ hx − 2, vy − y1 ] − curr ) ) + 
	f[ idx[ 5 ] ]   * (  Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx + 1, vy + y1 ] − curr ) + 
		Clip3( −c[ idx[ 5 ] ], c[ idx[ 5 ] ],     recPictureL[ hx − 1, vy − y1 ] − curr ) ) + 
	f[ idx[ 6 ] ]   * (  Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy + y1 ] − curr ) + 
		Clip3( −c[ idx[ 6 ] ], c[ idx[ 6 ] ],     recPictureL[ hx, vy − y1 ] − curr ) ) + 	(1424)
	f[ idx[ 7 ] ]   * (  Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx − 1, vy + y1 ] − curr ) + 
		Clip3( −c[ idx[ 7 ] ], c[ idx[ 7 ] ],     recPictureL[ hx + 1, vy − y1 ] − curr ) ) + 
	f[ idx[ 8 ] ]   * (  Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx − 2, vy + y1 ] − curr ) + 
		Clip3( −c[ idx[ 8 ] ], c[ idx[ 8 ] ],     recPictureL[ hx + 2, vy − y1 ] − curr ) ) + 
	f[ idx[ 9 ] ]   * (  Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx + 3, vy ] − curr ) + 
		Clip3( −c[ idx[ 9 ] ], c[ idx[ 9 ] ],     recPictureL[ hx − 3, vy ] − curr ) ) + 
	f[ idx[ 10 ] ] * (  Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx + 2, vy ] − curr ) + 
		Clip3( −c[ idx[ 10 ] ], c[ idx[ 10 ] ], recPictureL[ hx − 2, vy ] − curr ) ) + 
	f[ idx[ 11 ] ] * (  Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx + 1, vy ] − curr ) + 
		Clip3( −c[ idx[ 11 ] ], c[ idx[ 11 ] ], recPictureL[ hx − 1, vy ] − curr ) )

Hence, a hardware implementation capable of producing one luma sample per clock must implement twelve parallel multiplications f*g of size 8 bits (left factor f= f[ idx[ 0 ] ] can be between -127 and 127) times 12 bits (right factor g, can be between -2046 and 2046). For software implementations, this may not be a problem, since fast multiplications are often included in most SIMD instruction sets. For hardware implementations however, such multiplications require quite a significant silicon surface area. Apart from the twelve multiplications for luma, ALF chroma filtering uses another six multiplications per channel. It is the purpose of this contribution to reduce the hardware complexity of these ALF multiplications, both for luma and chroma.

# Coefficient restriction

At the last JVET meeting in Geneva, Qualcomm proposed in JVET-P0557 to restrict the filter coefficients in CC-ALF to pure powers of two, i.e., one of {0, 1, 2, 4, 8, 16, 32, 64, 128}. A multiplication f*g between a filter coefficient f and a luma value g can then be written as

f*g =-1n*g≪s1                (Eqn 1)

where g≪s1 denotes a shift of g with s1 steps to the left, and n is the sign bit of the coefficient f. Shifting, even with a variable number of bits, is much less expensive to implement in hardware than a full multiplication. During the meeting, the question came up if this type of restriction could be applied to ALF instead of CC-ALF, and the proponent of JVET-P0557 answered that this had been tried but resulted in far too big a loss in BD rate.

## Test 1 – allowing 3 times power of two

In this contribution, we suggest a less restrictive constraint. In our first test, test 1, we allow an ALF coefficient to be either a pure power of two, or 3 times a pure power of two. This means that the following coefficients are allowed: f  {0, 1, 2, 3, 4, 6 8, 12, 16, 24, 32, 48, 64}. The last two coefficient values 96 and 128 are excluded since they are rarely used. Since 3*g = (2+1)*g = 2*g + g = (g<<1) + g, multiplication with 3 times a power of two can be very inexpensively implemented as

f*g = -1n*g≪1+g≪s1.        (Eqn 2)

In Section 5. 1 about hardware complexity we describe how Equation 1 and Equation 2 can be efficiently implemented in hardware. The fixed ALF coefficients in the table AlfFixFiltCoeff  are also changed to belong to the restricted set, so that both fixed coefficients and signaled coefficients are guaranteed to be in the restricted set.

## Test 2 – allowing also 5 times power of two

In test 2, in addition to the allowed coefficient values in test 1, we also allow ALF coefficients that are 5 times a pure power of two. This means that the following coefficients are allowed: f  {0, 1, 2, 3, 4, 5, 6 8, 10, 12, 16, 20, 24, 32, 40, 48, 64}. Since 5*g = (4+1)*g = (g<<2)+g, multiplication with 5 times a power of two can be very inexpensively implemented as

f*g = -1n*g≪2+g≪s1.         (Eqn 3)

In Section 5.2 about hardware complexity it is described how this can be very inexpensively implemented in hardware.

# Software implementations

A software decoder with access to fast multiplication may choose to multiply the same way as is currently done in VTM-7.0. Since the allowed set of coefficients is a subset of the currently included ones, no change to the decoder software needs to be done after the coefficients have been decoded. Hence software implementation is straight-forward.

# Specification changes

## Coefficient reading

Instead of reading the magnitude directly using exponential Golomb k coding, an index to the magnitude is read using the simpler truncated binary coding. If the index is not zero, a sign is also read. For test 1 we propose the following change to the specification:

| for( sfIdx = 0; sfIdx <= alf_luma_num_filters_signalled_minus1; sfIdx++ ) { |  |
| --- | --- |
| for( j = 0; j < 12; j++ ) { |  |
| alf_luma_coeff_abs_idx[ sfIdx ][ j ] | uek(v) tb(v) |
| if( alf_luma_coeff_abs_idx[ sfIdx ][ j ] ) |  |
| alf_luma_coeff_sign[ sfIdx ][ j ] | u(1) |
| } |  |
| } |  |

The magnitude of the coefficient is obtained from a table as described in the following proposed specification text:

The variable filtCoeff[ sfIdx ][ j ] with sfIdx = 0..alf_luma_num_filters_signalled_minus1, j = 0..11 is initialized as follows:

filtCoeff[ sfIdx ][ j ] = AlfApsCoeffMap[ alf_luma_coeff_abs_idx[ sfIdx ][ j ] ] * 	(90)
	( 1 − 2 * alf_luma_coeff_sign[ sfIdx ][ j ] )

AlfApsCoeffMap = { 0, 1, 2, 3, 4, 6, 8, 12, 16, 24, 32, 48, 64 }

We also replaced the exponential Golumb k coding in the HLS part of the specification with truncated binary coding. This is further described in the attached draft specification JVET-Q0167_test1_vs_JVET-Q0041-v2_DraftText.docx.

## Changing the fixed filter coefficients

The fixed filter coefficients also need to be changed, so that they comply with the restriction. For test 1 the values in AlfFixFiltCoeff are quantized so that they obey the restriction:

AlfFixFiltCoeff =	                                                                                                                                                (92)

{

{ 0,   0,   2,  −3,   1,  −4,   1,   7,  −1,   1,  −1,   5}
  { 0,   0,   2,  -3,   1,  -4,   1,   6,  -1,   1,  -1,   4}

{ 0,   0,   0,   0,   0,  -1,   0,   1,   0,   0,  -1,   2}

{ 0,   0,   0,   0,   0,   0,   0,   1,   0,   0,   0,   0}

{ 0,   0,   0,   0,   0,   0,   0,   0,   0,   0,  -1,   1} 
  { 2,   2,  −7,  −3,   0,  −5,  13,  22,  12,  −3,  −3,  17}

{ 2,   2,  -6,  -3,   0,  -4,  12,  24,  12,  -3,  -3,  16} 
  {−1,   0,   6,  −8,   1,  −5,   1,  23,   0,   2,  −5,  10}

{-1,   0,   6,  -8,   1,  -4,   1,  24,   0,   2,  -4,   8}

{ 0,   0,  -1,  -1,   0,  -1,   2,   1,   0,   0,  -1,   4} 
  { 0,   0,   3, −11,   1,   0,  −1,  35,   5,   2,  −9,   9}

{ 0,   0,   3, -12,   1,   0,  -1,  32,   4,   2,  -8,   8} 
  { 0,   0,   8,  −8,  −2,  −7,   4,   4,   2,   1,  −1,  25}

{ 0,   0,   8,  -8,  -2,  -6,   4,   4,   2,   1,  -1,  24}

. . .

},

Only the first 9 lines of the 64 are shown above, for further details see the draft specification.

# Hardware complexity

## Test 1

The task of the multiplier in test 1 is to calculate f*g where f is either ±2p, or ±3*2p or 0. Figure 1 shows a corresponding exemplary hardware design. The top box marked “conditional zero out” sets the output to zero if k0=0, and lets the input pass through if k0=1. This can be very efficiently implemented using AND gates. The bottom box works the same way, zeroing out the input if k1=1 and otherwise leaving it unchanged. If both k0 and k1 are set to 1, the output of the adder will be 3*g. This is then possibly negated and then shifted to produce a value of g*{±3, ±6, ±12, ±24, ±48}. If both k0 and k1 are set to zero, the output of the adder is zero which produces the output 0. Finally, if k1=1 but k0=0 the output of the adder will be g and this can then be shifted to produce g*{±1, ±2, ±4, ±8, ±16, ±32, ±64}.

Figure 1: Hardware needed to multiply g with 0, 2p or 3*2p.

The box marked “conditional negate” is dashed. The reason for this is that this box does not need to be fully implemented inside the multiplication. A negation can be carried out by inverting all bits and adding +1. However, as is shown in Equation (1424) above, the result of the multiplication will be added to another product in a subsequent step, and it is possible to use the carry-in of this addition to do the +1. What is left is the ability to invert the bits, which can be done using a number of inexpensive XOR gates.

Hence, the main cost left in Figure 1 is the adder and the shifter ≪s1. These together are much less expensive than a full multiplication.

The values k0,k1,  s1 and n are easy to calculate from the coefficient value (see Section 6), and this can be done during decoding of the coefficients, which happens at most once per CTU, and does not need to happen per-sample. When stored, these they require 1+1+3+1=6 bits in the case of test 1, which is less than a general coefficient between -127 and 127, which requires 8 bits. This can also help to reduce the cost of AlffixFiltCoeff. This currently holds values in the range [-21, 45] which requires 7 bits per value and thus approximately to 12*64*7/8=672 bytes to hold. With the solution in test 1 this becomes 12*64*6/8 = 576 bytes.

## Test 2

In addition to the coefficient values used in test 1, test 2 also allows multiplication with 5. This can be done with very similar hardware as for test 1, except that the first shift can shift either 1 or 2 steps instead of always shifting 1. This is shown in Figure 2.

Figure 2: Hardware capable of multiplying g with 0, 2p, 3*2p or 5*2p.

Here s0 can assume the value 1 or 2. When it is set to 1, the circuit is equivalent to that of Figure 1 and all the multiplications g* {0, 1, 2, 3, 4, 6 8, 12, 16, 24, 32, 48, 64} can be calculated. When s0 is set to 2, the output of the adder is g*5 and by negating and shifting the remaining multiplications g* {5, 10, 20, 40} can be calculated.

The figure in test 2 can be represented using the formula

f*g =-1n*k0 &b g≪s0+k1 &b g≪s1,        (Eqn 4)

where k0 &b g performs bitwise AND between k0 and every bit in g. The formula for test 1 and test3 is similar, but with s0 always equal to 1.

# Obtaining variables

During the decoding of the ALF coefficients, it is possible to easily calculate the values k0,k1,  s1 and n that are needed for the multiplication. Below we show you how this is done for test 1 and test 3. Assume we have read alf_luma_coeff_abs_idx[ sfIdx ][ j ] and alf_luma_coeff_sign[ sfIdx ][ j ] according to Section 4. We then use:

n = alf_luma_coeff_sign[ sfIdx ][ j ];
idx = alf_luma_coeff_abs_idx[ sfIdx ][ j ];
k0 = idx & 1;
k1 = (idx < 2 ? 0 : 1);
s1 = s_from_idx[idx>>1];

Where s_from_idx is a small table:

s_from_index[7] = {0, 0, 1, 2, 3, 4, 5};

# Combinations with CC-ALF

## Test 3 – applying test 1 to both ALF and CC-ALF

In test 3, the ALF part is equivalent to test 1, but it also enables CC-ALF. In order to guarantee that the DC-filter response is zero (i.e., that the coefficients sum to zero), we have used the DC-neutral formulation presented in JVET-Q0165, which guarantees no change in chroma if luma is flat. The same restriction as in test 1 is applied to the coefficients of both ALF and CC-ALF, i.e., the following are allowed: {0, 1, 2, 3, 4, 6, 8, 12, 16, 24, 32, 48, 64}. Thus, the same type of multiplication is used both for ALF and CC-ALF. The YUV-BDR figures compared to the CC-ALF anchor in CE5 are -0.04% (AI), -0.06% (RA) -0.16% (LDB).

## Test 4 – further restricting CC-ALF coefficients

In test 4, while the ALF part is equivalent to test 1, it places further restrictions on the CC-ALF coefficients, allowing only pure powers of two {0, 1, 2, 4, 8, 16, 32, 64}. This makes the CC-ALF part equivalent to test 2 of JVET-Q0165, which is a DC-neutral version of CE5-2.1. As it seems, CC-ALF coefficients are not as sensitive to this further restriction as regular ALF coefficients; the YUV BDR figures compared to the CC-ALF anchor in CE5 are -0.03% (AI), -0.05% (RA) and -0.15% (LDB) – not too different compared to test 3. Hence, if combined with CC-ALF, test 4 may be a better solution given that the further restriction of the coordinates simplifies the CC-ALF multiplication further.

## Test 5

In test 5, the ALF part is equivalent to test 2, allowing the coefficients to assume the values of {0, 1, 2, 3, 4, 6 8, 12, 16, 24, 32, 48, 64}. For the CC-ALF coefficients, the restriction is the same as for test 4, i.e., allowing only pure powers of two {0, 1, 2, 4, 8, 16, 32, 64}. This makes the CC-ALF part equivalent to test 2 of JVET-Q0165, which is a DC-neutral version of CE5-2.1.

# Results

In this section we provide the BDR results for test 1, test 2 test 3 test 4 and test 5.

## Results test 1

These are the results for test 1, where, in addition to pure power-of-two coefficients, we also allow 3 times a pure power of two. Encoder run times are measured on the same machine for AI and LD, but not on RA since the division into several parts gives good time estimates anyway. Decoder run times are always measured on the same machine.

All intra: (YUV BD-rate = 0.01%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.00% | 0.04% | 100% | 100% |
| Class A2 | 0.04% | 0.03% | 0.03% | 100% | 99% |
| Class B | 0.02% | -0.07% | -0.06% | 100% | 100% |
| Class C | -0.02% | -0.07% | -0.07% | 99% | 100% |
| Class E | 0.05% | -0.05% | -0.05% | 101% | 98% |
| Overall | 0.02% | -0.04% | -0.03% | 100% | 100% |
| Class D | -0.02% | 0.09% | 0.05% | 101% | 102% |
| Class F | -0.02% | -0.09% | -0.08% | 100% | 99% |

Random access: (YUV BD-rate = 0.01%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.16% | -0.03% | 101% | 101% |
| Class A2 | 0.05% | -0.01% | 0.01% | 101% | 99% |
| Class B | 0.04% | -0.06% | -0.06% | 100% | 100% |
| Class C | 0.00% | -0.20% | -0.06% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.11% | -0.04% | 100% | 100% |
| Class D | -0.03% | -0.30% | -0.06% | 98% | 99% |
| Class F | -0.06% | -0.19% | -0.10% | 98% | 100% |

Low delay B: (YUV BD-rate = -0.07%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.03% | -0.53% | -0.24% | 101% | 97% |
| Class C | -0.03% | -0.34% | -0.60% | 101% | 100% |
| Class E | -0.01% | 0.17% | -0.38% | 99% | 98% |
| Overall | 0.00% | -0.29% | -0.39% | 101% | 99% |
| Class D | -0.13% | 0.31% | 0.11% | 100% | 101% |
| Class F | -0.13% | -0.48% | -0.18% | 100% | 101% |

## Results test 2

These are the results for test 2, where, in addition to pure power-of-two coefficients, we also allow 3 times a pure power of two and 5 times a power of two. Encoder run times are measured on the same machine for AI and LD, but not on RA since the division into several parts gives good time estimates anyway. Decoder run times are always measured on the same machine.

All intra: (YUV-BDR: -0.01%):

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class A2 | 0.02% | 0.01% | 0.00% | 102% | 97% |
| Class B | 0.00% | -0.08% | -0.07% | 100% | 100% |
| Class C | -0.03% | -0.09% | -0.09% | 101% | 101% |
| Class E | 0.00% | -0.06% | -0.06% | 102% | 101% |
| Overall | 0.00% | -0.05% | -0.05% | 101% | 100% |
| Class D | -0.02% | 0.04% | 0.00% | 100% | 100% |
| Class F | -0.03% | -0.09% | -0.12% | 100% | 101% |

Random access: (YUV-BDR: -0.01%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.02% | 98% | 100% |
| Class A2 | 0.01% | 0.05% | 0.05% | 97% | 100% |
| Class B | 0.01% | -0.16% | -0.02% | 99% | 100% |
| Class C | -0.01% | -0.27% | -0.23% | 98% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.12% | -0.06% | 98% | 100% |
| Class D | -0.05% | -0.32% | -0.09% | 99% | 99% |
| Class F | -0.01% | -0.06% | -0.09% | 105% | 101% |

Low delay B: (YUV-BDR: -0.08%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.24% | -0.03% | 101% | 99% |
| Class C | 0.01% | -0.36% | -0.15% | 100% | 102% |
| Class E | -0.13% | 0.05% | -0.86% | 100% | 100% |
| Overall | -0.04% | -0.21% | -0.28% | 100% | 100% |
| Class D | 0.01% | -0.25% | -0.44% | 100% | 102% |
| Class F | -0.14% | -0.25% | -0.03% | 99% | 100% |

## Results test 3

These are the results for test 3, where we use the same coefficient restriction as for test 1, but apply it not only to ALF but also to CC-ALF. The anchor in these simulations is the CC-ALF anchor from CE-5. In these simulations the encoder run times are not measured on the same machine. The decoder run times are measured on the same machine.

All intra: (YUV-BDR: -0.04%)

|  | Over CE5 CC-ALF anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.13% | 101% | 99% |
| Class A2 | 0.04% | -0.55% | -0.40% | 98% | 101% |
| Class B | 0.01% | -0.04% | -0.12% | 101% | 100% |
| Class C | -0.06% | -0.17% | -0.25% | 95% | 102% |
| Class E | 0.03% | 0.06% | -0.72% | 98% | 97% |
| Overall | 0.00% | -0.13% | -0.29% | 99% | 100% |
| Class D | -0.02% | -0.13% | -0.44% | 96% | 101% |
| Class F | -0.06% | -0.12% | -0.16% | 100% | 100% |

Random access: (YUV-BDR: -0.06%)

|  | Over CE5 CC-ALF anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.25% | -0.31% | 99% | 99% |
| Class A2 | 0.08% | -0.27% | -0.33% | 99% | 99% |
| Class B | 0.04% | -0.37% | -0.48% | 98% | 100% |
| Class C | -0.05% | -0.42% | -0.44% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | -0.34% | -0.41% | 98% | 100% |
| Class D | -0.06% | -0.55% | -0.50% | 99% | 100% |
| Class F | -0.05% | -0.31% | -0.34% | 106% | 98% |

Low delay B: (YUV-BDR: -0.16%)

|  | Over CE5 CC-ALF anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.96% | -0.49% | 100% | 100% |
| Class C | 0.00% | -0.48% | -0.79% | 103% | 98% |
| Class E | 0.03% | -0.81% | -1.62% | 100% | 101% |
| Overall | 0.01% | -0.76% | -0.87% | 101% | 100% |
| Class D | -0.05% | -1.36% | -0.69% | 97% | 100% |
| Class F | -0.07% | -0.50% | 0.17% | 103% | 100% |

## Results test 4

These are the results for test 4, where we use the same coefficient restriction as for test 1 for the ALF coefficients. The CC-ALF coefficients are further restricted to only pure powers of two. The anchor in these simulations is the CC-ALF anchor from CE-5. In these simulations the encoder run times are not measured on the same machine. The decoder run times are measured on the same machine.

All intra: (YUV-BDR: -0.03%)

|  | Over CE5 CC-ALF anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | -0.05% | 99% | 100% |
| Class A2 | 0.04% | -0.42% | -0.29% | 97% | 100% |
| Class B | 0.00% | 0.06% | -0.01% | 98% | 101% |
| Class C | -0.07% | -0.15% | -0.22% | 91% | 102% |
| Class E | 0.01% | 0.25% | -0.67% | 91% | 101% |
| Overall | -0.01% | -0.04% | -0.22% | 95% | 101% |
| Class D | -0.02% | -0.15% | -0.48% | 93% | 101% |
| Class F | -0.07% | -0.02% | -0.04% | 99% | 100% |

Random access: (YUV-BDR: -0.05%)

|  | Over CE5 CC-ALF anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.26% | -0.27% | 97% | 99% |
| Class A2 | 0.08% | -0.20% | -0.24% | 99% | 102% |
| Class B | 0.05% | -0.36% | -0.36% | 97% | 98% |
| Class C | -0.04% | -0.45% | -0.47% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.03% | -0.33% | -0.35% | 98% | 100% |
| Class D | -0.06% | -0.54% | -0.42% | 99% | 101% |
| Class F | -0.03% | -0.22% | -0.33% | 108% | 101% |

Low delay B: (YUV-BDR: -0.15%)

|  | Over CE5 CC-ALF anchor |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | -0.82% | -0.26% | 101% | 100% |
| Class C | 0.00% | -0.43% | -0.74% | 101% | 99% |
| Class E | 0.04% | -1.38% | -1.47% | 98% | 99% |
| Overall | 0.01% | -0.83% | -0.72% | 100% | 100% |
| Class D | -0.05% | -1.01% | -0.54% | 98% | 101% |
| Class F | -0.10% | -0.35% | -0.37% | 101% | 101% |
| Class F | -0.07% | -0.50% | 0.17% | 103% | 100% |

## Results test 5

These are the results for test 5, where we use the same coefficient restriction as for test 2 for the ALF coefficients. The CC-ALF coefficients are further restricted to only pure powers of two. The anchor in these simulations is the CC-ALF anchor from CE-5. In these simulations the encoder run times are not measured on the same machine. The decoder run times are measured on the same machine.

All intra: (YUV-BDR: -0.05%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | 0.04% | -0.07% | 97% | 100% |
| Class A2 | 0.02% | -0.46% | -0.29% | 98% | 101% |
| Class B | -0.02% | 0.05% | -0.04% | 99% | 98% |
| Class C | -0.08% | -0.18% | -0.25% | 95% | 100% |
| Class E | -0.04% | 0.23% | -0.69% | 96% | 100% |
| Overall | -0.03% | -0.06% | -0.24% | 97% | 100% |
| Class D | -0.02% | -0.19% | -0.53% | 96% | 98% |
| Class F | -0.08% | -0.06% | -0.06% | 98% | 100% |

Random Access: (YUV-BDR: -0.08%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.24% | -0.17% | 100% | 99% |
| Class A2 | 0.01% | -0.23% | -0.31% | 100% | 101% |
| Class B | 0.00% | -0.35% | -0.52% | 98% | 99% |
| Class C | -0.03% | -0.50% | -0.46% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.34% | -0.39% | 99% | 100% |
| Class D | -0.05% | -0.55% | -0.31% | 99% | 100% |
| Class F | -0.04% | -0.19% | -0.32% | 107% | 97% |

Low delay B: (YUV-BDR: -0.19%)

|  | Over VTM-7.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.83% | -0.58% | 98% | 102% |
| Class C | 0.02% | -0.57% | -0.79% | 99% | 102% |
| Class E | -0.01% | -1.57% | -1.65% | 99% | 101% |
| Overall | -0.01% | -0.93% | -0.92% | 99% | 101% |
| Class D | -0.08% | -1.69% | -1.17% | 98% | 102% |
| Class F | -0.13% | -0.53% | -0.24% | 102% | 101% |

# Proposal

We propose to adopt the changes corresponding to test 2 in the current version of VTM-7.0. These changes are described in detail in the attached document JVET-Q0167_test2_vs_JVET-Q0041-v2_DraftText.docx.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).