| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0168-v1 |
| --- | --- |

| Title: | CE4-related: On GEO cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yoshitaka Morigami Masaru Ikeda Teruhiko Suzuki 2-10-1 Osaki, Shinagawa-ku Tokyo, 141-8610 JAPAN | Tel: Email: | +81-50-3750-2782 Yoshitaka.Morigami@sony.com Masaru.Ikeda@sony.com Teruhiko.s@sony.com |
| Source: | Sony Corporation |  |  |

_____________________________

- Abstract

In the CE4 common base (JVET-P0884/P0885) there seems to be 3 inconsistencies in terms of region assignment A & B, offset change in direction and magnitude in specific angle indexes. This contribution proposes to fix those inconsistencies for the design cleanups. With proposed modifications, overall memory requirement can be reduced to 43 bytes by additional calculations utilizing pattern symmetry, while there is no impact in BD-Rate, -0.02% and 0.00% for RA and LDB, respectively.

Introduction

The wedge partition is determined by both angleIdx and distanceIdx and has 82 modes in total which includes TPM mode as well. In JVET-P0884[1]/P0885[2], there seems to be 3 inconsistencies.

The first one is for region assignment. The Figure 1 shows region A (green area) and B (red area) assigned according to angleIdx. The assignment is the same between upper and lower row except for the angleIdx which is equal to 9 and 21.

The second one is for offset direction according to angleIdx. The offset has 2 directions and 3 level of magnitude according to angleIdx and distanceIdx, respectively. It seems that the direction is inconsistent in case that the angleIdx is equal to 0 and 12. For the angleIdx which is equal to 12, the offset is shifted toward region B (red area) according to the distanceIdx, while the offset is done toward region A (green area) for both angleIdx, 11 and 13, which are adjacent angleIdx. It is the same as for the angleIdx which is equal to 0.

The third one is for offset derivation. The Figure 2 shows offset derivation for case where the angleIdx is equal to 5 and 1, respectively. The offset change in magnitude is 8 for angleIdx=5, while it is 2 for angleIdx=1. So, the offset change is different between horizontal and vertical boundary with the same ratio in terms of width and height.

Figure 1: GEO region assignment according to angleIdx

Figure 2: GEO offset derivation in angleIdx=5 and 1

Proposed method

The following 3 modifications are proposed to be applied to the issues which was raised above, respectively.

- The derivation of the region (A & B) is modified as follows: (address 1st issue)

hwOffset is set to Clip3(-2,3,Log2(hwRatio))

If angleIdx >=9 + hwOffset && angleIdx <= 20 + hwOffset, PART1 and PART2 are set equal to A and B respectively, otherwise PART1 and PART2 are set equal to B and A respectively.

Then, the region A and B for angleIdx 9 are changed, and all region assignment becomes consistent.

- The deviation of offsetX and offsetY in both direction and magnitude is modified as follows: (address 2nd and 3rd issue)

hwRatio is set to nH / nW

hwOffset is set to Clip3(-2,3,Log2(hwRatio))

If the following conditions are true, variable shiftHor is set equal to 0:

3-hwOffset < angleIdx % 12 < 9 + hwOffset

Otherwise, shiftHor is set equal to 1.

If shiftHor is equal to 0, offsetX and offsetY are derived as follows:

offsetX = ( 256 − nW ) >> 1

offsetY = ( 256 − nH ) >> 1 + angleIdx < 12 ? (distanceIdx * nH) >> 3 : −((distanceIdx * nH) >> 3)

Otherwise, if shiftHor is equal to 1, offsetX and offsetY are derived as follows:

offsetX = ( 256 − nW ) >> 1 + angleIdx < 12 ? −(distanceIdx * nW) >> 3 : +((distanceIdx * nW) >> 3)

offsetY = ( 256 − nH ) >> 1

Then, the offset change in direction for angleIdx 0 and 12 is the same as for adjacent angleIdx (23 & 1 and 11 & 13, respectively). The offset change in magnitude becomes the same between horizontal and vertical boundary with the same ratio in terms of width and height (e.g. angleIdx 1 and 5).

Figure 3: Modified wedge partition mode

Experimental results

The proposed modifications were implemented on top of CE4 common base and tested over CE4 common base as an anchor[3]. Figure 4 shows the BD-Rate results for RA and LDB under common test conditions. There is no impact in BD-Rate performance with these modifications.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0-CE4-common |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.16% | -0.03% | 100% | 100% |
| Class A2 | 0.00% | 0.02% | -0.12% | 100% | 100% |
| Class B | -0.02% | -0.01% | -0.05% | 101% | 100% |
| Class C | -0.04% | 0.04% | -0.02% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | -0.02% | -0.05% | 100% | 99% |
| Class D | -0.01% | -0.14% | 0.04% | 102% | 100% |
| Class F | 0.00% | 0.05% | 0.06% | 102% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0-CE4-common |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.02% | 0.02% | 101% | 100% |
| Class C | -0.01% | -0.16% | 0.09% | 99% | 100% |
| Class E | 0.03% | -0.23% | 0.22% | 102% | 99% |
| Overall | 0.00% | -0.12% | 0.10% | 101% | 100% |
| Class D | -0.01% | -0.07% | 0.09% | 102% | 101% |
| Class F | -0.01% | 0.39% | -0.20% | 100% | 100% |

Figure 4: BD-Rate results vs. CE4 anchor

Motion storage requirements analysis

With these modifications, overall memory requirement can be reduced from 204 bytes to 43 bytes by additional calculations utilizing pattern symmetry.

Overall memory requirement for proposed modifications is 4 * 14 * 4 + 4 * 6 * 2 + 6 * 12 * 1 = 344bits (43bytes).

blendWeight[4][14] = {

{ 8, 8, 8, 7, 5, 3, 1, 0, 0, 0, 0, 0, 0, 0 },

{ 8, 7, 7, 6, 5, 5, 4, 4, 3, 3, 2, 1, 1, 0 },

{ 8, 8, 8, 7, 6, 5, 3, 2, 1, 0, 0, 0, 0, 0 },

{ 8, 8, 8, 8, 6, 4, 2, 0, 0, 0, 0, 0, 0, 0 }

};

maskValue[4][6] = {

{ 0, 1, 1, 1, 1, 1 },

{ 0, 2, 2, 2, 2, 1 },

{ 0, 2, 2, 1, 1, 1 },

{ 0, 2, 1, 1, 1, 1 }

}

wmFlip[6][12] = {

{ 0, 1, 1, 0, 0, 0, 1, 1, 1, 1, 1, 1 },

{ 0, 1, 1, 0, 0, 0, 1, 0, 1, 1, 1, 1 },

{ 0, 1, 1, 0, 0, 0, 1, 0, 0, 1, 1, 1 },

{ 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 1, 1 },

{ 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 1 },

{ 0, 1, 1, 0, 0, 0, 1, 0, 0, 0, 0, 0 },

}

Additional calculation utilizing pattern symmetry

sampleWeightAxy can be derived as follows:

- wmIdx=angleIdx%6>3 ?6- angleIdx%6 :(angleIdx%6)

- weightxy=blendWeight[wmIdx][Clip3(0, 13,wIdx+5)

- flipIdx=Clip3(0,5,Log2hwRatio+2)

- sampleWeightAxy=wmFlip[flipIdx][angleIdx%12==1 ?8-weightxy : weightxy

sType can be derived as follows:

- wmIdx=angleIdx%6>3 ?6- angleIdx%6 :(angleIdx%6)

- maskxSbIdxySbIdx=maskValue[wmIdx][Clip3(0, 5,mIdx+1)

- flipIdx=Clip3(0,5,Log2hwRatio+2)

- sTypexSbIdxySbIdx=maskxSbIdxySbIdx==2 | wmFlipflipIdxangleIdx%12==0 ?maskxSbIdxySbIdx : maskxSbIdxySbIdx==1 ?0 :1

| angleIdx%12 = 0 | wIdx=x+offsetX-128 |
| --- | --- |
| 1 ≤ angleIdx%12 ≤ 2 | wIdx=y+offsetY-(x+offsetX-tmp≪shift) |
| 3 ≤ angleIdx%12 ≤ 5 | wIdx=x+offsetX-(y+offsetY-tmp≪shift) |
| angleIdx%12 = 6 | wIdx=y+offsetY-128 |
| 7 ≤ angleIdx%12 ≤ 9 | wIdx=255-(x+offsetX)-(y+offsetY-tmp≪shift) |
| 10 ≤ angleIdx%12 ≤ 11 | wIdx=255-(x+offsetX-tmp≪shift)-y+offsetY |

| angleIdx%12 = 0 | mIdx=xSbIdx+offsetX-32 |
| --- | --- |
| 1 ≤ angleIdx%12 ≤ 2 | mIdx=ySbIdx+offsetY-(xSbIdx+offsetX-tmp≪shift) |
| 3 ≤ angleIdx%12 ≤ 5 | mIdx=xSbIdx+offsetX-(ySbIdx+offsetY-tmp≪shift) |
| angleIdx%12 = 6 | mIdx=ySbIdx+offsetY-32 |
| 7 ≤ angleIdx%12 ≤ 9 | mIdx=63-(xSbIdx+offsetX)-(ySbIdx+offsetY-tmp≪shift) |
| 10 ≤ angleIdx%12 ≤ 11 | mIdx=63-(xSbIdx+offsetX-tmp≪shift)-ySbIdx+offsetY |

Conclusion

This contribution proposed design cleanups to have more consistencies on wedge partition mode. With proposed modifications, overall memory requirement can be reduced to 43 bytes by additional calculations utilizing pattern symmetry, while there is no impact in BD-Rate. It is suggested that these modifications are adopted into the next version of VVC.

References

[1] H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang (Huawei), M. Bläser, J. Sauer (RWTH Aachen), “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0884, October 2019.

[2] K. Reuzé, C.-C Chen, H. Huang, W.-J Chien, V. Seregin, M. Karczewicz (Qualcomm), R.-L Liao, J. Chen, Y. Ye, J. Luo (Alibaba), M. Bläser, J. Sauer (RWTH Aachen), “Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)”, JVET-P0885, October 2019.

[3] C.-C. Chen, R.-L. Liao, X. Xiu, H. Yang “Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning”, JVET-P2024, October 2019

Patent rights declaration(s)

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).