| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0180-v1 |
| --- | --- |

| Title: | AHG9: On inter-layer referencing of scaling list and LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input Document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Olena Chubach, Ching-Yeh Chen, Lulin Chen, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, olena.chubach, chingyeh.chen, lulin.chen, peter.chuang, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to add support for inter-layer referencing between a direct reference layer and the current layer for two adaptation parameter set (APS) types: scaling list (SL) and luma mapping with chroma scaling (LMCS). For that, two syntax designs are proposed.

In the first design, it is proposed to signal an on-off control flag in the sequence parameter set (SPS) to indicate whether inter-layer referencing of SL/LMCS is allowed in a current coding layer video sequence (CLVS). If inter-layer referencing of SL/LMCS is allowed in the current CLVS, an index specifying the reference layer NAL unit ID is signalled. After that, available APSs with aps_params_type equal to SCALING_APS/LMCS_APS from the reference layer may be referenced by all coded pictures in the current CLVS.

In the second design, the syntax in the SPS is the same as that in the first design. In addition, an on-off control flag is signalled in the picture header (PH) in the current CLVS, indicating whether inter-layer referencing of SL/LMCS is enabled for the current picture. If inter-layer referencing of SL/LMCS is allowed in both the SPS and the current picture, available SL/LMCS APSs from the reference layer are referenced by the current picture. Otherwise, if inter-layer referencing of SL/LMCS is allowed in the SPS but disabled in the current picture, the SL/LMCS APSs from the reference layer cannot be referenced by the current picture, and only SL/LMCS APSs in the current CLVS can be referenced by the current picture.

# Introduction

In the current VVC design [1], SL and LMCS are signalled in APSs with aps_params_type equal to SCALING_APS and LMCS_APS, respectively. Meanwhile, according to the current VVC Draft 7, APSs within one layer cannot be referenced by another layer. Therefore, when two CLVSs use the same SL/LMCS APS, signalling becomes inefficient.

# Proposed methods

In VVC Draft 7, inter-layer referencing is supported for reference picture list (RPL). An inter-layer reference picture (ILRP) can be used as a long-term reference picture in the RPL. In this case, only pictures from the same access unit (AU) with the current picture, and with nuh_layer_id less than the nuh_layer_id of the current layer, are considered as valid reference pictures for RPL.

In this contribution, it is proposed to add support of inter-layer referencing for SL/LMCS APSs in VVC. That is, the SL/LMCS APSs can reference APSs of the same type from different layers. However, in order to simpify data access within different layers, similar constraints to those noted above are suggested for inter-layer referencing of SL/LMCS APSs. That is:

- Only APSs from a direct reference layer can be referenced.

- The referenced APSs shall be the APSs available in a picture unit (PU) which is in the same AU with the current picture, and with nuh_layer_id less than the nuh_layer_id of the current picture.

For that, two kinds of syntax designs are proposed. The first design allows controlling inter-layer referencing on SPS level, and the second design allows extending on/off control to a PH level.

## Signalling on/off control and reference layer for inter-layer referencing of SL/LMCS in the SPS

In this design, an on-off flag is signalled in the SPS, to indicate whether inter-layer referencing of SL/LMCS is allowed in the CLVS. If inter-layer referencing of SL/LMCS is allowed in the CLVS, an index, e.g. sps_scaling_list_inter_layer_idx/sps_lmcs_inter_layer_idx, is signalled. This index indicates which reference layer from the list of the direct reference layers is signalled. Therefore, the signalled index shall be constrained in the range of 0 to NumDirectRefLayers[GeneralLayer[nuh_layer_id]]. This way, if inter-layer referencing of SL/LMCS is enabled in the SPS, all available SL/LMCS APSs from the reference layer may be referenced by all coded pictures in the CLVS.

The related syntax modifications are highlighted in blue.

Table 1. Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_lmcs_enabled_flag | u(1) |
| if (sps_lmcs_enabled_flag) { |  |
| sps_lmcs_inter_layer_ref_enabled_flag | u(1) |
| if (sps_lmcs_inter_layer_ref_enabled_flag) |  |
| sps_lmcs_inter_layer_idx | ue(v) |
| } |  |
| … |  |
| sps_scaling_list_enabled_flag | u(1) |
| if (sps_scaling_list_enabled_flag) { |  |
| sps_scaling_list_inter_layer_ref_enabled_flag | u(1) |
| if (sps_scaling_list_inter_layer_ref_enabled_flag) |  |
| sps_scaling_list_inter_layer_idx | ue(v) |
| } |  |

sps_lmcs_inter_layer_ref_enabled_flag equal to 0 specifies that the lmcs data of any direct reference layer are not referenced by any coded slice in the CLVS. sps_lmcs_inter_layer_ref_enabled_flag equal to 1 specifies that the lmcs data of a direct reference layer may be referenced by coded slices in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_lmcs_inter_layer_ref_enabled_flag is inferred to be equal to 0. When sps_lmcs_inter_layer_ref_enabled_flag is not present, the value of sps_lmcs_inter_layer_ref_enabled_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_lmcs_inter_layer_ref_enabled_flag shall be equal to 0.

sps_lmcs_inter_layer_idx specifies the index of a layer in the list of direct reference layers that all APS NAL units with aps_params_type equal to LMCS_APS from this direct reference layer may be referenced by coded slices in the CLVS. The value of sps_lmcs_inter_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

pic_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. If sps_lmcs_inter_ref_enabled_flag is equal to 1, pic_lmcs_aps_id specifies the adaptation_parameter_set_id of the lmcs APS in a layer with index equal to sps_lmcs_inter_layer_idx of the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to pic_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

sps_scaling_list_inter_layer_ref_enabled_flag equal to 0 specifies that the scaling list data of any direct reference layer are not referenced by any coded slices in the CLVS. sps_scaling_list_inter_layer_ref_enabled_flag equal to 1 specifies that the scaling list data of a direct reference layer may be referenced by coded slices in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_scaling_list_inter_layer_ref_enabled_flag is inferred to be equal to 0. When sps_scaling_list_inter_layer_ref_enabled_flag is not present, the value of sps_scaling_list_inter_layer_ref_enabled_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_scaling_list_inter_layer_ref_enabled_flag shall be equal to 0.

sps_scaling_list_inter_layer_idx specifies the index of a layer in the list of direct reference layers that all APS NAL units with aps_params_type equal to SCALING_APS from this direct reference layer may be referenced by coded slices in the CLVS. The value of sps_scaling_list_inter_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1.

pic_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS. If sps_scaling_list_inter_ref_enabled_flag is equal to 1, pic_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS in a layer with index equal to sps_scaling_list_inter_layer_idx of the list of direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to pic_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

## Optional disabling of signalling the reference layer in the SPS, and the inter-layer referencing in the PH

In addition to flags which allow enabling inter-layer referencing and the reference layer indices signalled in the SPS, it is proposed to signal enabling flags in each PH, to indicate the on-off control of inter-layer referencing in the current picture. In this case, even though inter-layer referencing for SL/LMCS is enabled in the SPS, some pictures in a CLVS may reference APSs from a different layer, while others reference APSs from the current layer. This will depend on whether the flag indicating inter-layer referencing for SL/LMCS is enabled in the PH. Therefore, APS IDs, pic_scaling_list_aps_id and pic_lmcs_aps_id, which will be signalled after the flags indicating inter-layer referencing on/off in the PH,  ph_scaling_list_inter_layer_ref_enabled_flag and ph_lmcs_inter_layer_ref_enabled_flag, will indicate that APS IDs are from the reference layer only when inter-layer referencing is enabled.

The related syntax modifications are highlighted in blue.

Table 2. Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_lmcs_enabled_flag | u(1) |
| if (sps_lmcs_enabled_flag) { |  |
| sps_lmcs_inter_layer_ref_enabled_flag | u(1) |
| if (sps_lmcs_inter_layer_ref_enabled_flag) |  |
| sps_lmcs_inter_layer_idx | ue(v) |
| } |  |
| … |  |
| sps_scaling_list_enabled_flag | u(1) |
| if (sps_scaling_list_enabled_flag) { |  |
| sps_scaling_list_inter_layer_ref_enabled_flag | u(1) |
| if (sps_scaling_list_inter_layer_ref_enabled_flag) |  |
| sps_scaling_list_inter_layer_idx | ue(v) |
| } |  |

sps_lmcs_inter_layer_ref_enabled_flag equal to 0 specifies that the lmcs data of any direct reference layer are not referenced by any coded slice in the CLVS. sps_lmcs_inter_layer_ref_enabled_flag equal to 1 specifies that the lmcs data of a direct reference layer may be referenced by coded slices in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_lmcs_inter_layer_ref_enabled_flag is inferred to be equal to 0. When sps_lmcs_inter_layer_ref_enabled_flag is not present, the value of sps_lmcs_inter_layer_ref_enabled_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_lmcs_inter_layer_ref_enabled_flag shall be equal to 0.

sps_lmcs_inter_layer_idx specifies the index of a layer in the list of direct reference layers that all APS NAL units with aps_params_type equal to LMCS_APS from this direct reference layer may be referenced by coded slices in the CLVS. The value of sps_lmcs_inter_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

sps_scaling_list_inter_layer_ref_enabled_flag equal to 0 specifies that the scaling list data of any direct reference layer are not referenced by any coded slices in the CLVS. sps_scaling_list_inter_layer_ref_enabled_flag equal to 1 specifies that the scaling list data of a direct reference layer may be referenced by coded slices in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of sps_scaling_list_inter_layer_ref_enabled_flag is inferred to be equal to 0. When sps_scaling_list_inter_layer_ref_enabled_flag is not present, the value of sps_scaling_list_inter_layer_ref_enabled_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of sps_scaling_list_inter_layer_ref_enabled_flag shall be equal to 0.

sps_scaling_list_inter_layer_idx specifies the index of a layer in the list of direct reference layers that all APS NAL units with aps_params_type equal to SCALING_APS from this direct reference layer may be referenced by coded slices in the CLVS. The value of sps_scaling_list_inter_layer_idx shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1.

Table 3. Picture header RBSP syntax

| if( sps_lmcs_enabled_flag ) { |  |
| --- | --- |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| if (sps_lmcs_inter_layer_ref_enabled_flag) |  |
| ph_lmcs_inter_layer_ref_enabled_flag | u(1) |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| if (sps_scaling_list_inter_layer_ref_enabled_flag) |  |
| ph_scaling_list_inter_layer_ref_enabled_flag | u(1) |
| pic_scaling_list_aps_id | u(3) |
| } |  |

ph_lmcs_inter_layer_ref_enabled_flag equal to 0 specifies that the lmcs data of any direct reference layer are not referenced by the current picture. ph_lmcs_inter_layer_ref_enabled_flag equal to 1 specifies that the lmcs data of a direct reference layer may be referenced by the current picture. When ph_lmcs_inter_layer_ref_enabled_flag is not present, the value of ph_lmcs_inter_layer_ref_enabled_flag is inferred to be equal to 0.

pic_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. If ph_lmcs_inter_layer_ref_enabled_flag is equal to 1, pic_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS in the layer with index equal to sps_lmcs_inter_layer_id to the list of the direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to pic_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

ph_scaling_list_inter_layer_ref_enabled_flag equal to 0 specifies that the scaling list data of any direct reference layer are not referenced by the current picture. ph_scaling_list_inter_layer_ref_enabled_flag equal to 1 specifies that the scaling list data of a direct reference layer may be referenced by the current picture. When ph_scaling_list_inter_layer_ref_enabled_flag is not present, the value of ph_scaling_list_inter_layer_ref_enabled_flag is inferred to be equal to 0.

pic_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS that the slices associated with the PH refers to. If ph_scaling_list_inter_layer_ref_enabled_flag is equal to 1, pic_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS in the layer with index equal to sps_scaling_list_inter_layer_id to the list of the direct reference layers. Otherwise, the TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to pic_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team, JVET-P2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).