| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0182-v1 |
| --- | --- |

| Title: | AHG9: Allowing slice-level scaling list and LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input Documents to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chen-Yen Lai, Tzu-Der Chuang, Olena Chubach, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {jenny.lai, peter.chuang, olena.chubach, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 7, a scaling list (SL)/luma mapping with chroma scaling (LMCS) adaptation parameter set (APS) ID and other related syntax elements can be signalled in a picture header (PH), and all slices of the picture share the same SL/LMCS settings. However, for video pictures composed of different scenes, e.g., pictures of video conference showing multiple people and PC desktop, picture-level SL/LMCS may be inefficient. For bitstream merging, picture-level SL/LMCS is also not desirable; for example, if two subpictures use different SL/LMCS settings, the two subpicture bitstreams cannot be merged easily. Therefore, more SL/LMCS flexibility within one picture is needed. In this contribution, two methods are proposed to allow different slices within one picture to refer to different SL/LMCS APSs. In Method 1, SL/LMCS syntax elements are signalled either at PH or slice header (SH) level. If SL/LMCS syntax elements are signalled in a PH, they cannot be signalled in any SH of the coded picture associated with the PH. In Method 2, overriding the PH SL/LMCS settings at SH level is allowed. If overriding is enabled by an overriding flag, additional SL/LMCS information is signalled in the SH; otherwise, the PH SL/LMCS settings are used for the current slice.

# Introduction

It is known [1] that there are three kinds of APS types including ALF_APS, SCALING_APS, and LMCS_APS. In the semantics of APS, it is mentioned that different slices within a picture can refer to different adaptive loop filtering (ALF) APSs. However, all slices in one picture only can refer to one SL/LMCS APS at most. This design lacks flexibility, and it is inefficient for the use case that one picture includes different scenes. For example, in one video conference, there could be six participants and two shared screens. Another use case is bitstream merging. One main purpose of subpicture is that each subpicture can be extracted and merged with other subpicture bistreams. However, since the SL/LMCS APS applied to all slices within one coded picture has to be the same, this functionality is hard to be supported. Two subpicture bitstreams with different SL/LMCS APSs cannot be merged together easily.

Therefore, it is suggested to allow different slices within one coded picture to refer to different SL/LMCS APSs. Two methods are proposed in the next section.

# Proposed methods

## Method 1: Signal SL/LMCS syntax elements in either PH or SH

It is proposed to signal the SL/LMCS syntax elements in either PH or SH. If signalled in PH, they cannot be signalled in any SH of the coded picture associated with the PH.

The proposed syntax modifications are highlighted in blue.

Table 1. Picture header RBSP syntax of Method 1

| if( sps_lmcs_enabled_flag ) { |  |
| --- | --- |
| pic_lmcs_enabled_present_flag |  |
| if (pic_lmcs_enabled_present_flag) { |  |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_enabled_present_flag |  |
| if (pic_scaling_list_enabled_present_flag) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| } |  |

pic_lmcs_enabled_present_flag equal to 1 specifies that pic_lmcs_enabled_flag, pic_lmcs_aps_id, and pic_chroma_residual_scale_flag are present in the PH. pic_lmcs_enabled_present_flag equal to 0 specifies that pic_lmcs_enabled_flag, pic_lmcs_aps_id and pic_chroma_residual_scale_flag are not present in the PH. When pic_lmcs_enabled_present_flag is not present, it is inferred to be equal to 0.

pic_scaling_list_enabled_present_flag equal to 1 specifies that pic_scaling_list_present_flag and pic_scaling_list_aps_id are present in the PH. pic_scaling_list_enabled_present_flag equal to 0 specifies that pic_scaling_list_present_flag and pic_scaling_list_aps_id are not present in the PH. When pic_scaling_list_enabled_present_flag is not present, it is inferred to be equal to 0.

Table 2. Slice header syntax of Method 1

if( sps_lmcs_enabled_flag && ! pic_lmcs_enabled_present_flag ) {

slice_lmcs_enabled_flag

if(slice_lmcs_enabled_flag) {

slice_lmcs_aps_id

if( ChromaArrayType != 0 )

slice_chroma_residual_scale_flag

}

}

if( sps_scaling_list_enabled_flag && ! pic_scaling_list_enabled_present_flag ) {

slice_scaling_list_present_flag

if(slice_scaling_list_present_flag) {

slice_scaling_list_aps_id

}

}

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled in the slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is disabled in the slice. When not present, the value of slice_lmcs_enabled_flag is inferred to be equal to pic_lmcs_enabled_flag.

slice_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to slice_lmcs_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_lmcs_enabled_flag is equal to 1 and slice_lmcs_aps_id is not present, the value of slice_lmcs_aps_id is inferred to be equal to the value of pic_lmcs_aps_id.

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual  scaling is enabled for the slice. slice_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is disabled for the slice. When slice_lmcs_enabled_flag is equal to 1 and slice_chroma_residual_scale_flag is not present, it is inferred to be equal to pic_chroma_residual_scale_flag.

slice_scaling_list_present_flag equal to 1 specifies that the scaling list data used for the slice is derived based on the scaling list data contained in the referenced scaling list APS. slice_scaling_list_present_flag equal to 0 specifies that the scaling list data used for the slice is set to be equal to 16. When not present, the value of slice_scaling_list_present_flag is inferred to be equal to pic_scaling_list_present_flag.

slice_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS. The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to slice_scaling_list_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_scaling_list_present_flag is equal to 1 and slice_scaling_list_aps_id is not present and, the value of slice_scaling_list_aps_id is inferred to be equal to pic_scaling_list_aps_id.

Notes for APS in VVC Draft 7:

NOTE 1 – Each type of APSs uses a separate value space for adaptation_parameter_set_id.

NOTE 2 – An APS NAL unit (with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type) can be shared across pictures, and different slices within a picture can refer to different ALF APSs.

NOTE 3 – A suffix APS NAL unit associated with a particular VCL NAL unit (this VCL NAL unit precedes the suffix APS NAL unit in decoding order) is not for use by the particular VCL NAL unit, but for use by VCL NAL units following the suffix APS NAL unit in decoding order.

## Method 2: Allow overriding PH SL/LMCS in SH

In addition to the current SH/LMCS syntax design in the PH, two override flags (one for SL, the other for LMCS) are signalled in each SH to indicate whether more SL/LMCS syntax elements are present in the SH. If the override flag is true, the related syntax elements are signalled in the SH and applied to the current slice. Otherwise, the settings signalled in PH are used for the current slice.

Compared with the previous method, this method can achieve sharing the same settings between some slices and allowing different settings for other slices at the same time. That is, the settings signalled in PH can be shared between different slices within one coded picture. For each slice, if it follows the settings signalled in PH, it signals the override flag equal to 0; otherwise, it signals the override flag equal to 1 and the related syntax elements.

The proposed syntax modifications are highlighted in blue and green.

Table 3. Slice header syntax of Method 2

if( sps_lmcs_enabled_flag) {

slice_lmcs_override_flag

if (slice_lmcs_override_flag) {

slice_lmcs_enabled_flag

if(slice_lmcs_enabled_flag) {

slice_lmcs_aps_id

if( ChromaArrayType != 0 )

slice_chroma_residual_scale_flag

}

}

}

if( sps_scaling_list_enabled_flag) {

slice_scaling_list_override_flag

if (slice_scaling_list_override_flag) {

slice_scaling_list_present_flag

if(slice_scaling_list_present_flag)

slice_scaling_list_aps_id

}

}

slice_lmcs_override_flag equal to 1 specifies that lmcs parameters are present in the slice header. slice_lmcs_override_flag equal to 0 specifies that lmcs parameters are not present in the slice header. When not present, the value of slice_lmcs_override_flag is inferred to be equal to 0.

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled in the slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is disabled in the slice. When not present, the value of slice_lmcs_enabled_flag is inferred to be equal to pic_lmcs_enabled_flag.

slice_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to slice_lmcs_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_lmcs_enabled_flag is equal to 1 and slice_lmcs_aps_id is not present, the value of slice_lmcs_aps_id is inferred to be equal to the value of pic_lmcs_aps_id.

slice_chroma_residual_scale_flag equal to 1 specifies that chroma residual scaling is enabled for the slice. slice_chroma_residual_scale_flag equal to 0 specifies that chroma residual scaling is disabled for the slice. When slice_lmcs_enabled_flag is equal to 1 and slice_chroma_residual_scale_flag is not present, it is inferred to be equal to pic_chroma_residual_scale_flag.

slice_scaling_list_override_flag equal to 1 specifies that scaling list parameters are present in the slice header. slice_scaling_list_override_flag equal to 0 specifies that scaling list parameters are not present in the slice header. When not present, the value of slice_scaling_list _override_flag is inferred to be equal to 0.

slice_scaling_list_present_flag equal to 1 specifies that the scaling list data used for the slice is derived based on the scaling list data contained in the referenced scaling list APS. slice_scaling_list_present_flag equal to 0 specifies that the scaling list data used for the slice is set to be equal to 16. When not present, the value of slice_scaling_list_present_flag is inferred to be equal to pic_scaling_list_present_flag.

slice_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS. The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to slice_scaling_list_aps_id shall be less than or equal to the TemporalId of the coded slice NAL unit. When not present and slice_scaling_list_present_flag is equal to 1, the value of slice_scaling_list_aps_id is inferred to be equal to pic_scaling_list_aps_id.

Notes for APS in VVC Draft 7:

NOTE 1 – Each type of APSs uses a separate value space for adaptation_parameter_set_id.

NOTE 2 – An APS NAL unit (with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type) can be shared across pictures, and different slices within a picture can refer to different ALF APSs.

NOTE 3 – A suffix APS NAL unit associated with a particular VCL NAL unit (this VCL NAL unit precedes the suffix APS NAL unit in decoding order) is not for use by the particular VCL NAL unit, but for use by VCL NAL units following the suffix APS NAL unit in decoding order.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team, JVET-P2001.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).