| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0190-v1 |
| --- | --- |

| Title: | CE5-related: On CC-ALF modifications related to coefficients and signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Olena Chubach, Ching-Yeh Chen, Chen-Yen Lai, Tzu-Der Chuang, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {olena.chubach, chingyeh.chen, jenny.lai, peter.chuang, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution contains two aspects related to CC-ALF modifications. Aspect 1 implies limiting the sum of the coefficients of CC-ALF filters to be equal to 0. This way, it is guaranteed that CC-ALF has a zero response at DC frequency, which results in CC-ALF behaving as a high pass, band pass, or other zero-DC filter. Aspect 2 implies updating the syntax of CC-ALF such that it is aligned to ALF process: disabling CC-ALF when luma ALF is disabled.

The combined results of Aspect 1 and Aspect 2 are as follows (over the CC-ALF common base, Y/Cb/Cr):
In AI: -0.01% / -0.06% / -0.11%
In RA: 0.00% / -0.10% / -0.06%
In LB: -0.01% / -0.22% / -0.05%

# 1 Introduction

A Cross Component Adaptive Loop Filter (CC-ALF) was introduced in JVET-O0636 [1] and is currently widely investigated in the CE5 [2]. This tool operates as part of the adaptive loop filter (ALF) process and makes use of luma sample values to refine each chroma component. The tool is controlled by information in the bitstream. This information includes filter coefficients for each chroma component and a coding tree unit (CTU) level selection of the filter. The filter coefficients are signalled in the adaptation parameter set (APS), and an appropriate APS is referenced in the slice header. At most four filters are allowed per chroma component, and each filter is a 3x4 diamond shaped filter containing 8 unique coefficients. Filter coefficient dynamic range is limited to 6-bit signed, i.e., [-32, 31]. Filtering along virtual boundary makes use of symmetrical line selection to align with regular ALF filtering.

An overview of the CC-ALF process is shown in Figure 1.

Figure 1. Illustration of CC-ALF with respect to other loop filters

# 2 Proposal methods

## Aspect 1. Constraint on the CC-ALF filter coefficients

When CC-ALF is enabled, a linear filter is applied to luma samples, and a residual correction for the chroma samples is generated. In order to retain the average values for Cb and Cr components and not introduce any noticeable color changes, it is proposed to constrain filter coefficients of each CC-ALF filter such that the sum of filter coefficients in each chroma component’s filter is equal to 0. With this constraint, each CC-ALF filter will have a zero response at DC frequency, which results in CC-ALF behaving as a high pass, band pass, or other zero-DC filter. The averages in chroma components are preserved.

In addition, knowing the number of filter coefficients at the decoder and their summation result (equal to 0), the last coefficient of the filter can be determined at the decoder. Thus, if the number of filter coefficients in CC-ALF is equal to M (M equal to 8 in the current design), only M-1 filter coefficients have to be signalled, and last coefficient is derived at the decoder.

The related syntax modifications are highlighted in yellow.

Table 1. ALF APS RBSP syntax

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| alf_chroma_filter_signal_flag | u(1) |
| alf_cross_component_cb_filter_signal_flag | u(1) |
| alf_cross_component_cr_filter_signal_flag | u(1) |
| … |  |
| if ( alf_cross_component_cb_filter_signal_flag ) { |  |
| alf_cross_component_cb_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cb_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 87; j++ ) |  |
| alf_cross_component_cb_coeff_plus32[ k ][ j ] | u(6) |
| } |  |
| } |  |
| if ( alf_cross_component_cr_ filter_signal_flag ) { |  |
| alf_cross_component_cr_filters_signalled_minus1 | ue(v) |
| for( k = 0; k < (alf_cross_component_cr_filters_signalled_minus1+1); k++ ) { |  |
| for ( j = 0; j < 87; j++ ) |  |
| alf_cross_component_cr_coeff_plus32[ k ][ j ] | u(6) |
| } |  |
| } |  |
| } |  |

alf_cross_component_cb_coeff_plus32[ k ][ j ] minus 32 specifies the value of the j-th coefficient of the signalled k-th cross-component Cb filter set. When alf_cross_component_cb_coeff_plus32[ k ][ j ] is not present, it is inferred to be equal to 32.

The signalled k-th cross component Cb filter coefficients CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ] with elements CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ j ], with j = 0..76 are derived as follows:

CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ j ] = alf_cross_component_cb_coeff_plus32[ k ][ j ] − 32	(7-51)

CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 7 ] 
                                  = − (  CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 0 ] 
                                       + CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 1 ] 
                                       + CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 2 ] 
                                       + CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 3 ] 
                                       + CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 4 ] 
                                       + CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 5 ] 
                                       + CcAlfApsCoeffCb[ adaptation_parameter_set_id ][ k ][ 6 ] )	(7-51)

alf_cross_component_cr_coeff_plus32[ k ][ j ] minus 32 specifies the value of the j-th coefficient of the signalled k-th cross-component Cr filter set. When alf_cross_component_cr_coeff_abs[ k ][ j ] is not present, it is inferred to be equal to 32.

The signalled k-th cross component Cr filter coefficients CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ] with elements CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ j ], with j = 0.. 76 are derived as follows:

CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ j ] = alf_cross_component_cr_coeff_plus32[ k ][ j ] − 32	   (7-51)

CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 7 ] 
                                  = − (  CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 0 ] 
                                       + CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 1 ] 
                                       + CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 2 ] 
                                       + CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 3 ] 
                                       + CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 4 ] 
                                       + CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 5 ] 
                                       + CcAlfApsCoeffCr[ adaptation_parameter_set_id ][ k ][ 6 ] )	(7-51)

## Aspect 2. Disabling CC-ALF when luma ALF is disabled

According to the current VVC design [3], chroma ALF can be enabled only when luma ALF is enabled. At the same time, in the CE5 common base [2], it is suggested that CC-ALF can be enabled for chroma samples of a slice/picture, even when ALF is not enabled for this slice/picture. Since CC-ALF is a part of ALF process, it is suggested to align design of CC-ALF to ALF, and therefore disable CC-ALF when luma ALF is not enabled, as it is done for chroma ALF.

For that, it is proposed to modify the CC-ALF syntax as follows:

- signal CC-ALF syntax in picture header (PH) or slice header (SH) only when pic_alf_enabled_flag is equal to 1 at PH level and when slice_alf_enabled_flag is equal to 1 at SH level.

Modifications in the syntax to make CC-ALF signalling conditioned on pic_alf_enabled_flag in PH and on slice_alf_enabled_flag in SH are shown in Table 2 and Table 3, respectively, and are highlighted in yellow.

Table 2. Picture header RBSP syntax.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| …… | u(1) |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

Table 3. General slice header syntax.

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(v) |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

# Simulation results

The proposed methods were implemented on top of VTM7.0 [3]. The simulations were performed following the JVET common test conditions (CTC) [4].

Table 4 contains results of Aspect 1 vs. CE5 CC-ALF common base, Table 5 contains results of proposed Aspect 2 vs. CE5 CC-ALF common base, Table 6 contains results of a combination of Aspect 1 and Aspect 2 vs. CE5 CCALF common base.

Table 4. Aspect 1 (Constraint on the CC-ALF filter coefficients) vs. CE5 CC-ALF common base

|  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 0.00% | 98% | 94% |
| Class A2 | 0.00% | -0.28% | -0.23% | -0.05% | 100% | 99% |
| Class B | -0.01% | -0.01% | -0.05% | -0.01% | 100% | 99% |
| Class C | -0.01% | -0.04% | -0.07% | -0.02% | 101% | 98% |
| Class E | -0.01% | -0.01% | -0.24% | -0.03% | 99% | 96% |
| Overall | -0.01% | -0.06% | -0.11% | -0.02% | 99% | 97% |
| Class D | 0.00% | -0.05% | -0.09% | -0.02% | 98% | 96% |
| Class F | -0.01% | -0.02% | -0.02% | -0.01% | 99% | 101% |
|  |  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.01% | -0.15% | -0.02% | -0.02% | 100% | 101% |
| Class A2 | 0.01% | -0.09% | -0.02% | 0.00% | 101% | 99% |
| Class B | 0.00% | -0.09% | -0.15% | -0.02% | 101% | 100% |
| Class C | 0.00% | -0.08% | -0.03% | -0.01% | 100% | 99% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.07% | -0.02% | 100% | 100% |
| Class D | -0.01% | -0.13% | -0.02% | -0.03% | 100% | 98% |
| Class F | 0.00% | -0.02% | -0.04% | 0.00% | 101% | 99% |
|  |  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.00% | -0.24% | 0.04% | -0.02% | 100% | 99% |
| Class C | 0.04% | -0.10% | -0.26% | -0.01% | 100% | 99% |
| Class E | -0.11% | -0.37% | 0.09% | -0.11% | 101% | 97% |
| Overall | -0.01% | -0.23% | -0.05% | -0.04% | 100% | 99% |
| Class D | -0.05% | -0.32% | -0.55% | -0.13% | 100% | 99% |
| Class F | 0.02% | -0.17% | 0.33% | 0.03% | 99% | 103% |

Table 5. Aspect 2 (Disabling CC-ALF when luma ALF is disabled) vs. CE5 CC-ALF common base

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 93% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 98% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 98% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 97% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 96% |
|  | Random access Main10 |  |  |  |  |  |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 103% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class B | 0.00% | 0.01% | 0.00% | 0.00% | 100% | 101% |
| Class C | 0.00% | 0.00% | 0.01% | 0.00% | 99% | 103% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class D | 0.00% | -0.01% | -0.01% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.00% | 0.02% | 0.00% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | -0.01% | -0.02% | 0.00% | -0.01% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 98% |
| Overall | 0.00% | -0.01% | 0.00% | 0.00% | 101% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class F | 0.01% | 0.00% | 0.16% | 0.03% | 99% | 99% |

Table 6. Aspect 1 + Aspect 2 vs. CE5 CC-ALF common base

|  | All Intra Main10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | 0.00% | -0.01% | 0.00% | 0.00% | 98% | 95% |
| Class A2 | 0.00% | -0.28% | -0.23% | -0.05% | 99% | 100% |
| Class B | -0.01% | -0.01% | -0.05% | -0.01% | 99% | 97% |
| Class C | -0.01% | -0.04% | -0.07% | -0.02% | 99% | 98% |
| Class E | -0.01% | -0.01% | -0.24% | -0.03% | 97% | 96% |
| Overall | -0.01% | -0.06% | -0.11% | -0.02% | 98% | 97% |
| Class D | 0.00% | -0.05% | -0.09% | -0.02% | 97% | 96% |
| Class F | -0.01% | -0.02% | -0.02% | -0.01% | 98% | 96% |
|  | Random access Main10 |  |  |  |  |  |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 | -0.01% | -0.15% | -0.02% | -0.02% | 99% | 100% |
| Class A2 | 0.01% | -0.09% | -0.02% | -0.01% | 99% | 99% |
| Class B | 0.00% | -0.08% | -0.15% | -0.02% | 99% | 101% |
| Class C | 0.00% | -0.08% | -0.02% | -0.01% | 99% | 102% |
| Class E |  |  |  |  |  |  |
| Overall | 0.00% | -0.10% | -0.06% | -0.02% | 99% | 101% |
| Class D | -0.01% | -0.13% | -0.03% | -0.03% | 99% | 100% |
| Class F | 0.00% | -0.02% | -0.03% | 0.00% | 100% | 102% |
|  | Low delay B Main10 |  |  |  |  |  |
|  | Over VTM-7.0 with CE5 common base |  |  |  |  |  |
|  | Y | U | V | YUV* | EncT | DecT |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B | 0.00% | -0.23% | 0.02% | -0.02% | 99% | 98% |
| Class C | 0.04% | -0.10% | -0.26% | -0.01% | 99% | 99% |
| Class E | -0.11% | -0.37% | 0.09% | -0.11% | 99% | 95% |
| Overall | -0.01% | -0.22% | -0.05% | -0.04% | 99% | 97% |
| Class D | -0.05% | -0.32% | -0.55% | -0.13% | 99% | 97% |
| Class F | 0.00% | -0.22% | 0.35% | 0.01% | 97% | 101% |

# References

- K. Misra F. Bossen A. Segall, “Cross-component adaptive loop filter for chroma,” Document of Joint Video Experts Team, JVET-O0636.

- C.-Y. Chen A. Segall, “Description of Core Experiment 5 (CE5): Cross-component adaptive loop filtering,” Document of Joint Video Experts Team, JVET-P2025.

- VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

- F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).