| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0193-v1 |
| --- | --- |

| Title: | LFNST signalling cleanup with TS checking |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu, Chia-Ming Tsai, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang, cw.hsu, chia-ming.tsai, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VVC Draft 7, combination of transform skip (TS) and low frequency non-separable secondary transform (LFNST) is not allowed, so if the TS flag for Y is true, the LFNST index is inferred to 0. However, TS was extended to Cb and Cr according to the adoption of JVET-P0058, so the TS flags are signalled for Y, Cb, and Cr, separately. Therefore, only checking the TS flag for Y is not enough for single tree and does not make sense for chroma dual tree. In this contribution, three methods are proposed. In Method 1, if any component’s TS flag in a coding unit (CU) of a coding tree is equal to 0, the LFNST index of the CU can be signalled. In Method 2, if every component’s TS flag in a CU of a coding tree is equal to 0, the LFNST index of the CU can be signalled. In Method 3, for any CU of single tree, if TS flag for Y is equal to 0, the LFNST index of the CU can be signalled; for any CU of dual tree, no additional check is added. With the proposed Method 1 and Method 2, LFNST is more properly used by considering the chroma TS settings. As for Method 3, it is a simple way to correct the wrong interaction between TS and LFNST, and it has no BD-rate impact to VTM7.0. Compared to VTM7.0 common test condition (CTC), the BD-rate and runtime changes of Method 1 and Method 2 are negligible.

# Introduction

In VVC Draft 7, LFNST can be utilized to further concentrate energy of transform coefficients after core transform. Totally, four transform sets and two non-separable transform matrices per transform set are used in LFNST. The selection of transform set depending on the intra prediction mode is defined in Table 1. Moreover, the LFNST index is signalled at CU level. If the LFNST index is equal to 0, no secondary transform is applied; otherwise, the LFNST index is used to indicate the selected transform matrix. For dual tree, separate LFNST indices are signalled for luma and chroma CUs. For single tree, luma and chroma CBs share the same LFNST index.

However, LFNST is disabled when TS is used, so in VVC Draft 7, the TS flag for Y is checked before the LFNST signalling. If the TS flag for Y is true, the LFNST index is inferred to 0. Per adoption of JVET-P0058 [1], TS is extended to chroma components, and the TB-level TS flags for Y, Cb, and Cr are signalled, respectively. According to this adoption, more checks should be added before the LFNST signalling. This contribution proposes to consider the TS flags more properly.

Table 1. Transform set selection depending on the intra prediction mode

| IntraPredMode | Transform set |
| --- | --- |
| IntraPredMode < 0 | 1 |
| 0 <= IntraPredMode <= 1 | 0 |
| 2 <= IntraPredMode <= 12 | 1 |
| 13 <= IntraPredMode <= 23 | 2 |
| 24 <= IntraPredMode <= 44 | 3 |
| 45 <= IntraPredMode <= 55 | 2 |
| 56 <= IntraPredMode<= 80 | 1 |
| 81 <= IntraPredMode<= 83 | 0 |

# Proposed methods

Three methods are proposed to consider the TS flags before the LFNST signalling.

## Method 1

For single tree, if any one of the TS flags for Y, Cb, and Cr is equal to 0, LFNST index can be signalled. For chroma dual tree, if any one of the TS flags for Cb and Cr is equal to 0, the LFSNT index can be signalled. For luma dual tree, if the TS flag for Y is equal to 0, the LFNST index can be signalled.

Note that the spirit for Method 1 can be achieved with the binary variable, LfnstDcOnly, which already exists in VVC Draft 7 to decide whether to signal the LFNST index. If LfnstDcOnly is true, the LFNST index is inferred to be 0. The derivation of LfnstDcOnly is described as follows. First, LfnstDcOnly is initialized with 1. Then, during parsing each TB, LfnstDcOnly is updated to 0 when any non-DC significant coefficient exists. Therefore, if TS is applied to all TBs, which means the condition of “any one of the TS flags equal to 0” fails, LfnstDcOnly is kept as 1, and the LFNST index is not signalled.

## Method 2

For single tree, if the TS flags for Y, Cb, and Cr are all equal to 0, LFNST index can be signalled. For chroma dual tree, if the TS flags for Cb and Cr are both equal to 0, the LFSNT index can be signalled. For luma dual tree, if the TS flag for Y is equal to 0, the LFNST index can be signalled.

## Method 3

For single tree, if the TS flag for Y is equal to 0, LFNST index can be signalled. For chroma and luma dual trees, no additional check is added. Because of the usage of LfnstDcOnly, in the dual-tree cases, if all TBs in the same tree use TS, the LFNST index is not signalled. Therefore, the redundancy between LFNST and TS is removed.

# Proposed changes to VVC draft text

In VVC Draft 7, the changes for the proposed methods are highlighted in Tables 2, 3, and 4.

Table 2. Method 1 in VVC Draft 7

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| … |  |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && (treeType == DUAL_TREE_CHROMA? (transform_skip_flag[ x0 ][ y0 ][ 1] = = 0 ¦¦ transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0) : (treeType == SINGLE_TREE ? (transform_skip_flag[ x0 ][ y0 ][ 0] = = 0 ¦¦ transform_skip_flag[ x0 ][ y0 ][ 1] = = 0 ¦¦ transform_skip_flag[ x0 ][ y0 ][ 2] = = 0): transform_skip_flag[ x0 ][ y0 ][ 0] = = 0)) && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

Table 3. Method 2 in VVC Draft 7

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| … |  |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && (treeType == DUAL_TREE_CHROMA? (transform_skip_flag[ x0 ][ y0 ][ 1] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2 ] = = 0) : (treeType == SINGLE_TREE ? (transform_skip_flag[ x0 ][ y0 ][ 0] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 1] = = 0 && transform_skip_flag[ x0 ][ y0 ][ 2] = = 0): transform_skip_flag[ x0 ][ y0 ][ 0] = = 0)) && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

Table 4. Method 3 in VVC Draft 7

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| chType = treeType = = DUAL_TREE_CHROMA ? 1 : 0 |  |
| … |  |
| if( cu_cbf ) { |  |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_sbt_enabled_flag && !ciip_flag[ x0 ][ y0 ] && !MergeTriangleFlag[ x0 ][ y0 ] && cbWidth <= MaxTbSizeY && cbHeight <= MaxTbSizeY ) { |  |
| allowSbtVerH = cbWidth >= 8 |  |
| allowSbtVerQ = cbWidth >= 16 |  |
| allowSbtHorH = cbHeight >= 8 |  |
| allowSbtHorQ = cbHeight >= 16 |  |
| if( allowSbtVerH ¦ ¦ allowSbtHorH ) |  |
| cu_sbt_flag | ae(v) |
| if( cu_sbt_flag ) { |  |
| if( ( allowSbtVerH ¦ ¦ allowSbtHorH ) && ( allowSbtVerQ ¦ ¦ allowSbtHorQ ) ) |  |
| cu_sbt_quad_flag | ae(v) |
| if( ( cu_sbt_quad_flag && allowSbtVerQ && allowSbtHorQ ) ¦ ¦ ( !cu_sbt_quad_flag && allowSbtVerH && allowSbtHorH ) ) |  |
| cu_sbt_horizontal_flag | ae(v) |
| cu_sbt_pos_flag | ae(v) |
| } |  |
| } |  |
| if( sps_act_enabled_flag && CuPredMode[ chType ][ x0 ][ y0 ] != MODE_INTRA && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && (treeType = = SINGLE_TREE && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0) && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

# Experimental results

The proposed methods have been implemented on VTM7.0 [2]. Table 5 and Table 6 show the results of the proposed Method 1 and Method 2 under the CTC [3], respectively. Method 3 does not change BD-rates.

Table 5.  Results of the proposed Method 1 under CTC (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.00% | -0.06% | 100% | 99% |
| Class C | 0.01% | 0.06% | 0.06% | 100% | 99% |
| Class E | 0.00% | 0.00% | 0.04% | 100% | 100% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | -0.02% | 0.09% | 102% | 98% |
| Class F |  |  |  |  |  |

Table 6.  Results of the proposed Method 2 under CTC (to be updated)

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.00% | -0.03% | 0.05% | 100% | 97% |
| Class E | 0.00% | 0.00% | -0.03% | 101% | 98% |
| Overall |  |  |  |  |  |
| Class D | 0.00% | 0.03% | 0.01% | 100% | 99% |
| Class F |  |  |  |  |  |

# Conclusion

When TS is applied, LFNST is disabled, and the LFNST index can be inferred to 0. Per adoption of JVET-P0058, TS is also applied to chroma components and the TS flags are signalled for Y, Cb, and Cr, separately. However, in VVC Draft 7, before the LFNST signalling, only the TS flag for luma is considered. In this contribution, three methods are proposed. For Method 1 and Method 2, the TS flags for Cb and Cr are also checked for single tree and chroma dual tree to make the syntax design more reasonable. For Method 3, only one check for single tree is added, and the condition of the LFNST signalling can be corrected with minimum changes. Method 1 and Method 2 cause very minor BD-rate changes compared to VTM7.0, while Method 3 has no BD-rate impact.

# References

[1]    T. Tsukuba, M. Ikeda, Y. Yagasaki, and T. Suzuki, “CE8-2.1: Transform skip for chroma with limiting maximum number of context-coded bin in TS residual coding,” Document of Joint Video Experts Team, JVET-P0058.

[2]    VTM7.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.0

[3]    F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).