| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0197-v1 |
| --- | --- |

| Title: | Miscellaneous fixes for HLS in Specification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Biao Wang, Semih Esenlik, Han Gao, Anand Meher Kotra, Elena Alshina | Tel: Email: | biao.wang@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

This contribution proposes to fix three aspects of specification bug/typos in the current working draft JVET-Q0041-v2. First, the flags of ref_pic_list_sps_flag and ref_pic_list_idx regarding reference picture list (RPL) are not defined, and all semantics related to these two flags are broken. Second, the signaling of flag inter_layer_ref_pics_present_flag is modified to be conditionally signalled, in order to align its semantics. Finally, two typos about the flag ph_poc_msb_cycle_present_flag by POC msb cycling signaling is fixed.

# Proposed fixes

For the latest working draft of VVC [1], three aspects of fixes about High Level Syntax (HLS) are proposed.

## RPL in Picture Header and Slice Header

A new syntax structre Picture Header (PH) is adopted in the 16th JVET meeting held in Geneva. The syntax elements in PH are applied to all slices within a picture, while the syntax elements in slice header (SH) are applied per slice. A two level method is used for signaling reference picture list (RPL), either in PH or SH. Because of this, the syntax and semantics of SH regarding RPL are largely inherited by PH. However, the old RPL syntax elements ref_pic_list_sps_flag and ref_pic_list_idx in slice header are not replaced by the new respective PH and SH syntax elements properly. In JVET-Q0041-v2, ref_pic_list_sps_flag shall be replaced by pic_rpl_sps_flag and slice_rpl_sps_flag in PH and SH, respectively, and ref_pic_list_idx shall be respectively replaced by pic_rpl_idx in PH and slice_rpl_idx in SH.

In addition to the above typo fixes, several semantics involving these two old syntax elements have been fixed as well. In particular, one type of semantics fix is for syntax element in Picture Parameter Set (PPS) which indicating whether RPL information shall be derived implicitly from Sequence Parameter Set (SPS) or shall be signaled explicitly in PH (or in SH).

The typo/bug fixes regarding this aspect are summarized in the attached work draft modification, with reviewer labeled as “RPL fix”

## Modification of inter_layer_ref_pics_present_flag signaling

The flag inter_layer_ref_pics_present_flag in SPS has following semantics in JVET-Q0041-v2:

“inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0.”

When sps_video_parameter_set_id is equal to 0, there is only one single layer in the current coded video sequence (CVS). Therefore, inter_layer_ref_pics_present_flag shall be 0 since no inter-layer dependency will occur in this case. To align with the semantics, it is suggested to signal this flag, conditionally based on sps_video_parameter_set_id, which is signaled also in SPS.

Similarly, when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the current layer is an independent layer. Therefore, there is no inter-layer reference picture (ILRP) will be used and the value of inter_layer_ref_pics_present_flag shall be 0.

It is suggested to modify the signaling of inter_layer_ref_pics_present_flag as follows, to restrict the signaling based on above two conditions, in order to reduce the bitstream conformance tests.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| … |  |
| long_term_ref_pics_flag | u(1) |
| if (sps_video_parameter_set_id != 0 ¦¦ vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] ==0) |  |
| inter_layer_ref_pics_present_flag | u(1) |
| … |  |

And the semantics is modified to as follows:

“inter_layer_ref_pics_present_flag equal to 0 specifies that no ILRP is used for inter prediction of any coded picture in the CLVS. inter_layer_ref_pics_flag equal to 1 specifies that ILRPs may be used for inter prediction of one or more coded pictures in the CLVS. When sps_video_parameter_set_id is equal to 0, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0. When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 1, the value of inter_layer_ref_pics_present_flag shall be equal to 0 When not present, the value of inter_layer_ref_pics_present_flag is inferred to be equal to 0.”

These modifications can also be found in the attached work draft modification with reviewer labeled as ‘inter-layer flag signaling’

## POC msb cycling flag fix

It is noted that in the latest working draft, the flag ph_poc_msb_cycle_present_flag is not defined. According to the context and its original proposal for adoption [2], it shall be replaced by ph_poc_msb_present_flag. This modification is labeled as ‘POC flag fix’

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- “AHG2: Editorial input on VVC draft text”, JVET-Q0041-v2, Y.-K. Wang, B. Bross, V. Drugeon, J. Chen

- “AHG8: Scalability - random access”, JVET-P0116-v1, Y.-K. Wang