| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0198-v1 |
| --- | --- |

| Title: | AHG9: On picture header dependency |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Ching-Yeh Chen, Chen-Yen Lai, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chingyeh.chen, jenny.lai, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to specify the picture header (PH) dependency for the picture associated to the PH. The proposed PH dependency with the picture identifier may reduce PH size toward the efficiency in the bitstream.

# Introduction

In the VVC Draft 7 [1], the PH is added. The PH collectively stores those parameters that may not change per slice for the slices associated to it and the PH is required as start of a picture. The PH structure is specified as follows.

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| if( !sps_loop_filter_across_virtual_boundaries_disabled_present_flag ) { |  |
| ph_loop_filter_across_virtual_boundaries_disabled_present_flag | u(1) |
| if( ph_loop_filter_across_virtual_boundaries_disabled_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_x[ i ] | u(13) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundaries_pos_y[ i ] | u(13) |
| } |  |
| } |  |
| if( separate_colour_plane_flag = = 1 ) |  |
| colour_plane_id | u(2) |
| if( output_flag_present_flag ) |  |
| pic_output_flag | u(1) |
| pic_rpl_present_flag | u(1) |
| if( pic_rpl_present_flag ) { |  |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| if( pic_rpl_sps_flag[ i ] ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 1 && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_idx[ i ] | u(v) |
| } else |  |
| ref_pic_list_struct( i, num_ref_pic_lists_in_sps[ i ] ) |  |
| for( j = 0; j < NumLtrpEntries[ i ][ RplsIdx[ i ] ]; j++ ) { |  |
| if( ltrp_in_slice_header_flag[ i ][ RplsIdx[ i ] ] ) |  |
| pic_poc_lsb_lt[ i ][ j ] | u(v) |
| pic_delta_poc_msb_present_flag[ i ][ j ] | u(1) |
| if( pic_delta_poc_msb_present_flag[ i ][ j ] ) |  |
| pic_delta_poc_msb_cycle_lt[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| } |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| pic_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag ) { |  |
| pic_sao_enabled_present_flag | u(1) |
| if( pic_sao_enabled_present_flag ) { |  |
| pic_sao_luma_enabled_flag | u(1) |
| if(ChromaArrayType != 0 ) |  |
| pic_sao_chroma_enabled_flag | u(1) |
| } |  |
| } |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| if ( !pps_dep_quant_enabled_flag ) |  |
| pic_dep_quant_enabled_flag | u(1) |
| if( !pic_dep_quant_enabled_flag ) |  |
| sign_data_hiding_enabled_flag | u(1) |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| if( picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| rbsp_trailing_bits( ) |  |
| } |  |

picture header (PH): A syntax structure containing syntax elements that apply to all slices of a coded picture.

# Proposal

When the PH functions as the indication of start of a picture with all stored parameters needed for coded slices in the picture associated to the PH, the PH size matters toward the efficiency in the bitstream. If the PH does not change often per picture basis, a shorter version of the PH may be used with dependency on the PH for a preceding picture.

In this contribution, we propose to enable the dependency between the PHs in terms of the picture order count in the PH as proposed as the picture identifier in JVET-Q0177 (On picture identification and PH repetition), i.e., a PH may depend on a preceding PH with a different value of ph_pic_order_cnt_lsb in the PH. The proposed text is as follows (highlighted in green)

dependent picture header: A picture header for which the values of some syntax elements of the picture header are inferred from corresponding syntax element values of the picture header for an eligible preceding picture in decoding order.

independent picture header: A picture header for which the values of the syntax elements of the picture header are not inferred from the values of the picture header for a preceding picture.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| … |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| pps_dependent_pic_header_enabled_flag | u(1) |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| … |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| picture_header_rbsp( ) { |  | Descriptor |
| --- | --- | --- |
| ph_pic_order_cnt_lsb |  | u(v) |
| non_reference_picture_flag |  | u(1) |
| gdr_pic_flag |  | u(1) |
| no_output_of_prior_pics_flag |  | u(1) |
| if( gdr_pic_flag ) |  |  |
| recovery_poc_cnt |  | ue(v) |
| ph_pic_parameter_set_id |  | ue(v) |
| if( pps_dependent_pic_header_enabled_flag ) |  |  |
| dependent_pic_header_flag |  | u(1) |
| if( dependent_pic_header_flag ) |  |  |
| dependnent_poc_cnt_minus1 | ue(v) |  |
| if( !dependent_pic_header_flag ) { |  |  |
| if( sps_poc_msb_flag ) { |  |  |
| ph_poc_msb_present_flag |  | u(1) |
| if( ph_poc_msb_present_flag ) |  |  |
| poc_msb_val |  | u(v) |
| } |  |  |
| … |  |  |
| if( sps_scaling_list_enabled_flag ) { |  |  |
| pic_scaling_list_present_flag |  | u(1) |
| if( pic_scaling_list_present_flag ) |  |  |
| pic_scaling_list_aps_id |  | u(3) |
| } |  |  |
| } |  |  |
| if( picture_header_extension_present_flag ) { |  |  |
| ph_extension_length |  | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |  |
| ph_extension_data_byte[ i ] |  | u(8) |
| } |  |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

pps_dependent_pic_header_enabled_flag equal to 1 specifies the presence of the syntax element dependent_pic_header_flag in the picture headers for coded pictures referring to the PPS. pps_dependent_pic_header_enabled_flag equal to 0 specifies the absence of the syntax element dependent_pic_header_flag in the picture headers for coded pictures referring to the PPS.

ph_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the ph_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the ph_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

NOTE – ph_pic_order_cnt_lsb may be used as the identifier when the PH repetition occurs. The value of ph_pic_order_cnt_lsb shall be the same in all repetitive picture headers. The first slice with slice_pic_order_cnt_lsb equal to ph_pic_order_cnt_lsb of the first PH is the first slice of the picture associated with the PH.

dependent_pic_header_flag equal to 1 specifies that the value of each picture header syntax element that is not present is inferred to be equal to the value of the corresponding picture header syntax element in the picture header for a preceding picture which is specified as the dependent point picture by the syntax element dependnent_poc_cnt_minus1, in decoding order. The current PH is a dependent PH. When dependent_pic_header_flag is not present, the value of dependent_pic_header_flag is inferred to be equal to 0 and the current PH is not a dependent PH.

It is a requirement of bitstream conformance that the value of dependent_pic_header_flag of the picture header for the current picture shall be equal to 0 when the following one or more conditions for the preceding picture as the dependent point picture in decoding order are not satisfied.

- The value of ph_pic_parameter_set_id for the preceding picture is equal to that of the current picture.

- The value of non_reference_picture_flag for the preceding picture is equal to 0.

- The value of nuh_temporal_id_plus1 of the preceding picture is equal to that of the current picture.

- The variable NoIncorrectPicOutputFlag for the preceding picture is set equal to 1.

dependnent_poc_cnt_minus1 plus 1 specifies the dependent point of the PH for decoded pictures in decoding order. If the current picture associated with the PH and there is a picA precedes the current picture in decoding order in the CLVS and that has PicOrderCntVal equal to the PicOrderCntVal of the current picture minus the value of dependnent_poc_cnt_minus1 plus 1, the picture picA is referred to as the dependent point picture and the the dependent point picture is the preceding picture for the dependent PH. The dependent point picture shall not follow the current picture in decoding order. The value of dependent_poc_cnt_minus1 shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

That means, when dependent_pic_header_flag is equal to 1, many syntax elements in the above shaded area are not present in the current PH and the current PH is a dependent PH.

# Conclusion

This contribution proposes to specify the PH dependency for the picture associated to the PH. The proposed PH dependency may reduce PH size toward efficiency in the bitstream. The proponents suggest the group adopt the proposed PH dependency.

# References

- “Versatile Video Coding (Draft 7),” JVET-P2001-vE, October 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).