| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0199-v4 |
| --- | --- |

| Title: | AHG8: Support of ROI (Region-Of-Interest) RPR |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Taoran Lu, Fangjun Pu, Peng Yin, Sean McCarthy, Walt Husak, Tao Chen 432 Lakeside Drive, Sunnyvale, CA, 94085 US | Tel: Email: | {pyin, tlu}@dolby.com |  |
| Source: | Dolby Laboratories, Inc. |  |  |  |

_____________________________

# Abstract

This contribution proposes syntax and semantics to support additional functionality and coding efficiency for reference picture resampling (RPR) in VVC. In particular, this contribution proposes a generalization of reference picture resampling (RPR) in which a region of interest (ROI) within a picture – rather than the entire picture – may be resampled to create a reference picture. The proposed syntax and semantics also support the case in which a reference picture is used to predict a region of interest within a current picture rather than the entire current picture. The coding tool proposed in this contribution may be thought of as either RPR using ROI (ROI RPR), or as reference region resampling (RRR) in which RPR is a special case. It is proposed to signal ROI RPR window offset parameters to substitute for the default RPR scaling window offset parameters when ROI RPR is enabled. The additional decoding complexity increase is minimal.

For test clips with a bitstream comprising regular frames and frames with ROI, test results indicate an average coding performance improvement of approximately 11%, compared to VTM7.0 without ROI support, for LDB and LDP test conditions. When using the ROI feature, decoder complexity relative to VTM7.0 is 102% for LDB and 100% for LDP. Encoder complexity is 105% for LDB and 104% for LDP. There is no change in performance if there is no ROI in the bitstream.

v2 contains additional discussion on bitstream conformance.to constrain memory bandwidth.

v3 contains improvement in the specification

v4 signals ROI window pairs for current picture and reference picture in picture header (PH) and index it in slice header. Editor helps further improve the specification. Remove Annex A section in the document and refers directly to the attached specification text.

# Introduction

The current draft of the VVC specification [1] supports a general framework for reference picture resampling (RPR). RPR enables previously decoded pictures to be resized to create a reference picture from which to predict the current picture. RPR enables pictures of different resolutions to be coded in a bitstream and decoded as a single layer. RPR, as currently specified, does not support creating a reference picture from an ROI within a picture. Nor does it support the use of reference picture to predict an ROI of a current picture.

This contribution proposes additional syntax and semantics for RPR that enable ROIs, rather the entire picture, to be used to create reference pictures, or conversely, to be inter predicted.

ROI RPR, as proposed, is intended to improve coding performance for video sequences that contain switching between wide shots and close-up shots of the same scene. Such shot combinations are common in film and television for dramas, sports, ballet and the arts, etc. With ROI RPR, the quality of the inter prediction can be improved.

At previous JVET meetings, several contributions proposed region-wise scalability using RPR [2][3]. This contribution supports spatial scalability using ROI RPR. ROI RPR enables subregions of previously decoded pictures to be resized to create a reference picture from which to predict all or parts of the current picture in the same layer or in a different spatial scalability layer.

Single-layer ROI RPR is illustrated and compared to RPR in Figure 1. The example is a video sequence coded in low-delay configuration in which each picture uses the previous picture as a reference for inter prediction. In the example illustrated, the video is coded at 3 different resolutions. For picture order count (POC) 0 through 3, pictures are coded at 1920x1080 resolution. POCs 4 through 6 are coded at 1280x720 resolution. POCs 7 through 9 are coded at 960x540 resolution. Finally, POC 10 is coded at 1920x1080 resolution.

In the example of Figure 1, RPR as currently specified in VVC is illustrated for the resolution changes between POCs 3 and 4 and between POCs 9 and 10. RPR is used to downsample POC 3 to create a reference picture for POC 4. RPR is also used to upsample POC 9 to create a reference picture for POC 10.

ROI RPR as proposed in this contribution is illustrated in Figure 1 for the resolution change between POCs 6 and 7. Note there are both downscaling and upscaling aspects to the relationship between POCs 6 and 7. The resolution of POC 7 is smaller than that of POC 6; but the image in POC 7 is an upscaled close-up shot of a subregion of the image of POC 6. RPR as currently specified in VVC does not support this common scenario efficiently, as illustrated in Figure 2 and Figure 3.

Figure 1 Illustration of the conceptual difference between RPR and ROI RPR

Figure 2 illustrates and compares RPR (panel A) and ROI RPR (panel B) from the encoder perspective. Figure 3 illustrates and compares RPR (panel A) and ROI RPR (panel B) from the decoder perspective. Note that for RPR, the reference picture is downsampled to match the smaller resolution of the current picture, whereas for ROI RPR, the ROI is upsampled to match both resolution and content of the current picture.

Figure 2 Comparison of RPR (A) and ROI RPR (B) from the encoder perspective.

Figure 3 Comparison of RPR (A) and ROI RPR (B) from the decoder perspective.

# Proposal

In the 16th JVET meeting, Geneva, CH, October, 2019, JVET-P0590 [4] was adopted as the default mode for signalling a scaling window for the purpose of deriving a scaling ratio for RPR.

This contribution proposes to replace the scaling window offset parameters by ROI window offset parameters when ROI RPR is enabled.

Whereas the scaling window signalled in the PPS is a property of the picture, the ROI window offsets are specified relative to the current picture and the reference picture and therefore cannot be derived from the RPR scaling window and must be signalled.

## ROI design

An ROI window is specified by four parameters, similar to the scaling window: leftOffset (LO), rightOffset (RO), topOffset (TO) and botOffset (BO).

(To introduce the concept, the following discussion does not take into consideration the padded boundary pixels, which make the picture size a multiple of 8, as specified by scaling window offset parameters.)

As illustrated in Figure 4, a set of (LOR, ROR, TOR, BOR) is specified in the reference picture, and a set of (LOC, ROC, TOC, BOC) is specified in the current picture.

In current RPR design, without ROI, the scaling ratio is calculated as:

- hor_scale = WR/WC

- ver_scale = HR/HC

With ROI window support, the scaling ratio calculation is modified as:

- hor_scale = (WR-LOR-ROR)/ (WC-LOC-ROC)

- ver_scale = (HR-TOR-BOR)/ (HC-TOC-BOC)

Figure 4 Illustration of ROI window offset parameters for reference and current pictures

To simplify the decoder implementation, the following constraints for ROI RPR are proposed:

- Any ROI picture is completely contained in the full picture (not considering padded boundaries).

- Any picture, current or reference, may contain at most one ROI window.

The first constraint eliminates the need to check for samples outside the ROI window. The second constraint avoids the complication that would result if it were necessary to determine which of several ROI windows should be used for each sample.

Consequently, to meet the first constraint, only three cases are proposed to be supported as listed below and shown in Figure 5:

- Case 1: All ROI offsets for both the reference and current pictures are 0 (LOR=ROR=TOR=BOR=LOC=ROC=TOC=BOC=0), ROI is disabled (default RPR case).

- Case 2: All ROI offsets for the reference picture are all equal to 0 (LOR=ROR=TOR=BOR=0) and at least one ROI offset for the current picture is non-zero, the full reference picture corresponds to an ROI window in the current picture.

- Case 3: All ROI offsets for the current picture are equal to 0 (LOC=ROC=TOC=BOC=0) and at least one ROI offset for the reference picture is non-zero, an ROI window in the reference picture corresponds to the full current picture.

Figure 5 Illustration of the ROI RPR cases proposed

Although not proposed, Case 4, as illustrated in Figure 4, is also supported by the ROI RPR syntax and semantics. In Case 4, the first constraint (an ROI picture is completely contained in the full picture) would be removed. However, extra care would be required in the encoder to properly handle motion estimation and compensation for samples outside of the ROI window.

(To meet the second constraint, the specification only supports one window indication for current picture and reference picture, as shown in Table 2.)

As currently specified, the RPR scaling window is used to handle the padded boundaries that makes the picture size a multiple of 8. It is proposed that the RPR scaling window be the default case when ROI window is not enabled, which is consistent with the decision at the 16th JVET meeting. What that means is as follows:

- If ROI RPR is enabled, scaling window offset parameters are replaced by ROI window offset parameters in the derivation of the scaling ratio and in offset handling in the fractional interpolation process;

- Otherwise (ROI RPR is disabled), scaling window offset parameters are applied as in the current VVC design.

## ROI syntax specification

It is proposed to signal ROI window offset parameters as a list in the SPS, as shown in Table 1. Doing so would support multiple reference pictures to be used to code the current picture, whereas different ROI window can be used for each reference picture.

Signalling candidate ROI window lists in the SPS would be efficient when the ROI window does not change frequently. Alternatively, if ROI windows changed frequently, the ROI window lists could be signalled in an APS. The maximum number of candidate ROI windows is set to 4. (The complete syntax and semantics can be found in the attached specification: syntax in 7.3.3.3 and semantics in 7.4.3.3.)

Table 1 ROI window list in sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ref_pic_resampling_enabled_flag ) { |  |
| sps_rpr_roi_window_present_flag | u(1) |
| if( sps_rpr_roi_window_present_flag ) { |  |
| num_candidate_roi_windows_minus1 | ue(v) |
| for( i = 0; i <= num_candidate_roi_windows_minus1; i++ ) { |  |
| roi_offset_prec_shift[ i ] | ue(v) |
| roi_offset_list[ i ][ 0 ] /*left */ | ue(v) |
| roi_offset_list[ i ][ 1 ] /*right */ | ue(v) |
| roi_offset_list[ i ][ 2 ] /*top */ | ue(v) |
| roi_offset_list[ i ][ 3 ] /*bottom */ | ue(v) |
| } |  |
| } |  |
| } |  |

It is proposed to signal the pair of ROI windows in the  picture header and in slice header to signal the index to the pairs, as shown in Table 2. Each pair of reference and current picture may have a different ROI correspondence. An ROI window may be present in a reference picture, or in the current picture, or not present at all.

Table 2 ROI window specification in picture and slice header syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if(sps_rpr_roi_window_present_flag ) { |  |
| pic_rpr_roi_window_enabled_flag | u(1) |
| if( pic_rpr_roi_window_enabled_flag ) { |  |
| pic_num_curr_ref_pairs_minus1 | ue(v) |
| for( i = 0; i < pic_num_curr_ref_pairs; i++ ) { |  |
| curr_roi_window_present_flag[ i ] | u(1) |
| if( curr_roi_window_present_flag[ i ] && num_candidate_roi_windows_minus1 > 0 ) |  |
| curr_roi_offset_idx[ i ] | u(v) |
| ref_roi_window_present_flag[ i ] | u(1) |
| if( ref_roi_window_present_flag[ i ] && num_candidate_roi_windows_minus1 > 0 ) |  |
| ref_roi_offset_idx[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

| slice_header( ) { | Descriptor |
| --- | --- |
| … | u(v) |
| if( pic_rpr_roi_window_enabled_flag ) |  |
| for( i = 0; i < 2; i++ ) |  |
| for( j = 0; j < NumRefIdxActive[ i ]; j++ ) |  |
| if( pic_num_curr_ref_pairs_minus1 > 0 ) |  |
| slice_curr_ref_pair_idx[ i ][ j ] | ue(v) |
| … |  |
| } |  |

Changes to the specification to support ROI RPR in the scaling ratio computation and scaling condition determination (RefPicIsScaled) are specified in 8.3.2 (Decoding process for reference picture lists construction). Changes for inter prediction to support ROI RPR are specified in 8.5.6.3.1 (Fractional sample interpolation process: General subsection). The key aspect is that the offset parameters need to be updated with the ROI window offset parameters. Other changes to the existing decoding process are minimal.

## Supporting ROI with RPR scaling ratio equal to 1.0

In the current VVC draft specification [1], RPR is only enabled when scaling ratio is not equal to 1, i.e., in the case that the reference picture and current picture have different effective resolutions. As such, a scaling ratio equal to 1 is equivalent to no RPR (reference picture and current picture have the same effective resolution). However, ROI RPR is meaningful in the case in which reference picture and current picture contain an ROI window having the same resolution (a scaling ratio equal to 1). In this case, the ROI is created by cropping alone without additional re-sampling.

The current VVC design for RPR includes tests if scaling ratio is equal to 1 to enable RPR and to determine the applicability of serveral inter coding tools, such as TMVP, PROF/BDOF, DMVR, etc. ROI RPR requires an additional check to determine if the current picture and reference picture have the same output resolution for scaling ratio equal to 1. If the scaling ratio is equal to 1, and reference and current pictures have different output resolutions, then the decoding process takes ROI window offset parameters into consideration.

To be more specific, currently, the RPR condition only checks if scaling ratio is equal to 1, see below:

RefPicIsScaled = (RefPicScale[ i ][ j ][ 0 ]  !=  ( 1   <<  14 ) )  | |( RefPicScale[ i ][ j ][ 1 ]  !=  ( 1   <<  14 ) )

The proposed ROI RPR condition adds one more condition to check if the PicOutputWidthL of the reference and current picture or the PicOutputHeightL of the reference and current picture is not equal when scaling ratio equals to 1 (as shown in Figure 6), which indicates there is an ROI either in current picture or reference picture.

Figure 6 ROI window with scaling ratio equal to 1 but different reference and current picture resolutions

## Supporting ROI scalability as an extended spatial scalability

Current VVC draft specification supports spatial scalability using the RPR framework, where the base layer is used as the long-term reference picture (LTRP). To support ROI RPR with the constraints specified in Section 2.1 above, the base layer (ROI picture) needs to be completely included in the enhancement full resolution picture. The base layer can have various scaling ratio (A:1x, B: <1x, and C: >1x), as shown in Figure 7.

Figure 7 ROI scalability support

Note that spatial scalability can be combined with single layer RPR. An example of a two layer bitstream consisting of a base layer (BL) and an enhancement layer (EL) is shown in Figure 8. In the BL, each frame uses the previous frame as the reference picture for inter prediction. The BL bitstream contains the cases which uses RPR (POC 8) and ROI RPR (POC 3). In the EL, each frame uses the previous frame as the reference picture for inter prediction. The EL bitstream contains the cases which uses RPR (POC 4 and 10) and ROI RPR (POC 7). In addition, BL pictures having the same POC as EL coding pictures are used as inter-layer reference pictures for spatial scalability: using RPR (POC 0,1,2,7,8,9,10); and ROI RPR (POC 3,4,5,6).

Figure 8 Mixed RPR, ROI and spatial scalability

# Software implementation and simulation

## Software Implementation

The software implementation of ROI RPR is based on VTM7.0 and MR1089 (https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/merge_requests/1089) JVET-P0590: scaling window implementation and JVET-P0592: chroma phase offset implementation. Changes are mainly modifications of the existing RPR framework. Most software changes are made at the encoder side. No additional memory storage is introduced. The key aspects of the implementation are:

Encoder side only:

- The RPR pre-processing module (rescale input pictures at a specified cadence, e.g., every 0.5 second), is modified to add support for cropping and for scaling of the cropped ROI. The cropping position (left and top offset on input picture grid), the scale factors (hor_scale and ver_scale) and the final ROI size are configurable encoder parameters. The right and bottom ROI offsets are automatically derived based on the other parameters. Examples are shown in Figure 9.

- ROI with 0.5X: crop and upscale by 2 to produce ROI

- ROI with 1X: crop only (no resampling) to produce ROI.

- ROI with 2X: crop and downscale by 2 to produce ROI.

Figure 9 Examples of ROI RPR with different scaling ratios

The encoder also supports the dumping of this pre-processed video (skipping the encoding process) to generate non-compressed YUV data for the comparison tests. When pictures have different sizes, the largest picture size is used as the container size and each smaller picture is be placed in the upper left corner of the container. Unused sample positions are set to 0.

- The encoder side reference picture rescaling function is modified to take into account the ROI window offset parameters, as shown in Figure 10:

  - If the reference picture corresponds to an ROI in current picture, scaling with boundary padding is performed to generate the scaled reference picture

  - If the current picture corresponds to an ROI in reference picture, scaling with cropping is performed to generate the scaled reference picture

Figure 10 Encoder-side reference picture handling

- High level syntax: the ROI window list is signalled in the SPS and the index of ROI window used for each reference picture is signalled in the slice header.

Common (Encoder and Decoder):

- Inter prediction module: fractional sample interpolation process is modified to take into account the ROI window offsets and ROI RPR scaling factor

- Enable RPR for scale=1X with ROI

Decoder side only:

- High level syntax: parsing the ROI window list in the SPS and the index of ROI window used for each reference picture in slice header.

## Experiment design and test condition

The test condition follows the test design for RPR. Low delay B and low delay P tests are performed with SDR CTC YCbCr4:2:0 test clips. The switching between original input and ROI RPR occurs every second 0.5 seconds, as indicated in Figure 11. To reduce the simulation time, FractionNumFrames is set to 0.5 as in RPR tests to encode only half of the entire clip.

Figure 11 experiment design: switching between original input and ROI every 0.5 sec

Experiments are designed to verify the implementation and to investigate the performance. ROI RPR with three different scaling factors are tested: 0.5X, 1X and 2X, as shown in Figure 9. The ROI offsets are set as follows, where an ROI window is set to the center of the picture:

LeftOffset = inputWidth / 4

TopOffset = inputHeight/ 4

For scale=0.5X tests, ROI finalWidth is set equal to inputWidth and ROI finalHeight is set equal to inputHeight;

For scale=1X tests, ROI finalWidth is set equal to 8*(floor(inputWidth/16)) and ROI finalHeight is set equal to 8*(floor(inputHeight/16));

For scale=2X tests, ROI finalWidth is set equal to 8*(floor(inputWidth/32)) and ROI finalHeight is set equal to 8*(floor(inputHeight/32));

The Right and Bottom ROI window offsets are automatically calculated based on the previous parameters:

RightOffset = inputWidth – LeftOffset – hor_scale*finalWidth

BottomOffset = inputHeight – TopOffset – ver_scale*finalHeight

Since ROI RPR involves the cropping of the original CTC content, it is not meaningful to compare objective metrics (PSNRs) vs. regular coding of the original CTC content. In order to investigate the performance of the proposed framework, an additional reference simulation was designed and performed. For the test of ROI RPR with 0.5X, since the final ROI size is equal to the original input size (as shown in Figure 12), it is possible to use such pre-processed YUV sequences as the input and encode them using VTM7.0 without any RPR or ROI RPR functionality, and compare the PSNR results from these tests. This comparison is meaningful because, for each frame, the input pictures are the same and PSNR are measured in the same (full) resolution. Thus, any difference in performance is attributable to ROI RPR.

Figure 12 Special test case in which ROI RPR pictures (cropping with 0.5X scaling) are the same size as original input, thus making it possible to be encoded by regular coding without RPR to serve as a reference for coding efficiency study.

## Experiment results and analysis

### Functionality:

The simulations included the complete SDR CTC YCbCr420 test set with the three test cases described in section 3.2: ROI RPR with upscaling, ROI RPR without scaling, and ROI RPR with downscaling. All encoder/decoder matched, thus indicating that the functional aspect of the design is verified.

### Coding efficiency

Test (ROI RPR): ROI RPR 0.5X, coding of input CTC clips (automatic ROI cropping/scaling occurs before encoding) with proposed SW;

Ref: coding of pre-processed CTC clips (dumping from automatic ROI cropping/ scaling using proposed SW) with VTM7.0 SW, no RPR.

The simulation results are listed in Table 3. The encoding time was measured using the cluster. The decoding time was measured using a PC station. It can be observed that more than 10% coding efficiency, on average, can be achieved with the proposed solution. The results suggest that signalling the ROI relationship between the reference picture and the current picture can improve the inter prediction quality significantly.

Table 3 Simulation results of coding efficiency test

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | ROI RPR Over ref |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -6.94% | -8.58% | -9.00% | 107% | 102% |
| Class C | -8.64% | -7.59% | -7.47% | 105% | 102% |
| Class E | -21.17% | -22.15% | -20.81% | 101% | 101% |
| Overall | -11.06% | -11.65% | -11.44% | 105% | 102% |
| Class D | -7.81% | -7.43% | -6.96% | 101% | 102% |
| Class F | -16.69% | -15.83% | -15.33% | 94% | 106% |
|  | Low delay P Main10 |  |  |  |  |
|  | ROI RPR Over ref |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.80% | -6.97% | -7.31% | 105% | 101% |
| Class C | -7.91% | -7.43% | -6.48% | 105% | 101% |
| Class E | -20.27% | -22.03% | -20.38% | 101% | 99% |
| Overall | -10.12% | -10.89% | -10.30% | 104% | 100% |
| Class D | -7.21% | -7.28% | -6.83% | 107% | 101% |
| Class F | -16.42% | -15.34% | -14.79% | 94% | 102% |

## Analysis

### Per-frame statistics, bitrate and PSNR

Figure 13 shows an example of detailed per-frame statistics of LDB coding of KristenAndSara QP22. Frame 24, 48, 72 etc. are frames where switching between original input and ROI RPR occurs. The first 0~23 frames are encoded without RPR, thus number of bits and PSNRs are the same for the test and reference. POC 24 is the switching point from original input to ROI. POC 48 is the switching point from ROI to original input. Note that the number of bits is significantly less for the ROI RPR test compared to the reference, but the PSNR for the test and reference are effectively equal.

Figure 13 Example of per-frame coding statistics of KristenAndSara QP22, LDB.

### Mode distribution analysis

To confirm the efficiency of ROI RPR implementation, we visualized mode distributions is using YUView (https://github.com/IENT/YUView).

For POC 24, the current picture is at the 0.5 sec switching point at which the reference picture is an original picture and the current picture (ROI picture) is the central ROI of the original picture resized to the original picture size. In Figure 14, screenshots of YUView show the prediction modes used by VTM 7.0 (left panel A) and the modes used by the proposed ROI RPR (right panel B). Intra prediction modes are indicated in blue and inter prediction modes are indicated in purple. Note that ROI RPR uses almost entirely inter prediction modes, whereas VTM 7.0 uses almost entirely intra prediction modes.

Figure 14 Prediction modes used by VTM7.0 (A) and ROI RPR (B) for KristenAndSara QP22, LDB: POC 24.

Another visualization is shown in Figure 15 for POC 48 where the current picture is the 1s switching point of a full picture and its reference picture is an ROI picture. Note that the ROI RPR test (B) uses more inter prediction modes in the central ROI region than does the VTM 7.0 test (A).

Figure 15 Prediction modes used by VTM7.0 (A) and ROI RPR (B) for KristenAndSara QP22, LDB: POC 48.

The results illustrated in Figure 14 and Figure 15 indicate that ROI RPR is significantly more likely to use inter prediction modes than intra prediction modes at the switch points compared to VTM7.0, which is consistent with the fewer number of bits (Figure 13) used at those point by ROI RPR compared to VTM7.0

### Bit rate overhead

The SPS overhead for signaling the ROI window offset parameters depends on the input picture resolution and the ROI window offset parameter values. The typical overhead collected from experiments is 43~65 bits, as presented in Table 4.

Table 4 ROI window offsets and corresponding SPS overhead in LDB ROI 0.5X tests

| Low Delay B Main 10, 0.5X |  |  |  | ROI offsets |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Width | Height | L | R | T | B | SPS overhead (bits) |
| Class B | MarketPlace | 1920 | 1080 | 480 | 480 | 270 | 270 | 65 |
|  | RitualDance | 1920 | 1080 | 480 | 480 | 270 | 270 | 65 |
|  | Cactus | 1920 | 1080 | 480 | 480 | 270 | 270 | 65 |
|  | BasketballDrive | 1920 | 1080 | 480 | 480 | 270 | 270 | 65 |
|  | BQTerrace | 1920 | 1080 | 480 | 480 | 270 | 270 | 65 |
| Class C | BasketballDrill | 832 | 480 | 208 | 208 | 120 | 120 | 43 |
|  | BQMall | 832 | 480 | 208 | 208 | 120 | 120 | 43 |
|  | PartyScene | 832 | 480 | 208 | 208 | 120 | 120 | 43 |
|  | RaceHorses | 832 | 480 | 208 | 208 | 120 | 120 | 43 |
| Class D | BasketballPass | 416 | 240 | 104 | 104 | 60 | 60 | 41 |
|  | BQSquare | 416 | 240 | 104 | 104 | 60 | 60 | 41 |
|  | BlowingBubbles | 416 | 240 | 104 | 104 | 60 | 60 | 41 |
|  | RaceHorses | 416 | 240 | 104 | 104 | 60 | 60 | 41 |
| ClassE | FourPeople | 1280 | 720 | 320 | 320 | 180 | 180 | 53 |
|  | Johnny | 1280 | 720 | 320 | 320 | 180 | 180 | 53 |
|  | KristenAndSara | 1280 | 720 | 320 | 320 | 180 | 180 | 53 |
| ClassF | BasketballDrillText | 832 | 480 | 208 | 208 | 120 | 120 | 43 |
|  | ArenaOfValor | 1920 | 1080 | 480 | 480 | 270 | 270 | 65 |
|  | SlideEditing | 1280 | 720 | 320 | 320 | 180 | 180 | 53 |
|  | SlideShow | 1280 | 720 | 320 | 320 | 180 | 180 | 53 |

Slice header bits overhead depends on how many reference pictures are used.

For CTC LDB configuration (up to 8 ref pics), slice header overhead is 1~25 bits per slice.

For CTC LDP configuration (up to 4 ref pics), slice header overhead is 1~13 bits per slice.

# Memory bandwidth constraints

The section contains memory bandwidth constraints required for bitstream conformance, as suggested by other contributions [5][6]. We did not list all of them.

## Constraints on scaling ratio

The constraints suggested in JVET-Q0331[5] item 1 alternative method are summarized below:

When RefPicIsScaled[ i ][ j ] is equal to 1, it is a requirement of bitstream conformance that the following constraints apply:

–	RefPicScale[ i ][ j ][ 0 ] shall be smaller than or equal to 2M

–	RefPicScale[ i ][ j ][ 0 ] shall be larger than or equal to 2N

–	RefPicScale[ i ][ j ][ 1 ] shall be smaller than or equal to 2M

–	RefPicScale[ i ][ j ][ 1 ] shall be larger than or equal to 2N

where M = 15 and N = 11.

## Memory access constraint

Any decision agreed to reduce the memory access can be applied to ROI RPR, as ROI window is no bigger than scaling window in RPR.

For example, JVET-Q0179 [6] proposed bitstream conformance requirements related to RPR scaling ratio for worst case MC memory bandwidth reduction. The same constraint can be applied for ROI RPR.

Using their method 1, as example, and rewritten in the context of ROI RPR, this proposal adds the following constraint.

When RefPicIsScaled[ i ][ j ] is equal to 1, it is a requirement of bitstream conformance that the following constraints apply:

- pic_width_in_luma_samples * pic_height_in_luma_samples * (16 * RefPicScale[ i ][ j ][ 0 ] / ( 1   <<  14)  + 7) * (4 * RefPicScale[ i ][ j ][ 1 ]  / ( 1  <<  14) + 7) <= pic_width_max_in_luma_samples * pic_height_max_in_luma_samples * 253

- pic_width_in_luma_samples * pic_height_in_luma_samples * (4 * RefPicScale[ i ][ j ][ 0 ] / ( 1   <<  14)    + 7) * (16 * RefPicScale[ i ][ j ][ 1 ]  / ( 1   <<  14) + 7) <= pic_width_max_in_luma_samples * pic_height_max_in_luma_samples * 253

# Conclusion

This contribution proposes syntax and semantics to support region of interest (ROI) within the RPR framework specified in the current draft of VVC. It is proposed to signal ROI RPR window offset parameters to substitute for the default RPR scaling window offset parameters when ROI RPR is enabled.

The proposed syntax and semantics support ROI RPR for single layer and for spatial scalability.

ROI RPR, as proposed, is intended to improve coding performance for video sequences that contain switching between wide shots and close-up shots of the same scene, which is common in cinematic and television productions. Tests that simulate such video sequences were performed with VTM software modified to support ROI RPR with minor decoding complexity increase. The results indicate average improvement in coding efficiency of approximately 11%, compared to VTM7.0 under LDB and LDP test conditions.”.

It is recommended to adopt the proposed syntax, semantics, and decoding process modification for ROI RPR in VVC.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang,“Versatile Video Coding (Draft 7)”, JVET-P2001-vE, Geneva, CH, Oct. 2019

- B. Choi, S. Zhao, I. Sodagar, S. Wenger, and S. Liu “AHG8: Region-wise scalability support with reference picture resampling”, JVET-O0336, Gothenburg, SE, 3–12 July 2019

- T. Lu, F. Pu, P. Yin, S. McCarty, W. Husak, and T. Chen, “AHG8: Support of ROI (Region-Of-Interest) scalability”, JVET-P0336, Geneva, CH, Oct. 2019

- V. Seregin, M. Coban, M. Karczewicz, “AHG8: On inter-layer reference picture index range”, JVET-P0590, Geneva, CH, Oct. 2019

- Y.-J. Chang, V. Seregin, M. Coban, M. Karczewicz, “AHG8: Constraints on the picture scaling ratios”, JVET-Q0331, Brussels, BE, Jan. 2020

- T.-D. Chuang, O. Chubach, C.-Y. Chen, C.-W. Hsu, L. Chen, Y.-W. Huang, S.-M. Lei, “AHG9: Bitstream conformance requirement related to RPR scaling ratio for worst case MC memory bandwidth reduction”, JVET-Q0179, Brussels, BE, Jan. 2020

# Acknowledgement

We would like to thank Alibaba for crosschecking the simulation results.

# Patent rights declaration(s)

Dolby Laboratories, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

- -