| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0201 |
| --- | --- |

| Title: | [AHG12]: On slice data signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com seethal.paluri@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

It is asserted the current slice data signalling has the following issues:

- The bin indicating the end of slice is checked for every CTU, which is considered unnecessary.

- The presence of the bins specifying the end of tile and end of subset are not conditioned correctly.

- For the SAO signalling, information regarding the position of the CTU with respect to its picture partition is not correctly derived.

This document proposes a modified design of the slice data structure to address these issues.

# Introduction

In the current VVC working draft, it is asserted that the signalling of slice data is inefficient and also may have problem for the following reasons:

- The bin indicating the end of the slice needs to be checked for every CTU.

- The presence of the bins specifying the end of tile and end of slice subset are not conditioned correctly. When a slice is a rectangular slice containing multiple tiles and the tiles are within one tile column or the tile column is the last tile column in the picture containing the slice, the conditions for the presence of end of tile and end of slice subset are incorrect.

- When SAO is enabled, during the decoding of each CTU, decoder needs to understand whether the CTU is at the top of the picture partition (i.e., either slice or the tile) that contained it. Currently this information is unavailable as the decoder only know whether the CTU is at the first CTU row or not. A case in point is when a slice contains two vertical tiles (i.e., one tile is on top of the other), CTU at the first CTU row of the second tile needs to be recognized as the CTU at the top of the partition as well and may affect SAO process.

# Proposal

To fix the above problems, we propose to modify the current signalling of the slice_data( ) structure as follows:

- End of the slice bit (i.e., end_of_slice_one_bit) is signalled at the end of the loop of the CTU loop.

- Define NextCtbAddrInRs (i.e., the index of the next CTU from the current CTU in decoder order in picture raster scan order). When the current CTU is the last CTU in the slice, the NextCtbAddrInRs is set to be the same as CtbAddrInRs.

- The bin indicating the end of tile (i.e., end_of_tile_one_bit) followed by a byte_alignment( ) is signalled when at least one of the following conditions is true:

  - The tile column border of the current CTU is not the same as the tile column border of the next CTU.

  - The tile row border of the current CTU is not the same as the tile row border of the next CTU.

- The bin indicating the end of subset (i.e., end_of_subset_one_bit) followed by a byte_alignment( ) is signalled when both of the following conditions are true:

  - The CTU row of the current CTU is not the same as the CTU row of the next CTU.

  - WPP is enabled (i.e., entropy_coding_sync_enabled_flag is equal to 1).

- The variable FirstCtbRowInSlice is renamed to FirstCtbRowInPicPartition and the value of FirstCtbRowInPicPartition is set as follows:

  - At the beginning of slice data, the value of FirstCtbRowInPicPartition is set to be equal to 1.

  - Every time the CTU row of the current CTU and the next CTU is different, set the FirstCtbRowInPicPartition to be equal to 0

  - When the end of a tile is detected, set the FirstCtbRowInPicPartition to be equal to 1

The implementation of the above proposed changes is shown in the table below.

| slice_data( ) { | Descriptor |
| --- | --- |
| FirstCtbRowInPicPartition = 1 |  |
| for( i = 0; i < NumCtuInCurrSlice; i++ ) { |  |
| CtbAddrInRs = CtbAddrInCurrSlice[ i ] |  |
| CtbAddrX = ( CtbAddrInRs % PicWidthInCtbsY ) |  |
| CtbAddrY = ( CtbAddrInRs / PicWidthInCtbsY ) |  |
| if(CtbAddrX = = CtbToTileColBd[ CtbAddrX ] ) { |  |
| NumHmvpCand = 0 |  |
| NumHmvpIbcCand = 0 |  |
| ResetIbcBuf = 1 |  |
| } |  |
| coding_tree_unit( ) |  |
| NextCtbAddrInRs = (i = = NumCtuInCurrSlice − 1) ? CtbAddrInRs : CtbAddrInCurrSlice[ i + 1 ] |  |
| NextCtbAddrX = ( NextCtbAddrInRs % PicWidthInCtbsY ) |  |
| NextCtbAddrY = ( NextCtbAddrInRs / PicWidthInCtbsY ) |  |
| if( CtbToTileRowBd[ CtbAddrY ] != CtbToTileRowBd[ NextCtbAddrY ] ¦¦ CtbToTileColBd[ CtbAddrX ] != CtbToTileColBd[ NextCtbAddrX ] ) { |  |
| end_of_tile_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| FirstCtbRowInPicPartition = 1 |  |
| } |  |
| else if( CtbAddrY != NextCtbAddrY ) { |  |
| if( entropy_coding_sync_enabled_flag ) { |  |
| end_of_subset_one_bit /* equal to 1 */ | ae(v) |
| byte_alignment( ) |  |
| } |  |
| FirstCtbRowInPicPartition = 0 |  |
| } |  |
| } |  |
| end_of_slice_one_bit /* equal to 1 */ | ae(v) |
| } |  |

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).