| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0203 |
| --- | --- |

| Title: | [AHG12]: On signalling of multiple rectangular slices in a tile |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email: | dr.hendry@lge.com seethal.paluri@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

In the current VVC specification draft, a tile may contain multiple rectangular slices. When a tile contains multiple slices, the height of all these slices, except for the last one in the tile, is explicitly signalled. It is asserted that such signalling is not necessary and inefficient for common case where the height of slices are uniform within a tile. In the 16th JVET meeting, a unified method for signalling uniform and non-uniform tile structure was adopted. It is asserted that the same method is applicable for signalling rectangular slices within a tile. This contribution proposes to modify the current signalling of rectangular slices within a tile by applying the same method for unified signalling of uniform and non-uniform tile.

# Proposal

At the 16th JVET meeting, it was agreed to remove brick signalling while still preserving the feature of having multiple rectangular slices in a tile. To signal such case (i.e., having multiple rectangular slices in a tile), the height of all slices, except the last one, in the tile is signalled explicitly. It is asserted that such signalling is not efficient since it is common that rectangular slices in a tile are uniformly spaced.

To improve the current signalling for multiple rectangular slices in a tile, we propose to use the same method used to unify the uniform and non-uniform tile structure signalling, which was also adopted at the 16th JVET meeting. Text implementation of the method is provided as an attachment in this contribution. It is asserted that the required changes is minimal and more importantly this uses the method that is already used for tile signalling.

Text for the proposal provided as attachment of this contribution.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).