| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0204 |
| --- | --- |

| Title: | [AHG12]: On single tile per slice flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seethal Paluri 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com seethal.paluri@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

This contribution proposes to signal a flag in PPS for specifying that all slices of the picture referring to the PPS are rectangular slices and each contains one tile. It is asserted that the use-case of having each tile contained in a slice is common and it would good if this case is facilitated by current signalling of the tiles and slices.

# Introduction

The current signalling method for tiles and slices can be improved to benefit some of the typical situations. A case in point is when a picture is partitioned into multiple tiles, it is a common scenario to have a single tile per rectangular slice. However, the current working draft (WD 7) of the VVC Specification text does not make provision for this feature.

It is asserted that the flag single_slice_per_subpic_flag specified in the current WD does not cover the above case because it is targetted for the case when subpicture is used such as in viewport dependent for 3600 videos.

# Proposal

We propose to have a flag called single_tile_per_slice_flag, which when equal to 1 is used to specified that all the slices of pictures referring to the PPS are rectangular slices and each of them contains exactly one tile. The proposed flag is present only when rect_slice_flag is equal to 1. When single_tile_per_slice_flag is equal to 1, there is no need to signal further information about the slices as they all can be inferred or derived.

It is also possible to condition the presence of single_tile_per_slice_flag such that it is not present when the value of single_slice_per_subpic_flag is equal to 1. When single_slice_per_subpic_flag is equal to 1, it does not matter what the value of single_tile_per_slice_flag is because derivation of information for each slice would already be handled by the derivation invoked when single_slice_per_subpic_flag is equal to 1.

The text for supporting the proposed flag is given below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) { |  |
| single_slice_per_subpic_flag | u(1) |
| if( !single_slice_per_subpic_flag ) |  |
| single_tile_per_slice_flag | u(1) |
| } |  |
| if( rect_slice_flag && ( !single_slice_per_subpic_flag ¦¦ !single_slice_per_tile_flag ) ) { |  |
| … | ue(v) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

…

single_tile_per_slice_flag equal to 1 specifies that each slice that refers to this PPS is rectangular slice and includes one tile. single_tile_per_slice_flag equal to 0 specifies that the above constraint does not apply. When not present, the value of single_tile_per_slice_flag is inferred to be equal to 0.

num_slices_in_pic_minus1 plus 1 specifies the number of rectangular slices in each picture referring to the PPS. The value of num_slices_in_pic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to 0. When single_tile_per_slice_flag is equal to 1, the value of num_slices_in_pic_minus1 is inferred to be equal to NumTilesInPic – 1.

tile_idx_delta_present_flag equal to 0 specifies that tile_idx_delta values are not present in the PPS and that all rectangular slices in pictures referring to the PPS are specified in raster order according to the process defined in clause 6.5.1. tile_idx_delta_present_flag equal to 1 specifies that tile_idx_delta values may be present in the PPS and that all rectangular slices in pictures referring to the PPS are specified in the order indicated by the values of tile_idx_delta. When single_tile_per_slice_flag is equal to 1, the value of tile_idx_delta_present_flag is inferred to be equal to 0.

slice_width_in_tiles_minus1[ i ] plus 1 specifies the width of the i-th rectangular slice in units of tile columns. The value of slice_width_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileColumns − 1, inclusive. When not present, the following applies:

- If single_tile_per_slice_flag is equal to 1, the value of slice_width_in_tiles_minus1[ i ] is inferred to be equal to 0

- Otherwise, the value of slice_width_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

slice_height_in_tiles_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of tile rows. The value of slice_height_in_tiles_minus1[ i ] shall be in the range of 0 to NumTileRows − 1, inclusive. When not present, the following applies:

- If single_tile_per_slice_flag is equal to 1, the value of slice_height_in_tiles_minus1[ i ] is inferred to be equal to 0

- Otherwise, the value of slice_height_in_tiles_minus1[ i ] is inferred as specified in clause 6.5.1.

num_slices_in_tile_minus1[ i ] plus 1 specifies the number of slices in the current tile for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of num_slices_in_tile_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice. When not present, the following applies:

- If single_tile_per_slice_flag is equal to 1, the value of num_slices_in_tile_minus1[ i ] is inferred to be equal to 0

- Otherwise, the value of num_slices_in_tile_minus1[ i ] is inferred to be equal to 0.

slice_height_in_ctu_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of CTU rows for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of slice_height_in_ctu_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice.

…

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).