| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0205 |
| --- | --- |

| Title: | [AHG12]: On the presence of entry point signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim SangHeon Lee 10225 Willow Creek Rd, San Diego, CA 92131, USA 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Email: | dr.hendry@lge.com seunghwan3.kim@lge.com sangheon1.lee@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

At the 16th JVET meeting, it was agreed to allow entry point signalling to be not present even when tile or WPP is enabled by signalling a flag called entry_point_offsets_present_flag in PPS. This contribution asserted that omitting signalling of tile entry point is problematic for decoder implementation that runs on single core and processing coding tree block in picture raster scan order.

Furthermore, it was also agreed that when entry point signalling is present, all entry points are signalled, which means that when both tiles and WPP are enabled and entry_point_offsets_present_flag is equal to 1, all entry points for tiles and for WPP shall be present. It is asserted that this may not be necessary.

This contribution proposes the following:

- Mandate the presence of entry point signalling in the header of slices that contain multiple tiles.

- Modify entry_point_offsets_present_flag to specify only whether WPP entry point is present or not.

- As the value of entropy_coding_sync_enabled_flag is constrained to be the same for all PPSs that are referred to by coded pictures within a CLVS, the same constraint shall be applied to the value of entry_point_offsets_present_flag as it does not make sense that entry point signalling sometimes is present and some other times is not present.

# Introduction

At the 16th JVET meeting, the following was agreed:

- Entry point for tile and/or WPP may or may not be present. This is controlled by a newly introduced flag called entry_point_offsets_present_flag.

- When entry_point_offsets_present_flag is equal to 1, all entry points are signalled. This means when both tiles and WPP are enabled, both entry points for tile and for WPP are present.

We asserted that while omitting entry point signalling for WPP may be fine since WPP may be more of a parallelism feature for encoder rather than for decoder, and decoder may operate normally without the presence of WPP entry point signalling. However, it is not the case for tile entry point signalling. When a picture is partitioned into two or more tiles, CTU order within the picture changes. Such change may affect the operation of decoder that is a single core and operate based on picture raster scan order only.

Figure below illustrates the case where a picture has only one slice but is partitioned into two tiles. For a single-core decoder implementation, the preferred order of CTU processing would be CTU 0  1  8  9  2  3  10  11  4  5  12  13  6  7  14  15. Such processing would not be possible when tiles’ entry points are not present. Therefore, it is needed that when a slice contains multiple tiles, tiles’ entry points for the slice are signalled.

Figure 1 – A picture with 1 slice, 2 tiles, and 16 CTUs

In the current spec, the values of both no_pic_partition_flag and entropy_coding_sync_enabled_flag are constrained such that each of them has the same value for all PPSs that are referred to by coded pictures within a CLVS. The same constraint should naturally be applied to the flag that control the presence of entry point signalling since it does not make sense if entry point signalling sometimes is present but some other times is not.

# Proposals

We propose the following:

- Mandate the presence of entry point signalling in the header of slices that contain multiple tiles.

- Modify entry_point_offsets_present_flag to specify only whether WPP entry point is present or not.

- As the value of entropy_coding_sync_enabled_flag is constrained to be the same for all PPSs that are referred to by coded pictures within a CLVS, the same constraint shall be applied to the value of entry_point_offsets_present_flag as it does not make sense that entry point sometimes is present and some other times is not present.

The necessary text for the above proposed items are given below:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| … |  |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| if( !no_pic_partition_flag && entropy_coding_sync_enabled_flag ) |  |
| entry_point_offsets_present_flag | u(1) |
| … |  |
| } |  |

entry_point_offsets_present_flag equal to 1 specifies that signalling for subsets of CTU rows may be present in the slice headers of pictures referring to the PPS. entry_point_offsets_present_flag equal to 0 specifies that signalling for subsets of CTU rows are not present in the slice headers of pictures referring to the PPS. When not present, the value of entry_point_offsets_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when present, the value of entry_point_offsets_present_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

…

When no_pic_partition_flag is equal to 0 or entry_point_offsets_present_flag is equal to 1, the variable NumEntryPoints, which specifies the number of entry points in the current slice, is derived as follows:

NumEntryPoints = 0
for( i = 1; i < NumCtuInCurrSlice; i++ ) {
	CctbAddrInRs = CtbAddrInCurrSlice[ i ]
	CctbAddrX = ( CctbAddrInRs % PicWidthInCtbsY )
	CctbAddrY = ( CctbAddrInRs / PicWidthInCtbsY )	(147)
	prevCtbAddrX = (CtbAddrInCurrSlice[ i − 1 ] % PicWidthInCtbsY )
	prevCtbAddrY = (CtbAddrInCurrSlice[ i − 1 ] / PicWidthInCtbsY )
	if( CtbAddrX  = =  CtbToTileColBd[ CtbAddrX ]|  &&
		( CtbAddrY  = =  CtbToTileRowBd[ CtbAddrY ]  | |  entropy_coding_sync_enabled_flag ) )
	if(CtbToTileRowBd[ ctbAddrY ]  !=  CtbToTileRowBd[ prevCtbAddrY ]  | |
		CtbToTileColBd[ ctbAddrX ]  !=  CtbToTileColBd[ prevCtbAddrX ]  | |
		( ctbAddrY  !=  prevCtbAddrY  &&  entropy_coding_sync_enabled_flag ) )
		NumEntryPoints++
}

…

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).