| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0206 |
| --- | --- |

| Title: | AHG9: On picture with mixed NAL unit types |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com |
| Source: | LG Electronics USA |  |  |

_____________________________

# Abstract

It is asserted that the current specification text regarding picture with mixed NAL unit types has the following problems:

- When a CLVS contains one or more PPSs with mixed_nalu_types_in_pic_flag is equal to 1, it is still possible that the value of sps_idr_rpl_present_flag is not equal to 1. Such situation would make the signalling of RPL in slice header of picture with mixed IDR and non-IRAP NAL unit types does not work.

- Currently for picture with mixed NAL unit types, the picture shall contain only one particular type of IRAP NAL unit type and one particular type of non-IRAP NAL unit type. If it is possible that a picture contains mixed of CRA and non-IRAP NAL unit types, it is inevitable that a picture with leading picture and non-IRAP NAL unit types is allowed, which is not yet.

This contribution proposes the following:

- Specify a constraint such that the value of sps_idr_rpl_present_flag shall be equal to 1 when there is at least one PPS referring to the SPS with the value of mixed_nalu_types_in_pic_flag is equal to 1.

- Allow picture to have mixed NAL unit types of leading picture type (i.e., RASL / RADL) and non-IRAP, non-leading picture NAL unit types.

# Problem statements

At the 16th JVET meeting, it was agreed to allow a picture to contain slices of mixed NAL unit types to support use-cases of merging two or more bitstreams with different intra period. It is asserted that the current specification text for such feature in the current VVC spec text has the following problems:

- When a picture contains IDR and non-IRAP NAL units, it is necessary that when RPL signalling is present in slice header, it is also present in the header of the IDR slice(s). RPL signalling is present in the slice header of an IDR slice when the value of sps_idr_rpl_present_flag is equal to 1. Currently, it is possible that the value of that flag is equal to 0 even when there is one or more picture with mixed NAL unit types.

- It is specified that when the current picture refers to a PPS in which the value of mixed_nalu_types_in_pic_flag is equal to 1, one or more of the VCL NAL units of the current picture shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GRA_NUT. This constraint only covers the case when the current picture contains mixed of IRAP and non-IRAP NAL unit types. However, the case of picture contains mixed of RASL/RADL and non-IRAP NAL unit types is not handled properly yet.

# Proposals

To solve the above problems, we propose the following:

- Specify a constraint such that the value of sps_idr_rpl_present_flag shall be equal to 1 when there is at least one PPS referring to the SPS with the value of mixed_nalu_types_in_pic_flag is equal to 1.

- Allow picture to have mixed NAL unit types of leading picture type (i.e., RASL / RADL) and non-IRAP, non-leading picture NAL unit types.

The text for the proposals above is as follows:

Update the constraints in subclause 7.4.2.2 about mixed_nalu_types_in_pic_flag as follows:

For VCL NAL units of any particular picture, the following applies:

- If mixed_nalu_types_in_pic_flag is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU.

- Otherwise (mixed_nalu_types_in_pic_flag equal to 1), one of the following shall be true:

  - One or more of the VCL NAL units shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GRA_NUT.

  - One or more of the VCL NAL units shall all have a particular value of nal_unit_type equal to RADL_NUT or RASL_NUT and the other VCL NAL units shall all have a particular value of nal_unit_type equal to TRAIL_NUT, STSA_NUT, RSV_VCL_4, RSV_VCL_5, RSV_VCL_6, or GRA_NUT.

Add the following constraint:

…

sps_idr_rpl_present_flag equal to 1 specifies that reference picture list syntax elements are present in slice headers of IDR pictures. sps_idr_rpl_present_flag equal to 0 specifies that reference picture list syntax elements are not present in slice headers of IDR pictures.

It is a requirement of bitstream conformance that the value of sps_idr_rpl_present_flag shall be equal to 1 when there is at least one PPS referring to the SPS with the value of mixed_nal_types_in_pic_flag eqaul to 1.

…

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).