| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0207 |
| --- | --- |

| Title: | [AHG9]: On signalling of TMVP enabled flag and collocated reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Seung Hwan Kim Junghak Nam Jaehyun Lim 10225 Willow Creek Rd, San Diego, CA 92131, USA 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea | Email | dr.hendry@lge.com seunghwan3.kim@lge.com junghak.nam@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the TMVP enabled flag and collocated reference picture signalling:

- Update the constraint for the value of pic_temporal_mvp_enabled_flag when there is no reference picture in the DPB with the same spatial resolution with the current picture. Rather than considering all reference picture in the DPB, the constraint should be expressed more precisely by considering only the active reference picture(s) of the picture.

- Update the signalling of collocated reference picture when TMVP is enabled for a picture. Two options are proposed:

  - Option 1: Signal TMVP collocated reference picture in picture header instead of in slice header and identify the collocated reference picture using its delta POC relative to the current picture’s POC.

  - Option 2: Keep the signalling of TMVP reference picture in SH and improve the signalling for the case when there is only one reference picture in the DPB with the same spatial resolution as the current picture.

# Proposal 1

In the current VVC specification, it is specified that when reference picture resampling (RPR) is enabled for coding of pictures in a CLVS, it is constrained that a picture can only use TMVP by referring to collocated reference picture that has the same resolution. One of the ways for enforcing this is by having the following constraint: “When no reference picture in the DPB has the same spatial resolution as the current picture, the value of pic_temporal_mvp_enabled_flag shall be equal to 0”.

While the constraint is not incorrect, it is not precise because the focus of the constraint should actually be the active reference pictures, rather than any reference pictures in the DPB. When the DPB has reference pictures with different resolution, the existing constraint fails to restrict the value of pic_temporal_mvp_enabled_flag to be equal to 0 even if slices of the picture associated with the picture header only refer to reference pictures with different resolution. In other words, a tighter constraint is needed to prevent pic_temporal_mvp_enabled_flag from being falsely enabled when there are reference frames with the same resolution as the current picture but those reference frames are not referred to by slices of the current picture.

We propose to update the existing constraint as follows:

pic_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If pic_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (pic_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of pic_temporal_mvp_enabled_flag is inferred to be equal to 0.
When no reference picture in the DPB has the same spatial resolution as the current picture, the value of pic_temporal_mvp_enabled_flag shall be equal to 0.
When no reference picture in the DPB that are referred to by slices of the current picture has the same spatial resolution as the current picture, the value of pic_temporal_mvp_enabled_flag shall be equal to 0.

NOTE – A reference picture is referred to by a slice if it is referred to by one of the active entry in either RefPicList[ 0 ] or RefPicList[ 1 ] of the slice.

# Proposal 2

In the current design of TMPV tool, it is constrained that when TMVP is enabled for a picture, all slices of the picture shall use the same reference picture as their TMVP collocated reference picture. This means that collocated reference picture is a picture level concept and it is naturally to be signalled in picture header.

The current signalling mechanism for collocated reference picture relies on the reference picture list (RPL) and RPL of each slice header using syntax element collocated_from_l0_flag and collocated_ref_idx. We assert that the signalling of collocated_from_l0_flag and collocated_ref_idx is redundant in situation when pic_temporal_mvp_enabled_flag is equal to 1 and there are more than one reference pictures that are referred to by the current slice, but among those reference pictures, there is only one reference picture that has the same resolution as the current picture. In such situation, there is no need to signal those two syntax element as it is clear which of the reference pictures is used as collocated reference picture.

To improve the signalling of TMVP collocated reference picture, we propose the following two options:

Option 1

- When TMVP is enabled for the current picture (i.e., pic_temporal_mvp_enabled_flag is equal to 1), the identification of collocated reference picture for slices of the current picture is signalled in picture header, instead of in slice header.

- The presence of the identification of collocated reference picture is conditioned on the value of pic_temporal_mvp_enabled_flag.

- The identification of collocated reference picture can be signalled as a delta value between the Picture Order Count (POC) of the current picture and the POC of the collocated reference picture. This delta value can be signalled using a pair of absolute delta value (i.e., col_ref_abs_delta_poc_val_minus1) and a sign flag (i.e., col_ref_abs_delta_poc_sign_flag).

- It is constrained that the collocated reference picture for the current picture shall not be the current picture itself.

- It is also constrained that the collocated reference picture for the current picture shall be one of the reference picture referred to by an active entry in the reference picture list (RPL) of any slice of the picture.

The text changes for Option 1 is as follows:

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if( pic_temporal_mvp_enabled_flag ) { |  |
| col_ref_abs_delta_poc_val_minus1 | ue(v) |
| col_ref_abs_delta_poc_sign_flag | u(1) |
| } |  |
| … |  |
| } |  |

pic_temporal_mvp_enabled_flag specifies whether temporal motion vector predictors can be used for inter prediction for slices associated with the PH. If pic_temporal_mvp_enabled_flag is equal to 0, the syntax elements of the slices associated with the PH shall be constrained such that no temporal motion vector predictor is used in decoding of the slices. Otherwise (pic_temporal_mvp_enabled_flag is equal to 1), temporal motion vector predictors may be used in decoding of the slices associated with the PH. When not present, the value of pic_temporal_mvp_enabled_flag is inferred to be equal to 0. When no reference picture in the DPB has the same spatial resolution as the current picture, the value of pic_temporal_mvp_enabled_flag shall be equal to 0.

col_ref_abs_delta_poc_val_minus1 plus 1 specifies absolute difference between the POC value of the current picture associated with the picture header and the POC value of the reference picture assigned as collocated reference picture of the current picture.

col_ref_abs_delta_poc_sign_flag equal to 0 specifies that the value of col_ref_abs_delta_poc_value_minus1 + 1 has a positive value. col_ref_abs_delta_poc_sign_flag equal to 1 specified that the value of col_ref_abs_delta_poc_value_minus1 + 1 has a negative value.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type != I ) { |  |
| … |  |
| if( pic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

…

slice_pic_order_cnt_lsb specifies the picture order count modulo MaxPicOrderCntLsb for the current picture. The length of the slice_pic_order_cnt_lsb syntax element is log2_max_pic_order_cnt_lsb_minus4 + 4 bits. The value of the slice_pic_order_cnt_lsb shall be in the range of 0 to MaxPicOrderCntLsb − 1, inclusive.

When the current picture is a GDR picture, the variable RpPicOrderCntVal is derived as follows:

RpPicOrderCntVal = PicOrderCntVal + recovery_poc_cnt	(114)

…

slice_delta_poc_msb_cycle_lt[ i ][ j ] specifies the value of the variable FullPocLt[ i ][ j ] as follows:

if( pic_rpl_present_flag )
	FullPocLt[ i ][ j ] = PicFullPocLt[ i ][ j ]
else {
	if( j  = =  0 )
		DeltaPocMsbCycleLt[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ]
	else				(144)
		DeltaPocMsbCycleLt[ i ][ j ] = delta_poc_msb_cycle_lt[ i ][ j ] + DeltaPocMsbCycleLt[ i ][ j − 1 ]
	FullPocLt[ i ][ j ] = PicOrderCntVal − DeltaPocMsbCycleLt[ i ][ j ] * MaxPicOrderCntLsb  −
		( PicOrderCntVal & ( MaxPicOrderCntLsb − 1 ) ) + PocLsbLt[ i ][ j ]
}

The value of slice_delta_poc_msb_cycle_lt[ i ][ j ] shall be in the range of 0 to 2(32 − log2_max_pic_order_cnt_lsb_minus4 − 4 ), inclusive. When not present, the value of slice_delta_poc_msb_cycle_lt[ i ][ j ] is inferred to be equal to 0.

When pic_temporal_mvp_enabled_flag is equal to 1, the variable ColPicPoc which specifies the POC value of the collocated picture used for temporal motion vector prediction is derived as follows:

ColPicPoc = PicOrderCntVal – ( ( col_ref_abs_delta_poc_val_minus1 + 1 ) *
							( 1 − 2 * col_ref_abs_delta_poc_sign_flag ) )

When pic_temporal_mvp_enabled_flag is equal to 1, it is a requirement of bitstream conformance that the following applies:

- The reference picture referred to by ColPicPoc shall be referred to by one of the active entry of either RefPicList[ 0 ] or RefPicList[ 1 ]

- The resolutions of the reference picture referred to by ColPicPoc and the current picture shall be the same.

ColPic that specifies the collocated picture is a picture having ColPicPoc of the active entry of either RefPicList[ 0 ] or RefPicList[ 1 ].

The variable CollocatedFromL0Flag which specifies whether the collocated picture used for temporal motion vector prediction is derived from reference picture list 0 is derived as follows:

- If ColPic is referred to by one of the active entry in RefPicList[ 0 ], CollocatedFromL0Flag is set to 1.

- Otherwise, CollocatedFromL0Flag is set to 0.

Note that the variable CollocatedFromL0Flag is used to replace the syntax element collocated_from_l0_flag that is signalled in slice header in the current version of VVC draft.

collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When collocated_from_l0_flag is not present, the following applies:

- If slice_type is not equal to B, the value of collocated_from_l0_flag is inferred to be equal to 1.

- Otherwise (slice_type is equal to B), the value of collocated_from_l0_flag is inferred to be equal to pps_collocated_from_l0_idc − 1.

collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, collocated_ref_idx refers to a picture in list 0, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and collocated_from_l0_flag is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When collocated_ref_idx is not present, the value of collocated_ref_idx is inferred to be equal to 0.

It is a requirement of bitstream conformance that the picture referred to by collocated_ref_idx shall be the same for all slices of a coded picture.

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture shall be the same and RefPicIsScaled[ collocated_from_l0_flag? 0 : 1 ][ collocated_ref_idx  ] shall be equal to 0.

Option 2

- Keep the signalling of TMVP collocated reference picture in slice header.

- When the number of reference pictures in the decoded picture buffer (DPB) referred to by a slices has the same resolution as the picture which the slice belongs to is zero or one, the signalling of information about collocated reference picture may not be present since they can be derived. Consequently, collocated_from_l0_flag and collocated_ref_idx may not be present.

- When TMVP is enabled for a picture (i.e., the value of pic_temporal_mvp_enabled_flag is equal to 1) and there is only one reference picture that is referred to by a slice that belongs to the picture that has the same resolution as the picture, collocated_from_l0_flag may not be present and can be derived as follows:

  - If the reference picture that has the same resolution as the picture is included in the reference picture list 0, the value of collocated_from_l0_flag is equal to 1

  - Otherwise, the value of collocated_from_l0_flag is equal to 0

- When the value of collocated_from_l0_flag is equal to 1 and there is only one reference picture in the reference picture list 0 that has the same resolution as the current picture, the syntax element collocated_ref_idx may not be present and can be inferred to the index of the reference picture (i.e., the one that has the same resolution as the current picture) in the reference picture list 0

- When the value of collocated_from_l0_flag is equal to 0 and there is only one reference picture in the reference picture list 1 that has the same resolution as the current picture, the syntax element collocated_ref_idx may not be present and can be inferred to the index of the reference picture (i.e., the one that has the same resolution as the current picture) in the reference picture list 1.

The text changes for Option 2 is as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type != I ) { |  |
| … |  |
| if( pic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc && TotalSameResolutionRefPic > 1) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| if( ( collocated_from_l0_flag && NumSameResolutionRefPic[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumSameResolutionRefPic [ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| … |  |
| } |  |
| … |  |
| } |  |

…

The variable TotalSameResolutionRefPic, NumSameResolutionRefPic[ i ], and FirstIdxSameResolutionRefPic[ i ] for i ranges from 0 to 1, inclusive, are derived as follows:

TotalSameResolutionRefPic = 0
for( i = 0; i < 2; i++ ) {
	for( j = NumRefIdxActive[ i ] − 1; j  >=  0; j− − ) {
		if( !RefPicIsScaled[ i ][ j ] ) {
			NumSameResolutionRefPic[ i ]++
			FirstIdxSameResolutionRefPic[ i ] = j
			TotalSameResolutionRefPic++
			if( i  = =  1 )
				for( sameRef = 0; k = 0; k < NumRefIdxActive[ 0 ]  &&  !sameRef; k++ )
					if(RefPicPocList[ 0 ][ k ]  ==  RefPicPocList[ i ][ j ] ) {
						TotalSameResolutionRefPic− −

						sameRef = 1
					}
		}
	}
}

collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When collocated_from_l0_flag is not present, the following applies:

- If slice_type is not equal to B, the value of collocated_from_l0_flag is inferred to be equal to 1.

- Otherwise (slice_type is equal to B), the value of collocated_from_l0_flag is inferred to be equal to pps_collocated_from_l0_idc − 1.

- Otherwise, if NumSameResolutionRefPic[ 0 ] is equal to 1, the value of collocated_from_l0_flag is inferred to be equal to 1.

- Otherwise, if NumSameResolutionRefPic[ 1 ] is equal to 1, the value of collocated_from_l0_flag is inferred to be equal to 0.

- Otherwise, (slice_type is equal to B and neither NumSameResolutionRefPic[ i ] is equal to 1), the value of collocated_from_l0_flag is inferred to be equal to pps_collocated_from_l0_idc − 1.

collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, collocated_ref_idx refers to a picture in list 0, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and collocated_from_l0_flag is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When collocated_ref_idx is not present, the following applies:

- If collocated_from_l0_flag is equal to 1 and NumSameResolutionRefPic[ 0 ] is equal to 1, the value of collocated_ref_idx is inferred to be equal to FirstIdxSameResolutionRefPic[ 0 ].

- Otherswise, if collocated_from_l0_flag is equal to 0 and NumSameResolutionRefPic[ 1 ] is equal to 1, the value of collocated_ref_idx is inferred to be equal to FirstIdxSameResolutionRefPic[ 1 ].

- Otherwise, the value of collocated_ref_idx is inferred to be equal to 0.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).