| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0208 |
| --- | --- |

| Title: | [AHG9]: On TMVP collocated reference picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry 10225 Willow Creek Rd, San Diego, CA 92131, USA | Email | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

When the current picture contains mixed NAL unit types of IRAP and non-IRAP, the current VVC specification does not disallow the pictures that follow the current picture in decoding order to use pictures that precede the current picture in decoding order as collocated reference picture for TMVP. It is asserted such case must be disallowed in order for the concept of allowing picture to have mixed NAL unit types to work since as all the slices of a picture share the same collocated reference index picture.

This contribution proposes that a constraint should be placed to ensure that the pictures that follow the current picture in the decoding order do not use pictures that precede the current picture in decoding order as TMVP collocated reference picture.

Furthermore, this contribution suggest the JVET group to consider signalling a flag in picture header such that when the flag is equal to 1, it specifies that pictures that follow the picture associated with the picture header shall not use pictures that precede the picture associated with the picture header in decoding order as TMVP collocated reference picture.

# Proposal

When TMVP is enabled for the current picture, it is constrained that all slices of the picture shall use the same reference picture as collocated reference picture for the TMVP process. When the current picture is a trailing picture that follow an IRAP picture (e.g., Pic N+5, N+6, and so forth from Bitstream 1 in Figure 1 below) it cannot use pictures that precede the IRAP picture (i.e., Pic N+2 from Bitstream 1 in Figure 1) as collocated picture because those picture are not available. On the other hand, the pictures Pic N+5, N+6, and so forth from Bitstream 2 in the Figure 1 below may use pictures precede Pic N+2. When sub-bitstream merging happens, it is possible that slices in the merged picture does not obey the constraint that they all have to use the same collocated reference picture.

Figure 1 Shows the merged bitstream generated from a bitstream with CRA and Trailing picture.

To overcome the above problem, it is proposed that a constraint regarding the collocated reference index needs to be put in place when the current picture contains slices with mixed NAL unit types of IRAP and non-IRAP such that any picture that follows the current picture (i.e., the picture contain the mixed NAL unit types) in the decoding order shall not use any picture that precedes the current picture in decoding order a the collocated reference picture.

Add the following constraint:

collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1.

When collocated_from_l0_flag is not present, the following applies:

- If slice_type is not equal to B, the value of collocated_from_l0_flag is inferred to be equal to 1.

- Otherwise (slice_type is equal to B), the value of collocated_from_l0_flag is inferred to be equal to pps_collocated_from_l0_idc − 1.

collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

When slice_type is equal to P or when slice_type is equal to B and collocated_from_l0_flag is equal to 1, collocated_ref_idx refers to a picture in list 0, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 0 ] − 1, inclusive.

When slice_type is equal to B and collocated_from_l0_flag is equal to 0, collocated_ref_idx refers to a picture in list 1, and the value of collocated_ref_idx shall be in the range of 0 to NumRefIdxActive[ 1 ] − 1, inclusive.

When collocated_ref_idx is not present, the value of collocated_ref_idx is inferred to be equal to 0.

It is a requirement of bitstream conformance that the picture referred to by collocated_ref_idx shall be the same for all slices of a coded picture.

It is a requirement of bitstream conformance that the resolutions of the reference picture referred to by collocated_ref_idx and the current picture shall be the same and RefPicIsScaled[ collocated_from_l0_flag ? 0 : 1 ][ collocated_ref_idx ] shall be equal to 0.

It is a requirement of bitstream conformance that when the value of mixed_nalu_types_in_pic_flag is equal to 1 and the current picture contains one of more NAL unit of IRAP type, the collocated picture used by any of picture that follows the current picture in decoding order shall not be picture that precedes the current picture in decoding order.

NOTE – When the current picture contains one or more subpicture that may be merged with subpicture from other bitstream and the resulted picture may contains mixed NAL unit types with one or more NAL units is IRAP type, encoder has to ensure that collocated picture selected for any pictures that follows the current picture in decoder order is not picture that precedes the current picture in decoding order.

# Discussion

To implement TMVP, decoder may use a buffer for storing motion vector field information. This buffer typically contains motion vector fields from reference pictures that have been used as collocated reference pictures and / or reference pictures that may be used as collocated reference picture. It would be beneficial for decoder implementation if decoder can flush the buffer. However, in the current specification, such feature is not provided.

We suggest the JVET group to consider whether it is beneficial to signal a flag in picture header such that when the flag is equal to 1, it specifies that pictures that follow the picture associated with the picture header shall not use pictures that precede the picture associated with the picture header as TMVP collocated reference picture.

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).