| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0213 |
| --- | --- |

| Title: | [AHG9/AHG12] On sub-picture ID mapping signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two options to modify subpicture signaling to reduce subpicture ID mapping redundancy.

- Option 1 proposes to signal subpicture sets in SPS, each subpicture set includes a number of subpictures applying to a CLVS of the CVS referring to the SPS

- Option 2 proposes to signal the subpicture ID mapping for those subpictures in a coded picture associated to the PPS and/or PH

# Introduction

Subpicture layout is signaled in SPS assuming the layout is consistent in each CLVS of the CVS that refers  to the SPS, the subpicture ID mapping may be signaled in SPS, PPS or PH. The relevant syntax elements are shown in the following tables.

Table 1 SPS RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

Table 2 PPS RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| … |  |
| } |  |

Table 3 Picture Header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

A single SPS may be referred by multiple CLVS, each with different subpicture boundaries, of the CVS, a subset of these subpictures signaled in SPS may apply to a CLVS. As a result, it is redundant to signal all subpicture ID mapping in PPS and/or PH that is referred by a CLVS.

# Proposal

## Option 1

It is proposed to signal subpicture set in SPS, each subpicture set may consist of one or multiple subpictures. Each CLVS of a CVS associated to the SPS may refer to a subpicture set. Table 4 is the proposed SPS syntax.

Table 4 Proposed SPS RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpic_set_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpic_set_minus1; i++ ) { |  |
| sps_num_subpics_minus1[ i ] | u(8) |
| for( j = 0; j <= sps_num_subpics_minus1[ i ]; j++ ) { |  |
| subpic_ctu_top_left_x[ i ][ j ] | u(v) |
| subpic_ctu_top_left_y[ i ][ j ] | u(v) |
| subpic_width_minus1[ i ][ j ] | u(v) |
| subpic_height_minus1[ i ][ j ] | u(v) |
| subpic_treated_as_pic_flag[ i ][ j ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpic_set_minus1; i++ ) |  |
| for( j = 0; j <= sps_num_subpics_minus1[ i ]; j++ ) |  |
| sps_subpic_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

sps_num_subpic_set_minus1 plus 1 specifies the number of subpicture set present in the SPS.

sps_num_subpics_minus1[ i ]  plus 1 specifies the number of subpictures of i-th subpicture set. sps_num_subpics_minus1[ i ] shall be in the range of 0 to 254. When not present, the value of sps_num_subpics_minus1[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_x[ i ][ j ] specifies horizontal position of top left CTU of j-th subpicture of i-th subpicture set in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ][ j ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ][ j ] specifies vertical position of top left CTU of j-th subpicture of i-th subpicture set in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ][ j ] is inferred to be equal to 0.

subpic_width_minus1[ i ][ j ] plus 1 specifies the width of the j-th subpicture of i-th subpicture set in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ][ j ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples / CtbSizeY ) − 1.

subpic_height_minus1[ i ][ j ] plus 1 specifies the height of the j-th subpicture of i-th subpicture set in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples / CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ][ j ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples / CtbSizeY ) − 1.

subpic_treated_as_pic_flag[ i ][ j ] equal to 1 specifies that the j-th subpicture in the coded picture associated with i-th subpicture set in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the j-th subpicture in the coded picture associated with i-th subpicture set in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ][ j ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ][ j ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the j-th subpicture in the coded picture associated with i-th subpicture set in the CLVS. loop_filter_across_subpic_enabled_flag[ i ][ j ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the j-th subpicture in the coded picture associated with i-th subpicture set in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ][ j ] is inferred to be equal to 1.

It is a requirement of bitstream conformance that the following constraints apply:

- For any two subpictures subpicA and subpicB, when the subpicture index of subpicA is less than that of subpicB, any coded slice NAL unit of subPicA shall precede any coded slice NAL unit of subPicB in decoding order.

- The shapes of the subpictures shall be such that each subpicture, when decoded, shall have its entire left boundary and entire top boundary consisting of picture boundaries or consisting of boundaries of previously decoded subpictures.

There is no change on PPS RBSP syntax except the bitstream  conformance requirement of  pps_num_subpics_minus1 is updated as following:

pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS.

It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to one of sps_num_subpics_minus1 list.

Table 5 is the proposed PH syntax for subpicture mapping signal.

Table 5 Proposed PPS RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_sps_subpic_set_idx | ue(v) |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1[ph_sps_subpic_set_idx]; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

ph_sps_subpic_set_idx specifies the index, to the list of subpicture set syntax structure in SPS, of the subpicture set syntax structure that applies to the coded picture referring the PH. When present, the value of ph_sps_subpic_set_idx shall be in the range of 0 to sps_num_subpic_set_minus1, inclusive.

## Option 2

To minimize the subpicture mapping redundence for each coded picture, it is proposed to signal the subpicture ID for those subpictures applied to the coded picture.

Table 6 Proposed PPS syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_subpics_idx | ue(v) |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| … |  |

pps_subpics_idx specifies the index, to the list of subpictures in the SPS, of the subpictures that applies to the first subpicture in the coded pictures referring to the PPS. When present, the value of pps_subpics_idx shall be in the range of 0 to sps_num_subpics_minus1, inclusive.

pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS. The value of ph_num_subpics_minus1 shall be in the range of 0 to sps_num_subpics_minus1, inclusive.

pps_subpic_id[ i ] specifies the subpicture ID of the ( pps_subpics_idx + i )-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

It is a requirement of bitstream conformance that the value of sum of pps_num_subpic_minus1 and pps_subpics_idx shall be in the range of 0 to  sps_num_subpics_minus1, inclusive.

Table 7 Proposed PH syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| ph_subpics_idx | ue(v) |
| ph_num_subpics_minus1 | ue(v) |
| for( i = 0; i <= ph_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| … |  |
| } |  |

ph_subpics_idx specifies the index, to the list of subpictures in the SPS, of the subpictures that applies to the first subpicture in the coded pictures referring to the PH. When present, the value of pps_subpics_idx shall be in the range of 0 to sps_num_subpics_minus1, inclusive.

ph_num_subpics_minus1 plus 1 specifies the number of subpictures in pictures associated to the PH. The value of ph_num_subpics_minus1 shall be in the range of 0 to sps_num_subpics_minus1, inclusive.

pps_subpic_id[ i ] specifies the subpicture ID of the ( ph_subpics_idx + i )-th subpicture. The length of the ph_subpic_id[ i ] syntax element is ph_subpic_id_len_minus1 + 1 bits.

It is a requirement of bitstream conformance that the value of sum of ph_num_subpic_minus1 and ph_subpics_idx shall be in the range of 0 to  sps_num_subpics_minus1, inclusive.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).