| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0214 |
| --- | --- |

| Title: | [AHG9/AHG12] Subpicture reposition information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Ahmed Hamza | Tel: Email: | 1-858-210-4807 Yong.He@InterDigital.com |
| Source: | InterDigital Communications, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes SEI message to indicate the mergeable sub-pictures and the reposition layout.

# Introduction

One of the main use cases for subpicture deployment is viewport dependent omnidirectional video streaming. Depending on the viewing orientation, different resolution or quality ranked subpicture may be extracted from the original bitstreams and repositioned or merged into a new composite bitstream for a single decoder-based VR device. The original bitstreams with different resolutions or quality ranking may be multiplexed into multi-layer bitstream, some subpicture may be extracted from the same layer and other subpictures may be extracted from a different layer. Figure 1 is an example of subpicture reposition.

Figure 1 subpicture merging example

The parameter set such as SPS and slice header of the new composite bitstream may be generated for the composite bitstream. The subpicture relevant content of SPS, PPS, PH may be changed. An SEI message providing the mergeable subpicture information is useful for identifying the corresponding subpictures, extracting and merging subpictures, generating subpicture layout structure in SPS, and updating slice address in slice header.

Since the subpicture boundary from the original bitstream may not align with CTU boundary at right or bottom picture boundary, an SEI message with recommended layout information would benefit the client to avoid reposition these subpictures somewhere else.

JVET-P0459 proposed an SEI message with mergeability ID and mergeability type to support merging of subpictures. This contribution proposes an SEI message indicating the mergeable subpictures and the information such as new subpicture layout and ID mapping for the composite bitstream, it is asserted that the proposed SEI message will simplify the subpicture extraction and reposition process.

# Proposal

The subpicture reposition SEI message carries information that allows identification of the mergeable subpicture and the required reposition operation.

Table 1 Proposed subpicture reposition information SEI message

| subpicture_reposition_info( payloadSize ) { | Descriptor |
| --- | --- |
| num_subpic_reposition_set_minus1 | u(8) |
| for( i = 0; i <= num_subpic_reposition_set_minus1; i++) { |  |
| reposition_pic_width_max_in_luma_samples[ i ] | ue(v) |
| reposition_pic_height_max_in_luma_samples[ i ] | ue(v) |
| reposition_num_subpics_minus1[ i ] | u(8) |
| for( j = 0; j <= num_subpics_minus1[ i ]; j ++) { |  |
| source_subpic_layer_id[ i ][ j ] | u(6) |
| source_subpic_id[ i ][ j ] | ue(v) |
| reposition_subpic_ctu_top_left_x[ i ][ j ] | ue(v) |
| reposition_subpic_ctu_top_left_y[ i ][ j ] | ue(v) |
| } |  |
| } |  |
| } |  |

num_subpic_reposition_set_minus1 plus 1 specifies the number of mergeable subpicture sets.

reposition_pic_width_max_in_luma_samples specifies the maximum width, in units of luma samples, of each composite picture.

reposition_pic_height_max_in_luma_samples specifies the maximum height, in units of luma samples, of each composite picture.

reposition_num _subpics_minus1[ i ] plus 1 specifies the number of subpictures in i-th mergeable subpicture set.

source_subpic_layer_id[ i ][ j ] specifies the source layer ID associated with the j-th subpicture of i-th mergeable subpicture set.

source_subpic_id[ i ][ j ] specifies the source subpicture ID associated with the j-th subpicture in i-th mergeable subpicture set.

reposition_subpic_ctu_top_left_x[ i ][ j ] specifies reposition horizontal position of top left CTU of j-th subpicture of i-th mergeable subpicture set in unit of CtbSizeY.

reposition_subpic_ctu_top_left_y[ i ][ j ] specifies reposition vertical position of top left CTU of j-th subpicture of i-th mergeable subpicture set in unit of CtbSizeY.

The extractor first identifies the reposition subpicture set with the corresponding composite picture resolution, then extracts the slices associated with the source layer ID and source subpicture ID, the new subpicture layout can be derived from the reposition_subpic_ctu_top_let_x and reposition_subpic_ctu_top_let_y straightforwardly.

# Patent rights declaration(s)

InterDigital Communications, Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).