| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0220-v1 |
| --- | --- |

| Title: | On VPS and Output Layer Set Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

We propose modifications to the video parameter set and output layer set signaling for asserted improvements and bug-fix.

Following is proposed:

- Proposal 1: It is proposed to conditionally signal HRD parameters related information in VPS when each layer is an OLS. When not present, the values for HRD parameters for the output layer sets are inferred in this case.

- Proposal 2:  It is proposed to conditionally signal the profile_tier_level() structures and  associated syntax elements (vps_num_ptls, pt_present_flag[ i ], ptl_max_temporal_id[ i ], vps_ptl_byte_alignment_zero_bit  and ols_ptl_idx[ i ]).

- Proposal 3:  If the total number of output layer sets is equal to the ols_hrd_parameters( ) syntax structures in the VPS, index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS syntax structure is not signalled, instead it is inferred.

- Proposal 4: A missing inference rule is proposed for ols_ptl_idx[ i ], which specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS.

# Introduction

VVC WD [1] includes a video parameter set and output layer set signalling.

We propose modifications to the video parameter set and output layer set signaling for asserted improvements and bug-fix.

Following is proposed:

- Proposal 1: It is asserted that when each layer is an output layer set, the HRD parameters for the output layer set do not need to be signalled in VPS as they are already specified in the SPS. Thus, it is proposed to conditionally signal HRD parameters related information in VPS when each layer is an OLS. When not present, the values for HRD parameters for the output layer sets are inferred in this case.

- Proposal 2:  It is asserted that when each layer is an output layer set, the profile-tier-level information for each output layer set is not signalled via ols_ptl_idx[ i ], which specifies the index, to the list of profile_tier_level( ) syntax structures. Thus, it is proposed to conditionally signal the profile_tier_level() structures and  associated syntax elements (vps_num_ptls, pt_present_flag[ i ], ptl_max_temporal_id[ i ], vps_ptl_byte_alignment_zero_bit  and ols_ptl_idx[ i ]) in this case.

- Proposal 3:  If the total number of output layer sets is equal to the ols_hrd_parameters( ) syntax structures in the VPS, index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS syntax structure is not signalled, instead it is inferred.

- Proposal 4: A missing inference rule is proposed for ols_ptl_idx[ i ], which specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS.

# Proposals

## Proposal 1

It is asserted that when each layer is an output layer set, the HRD parameters for the output layer set do not need to be signalled in VPS as they are already specified in the SPS. Thus, it is proposed to conditionally signal HRD parameters related information in VPS when each layer is an OLS. When not present, the values for HRD parameters for the output layer sets are inferred in this case.

The proposed changes are shown highlighted compared with VVC WD.

#### Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| vps_num_ptls | u(8) |
| for( i = 0; i < vps_num_ptls; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_byte_alignment_zero_bit /* equal to 0 */ | u(1) |  |
| for( i = 0; i < vps_num_ptls; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 && vps_num_ptls > 1 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| if( vps_num_dpb_params > 0 ) { |  |  |
| same_dpb_size_output_or_nonoutput_flag | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| } |  |  |
| for( i = 0; i < vps_num_dpb_params; i++ ) { |  |  |
| dpb_size_only_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| dpb_max_temporal_id[ i ] | u(3) |  |
| dpb_parameters( dpb_size_only_flag[ i ], dpb_max_temporal_id[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < vps_max_layers_minus1 && vps_num_dpb_params > 1; i++ ) { |  |  |
| if( !vps_independent_layer_flag[ i ] ) |  |  |
| layer_output_dpb_params_idx[ i ] | ue(v) |  |
| if( LayerUsedAsRefLayerFlag[ i ] && !same_dpb_size_output_or_nonoutput_flag ) |  |  |
| layer_nonoutput_dpb_params_idx[ i ] | ue(v) |  |
| } |  |
| if(!each_layer_is_an_ols_flag) |  |
| vps_general_hrd_params_present_flag | u(1) |  |
| if( vps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_cpb_params_present_flag | u(1) |  |
| if( TotalNumOlss > 1 ) |  |  |
| num_ols_hrd_params_minus1 | ue(v) |  |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| hrd_max_tid[ i ] | u(3) |  |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_temporal_id[ i ] ) |  |  |
| } |  |  |
| if( num_ols_hrd_params_minus1 > 0 ) |  |  |
| for( i = 1; i < TotalNumOlss; i++ ) |  |  |
| ols_hrd_idx[ i ] | ue(v) |  |
| } |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

#### Video parameter set RBSP semantics

…

vps_general_hrd_params_present_flag equal to 1 specifies that the syntax structure general_hrd_parameters( ) and other HRD parameters are present in the VPS RBSP syntax structure. vps_general_hrd_params_present_flag equal to 0 specifies that the syntax structure general_hrd_parameters( ) and other HRD parameters are not present in the VPS RBSP syntax structure.

When not present vps_general_hrd_params_present_flag is inferred to be equal to 0. When each_layer_is_an_ols_flag is equal to 1, and general_hrd_parameters( ) syntax  is not present in VPS, the general_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

vps_sublayer_cpb_params_present_flag equal to 1 specifies that the i-th ols_hrd_parameters( ) syntax structure in the VPS contains HRD parameters for the sublayer representations with TemporalId in the range of 0 to hrd_max_tid[ i ], inclusive. vps_sublayer_cpb_params_present_flag equal to 0 specifies that the i-th ols_hrd_parameters( ) syntax structure in the VPS contains HRD parameters for the sublayer representation with TemporalId equal to hrd_max_tid[ i ] only. When vps_max_sublayers_minus1 is equal to 0, the value of vps_sublayer_cpb_params_present_flag is inferred to be equal to 0.

When vps_sublayer_cpb_params_present_flag is equal to 0, the HRD parameters for the sublayer representations with TemporalId in the range of 0 to hrd_max_tid[ i ] − 1, inclusive, are inferred to be the same as that for the sublayer representation with TemporalId equal to hrd_max_tid[ i ]. These include the HRD parameters starting from the fixed_pic_rate_general_flag[ i ] syntax element till the sublayer_hrd_parameters( i ) syntax structure immediately under the condition "if( general_vcl_hrd_params_present_flag )" in the ols_hrd_parameters syntax structure.

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the general_hrd_parameters( ) syntax structure. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to 63, inclusive. When TotalNumOlss is greater than 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0.

hrd_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure. The value of hrd_max_tid[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of hrd_max_tid[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of hrd_max_tid[ i ] is inferred to be equal to vps_max_sublayers_minus1.

ols_hrd_idx[ i ] specifies the index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive.

When each_layer_is_an_ols_flag is equal to 1, the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

When not present and when each_layer_is_an_ols_flag is equal to 0, the value of ols_hrd_idx[[ i ] is inferred to be equal to 0.

## Proposal 2

It is asserted that when each layer is an output layer set, the profile-tier-level information for each output layer set is not signalled via ols_ptl_idx[ i ], which specifies the index, to the list of profile_tier_level( ) syntax structures. Thus, it is proposed to conditionally signal the profile_tier_level() structures and  associated syntax elements (vps_num_ptls, pt_present_flag[ i ], ptl_max_temporal_id[ i ], vps_ptl_byte_alignment_zero_bit  and ols_ptl_idx[ i ]) in this case.

The proposed changes are shown highlighted compared with VVC WD.

#### 7.3.2.2 Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| vps_max_layers_minus1 | u(6) |  |
| vps_max_sublayers_minus1 | u(3) |  |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |  |
| if( vps_max_layers_minus1 > 0 ) |  |  |
| vps_all_independent_layers_flag | u(1) |  |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |  |
| vps_layer_id[ i ] | u(6) |  |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |  |
| vps_independent_layer_flag[ i ] | u(1) |  |
| if( !vps_independent_layer_flag[ i ] ) |  |  |
| for( j = 0; j < i; j++ ) |  |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| if( vps_max_layers_minus1 > 0 ) { |  |  |
| if( vps_all_independent_layers_flag ) |  |  |
| each_layer_is_an_ols_flag | u(1) |  |
| if( !each_layer_is_an_ols_flag ) { |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| ols_mode_idc | u(2) |  |
| if( ols_mode_idc = = 2 ) { |  |  |
| num_output_layer_sets_minus1 | u(8) |  |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| if(!each_layer_is_an_ols_flag) { |  |  |
| vps_num_ptls | u(8) |
| for( i = 0; i < vps_num_ptls; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_byte_alignment_zero_bit /* equal to 0 */ | u(1) |  |
| for( i = 0; i < vps_num_ptls; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 && vps_num_ptls > 1 ) |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| } |  |  |
| if( !vps_all_independent_layers_flag ) |  |  |
| vps_num_dpb_params | ue(v) |
| … |  |  |
| vps_extension_flag | u(1) |  |
| if( vps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| vps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

each_layer_is_an_ols_flag equal to 1 specifies that each OLS contains only one layer and each layer itself in a CVS referring to the VPS is an OLS with the single included layer being the only output layer. each_layer_is_an_ols_flag equal to 0 that an OLS may contain more than one layer. If vps_max_layers_minus1 is equal to 0, the value of each_layer_is_an_ols_flag is inferred to be equal to 1. Otherwise, when vps_all_independent_layers_flag is equal to 0, the value of each_layer_is_an_ols_flag is inferred to be equal to 0.

…

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls − 1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

## Proposal 3

If the total number of output layer sets is equal to the ols_hrd_parameters( ) syntax structures in the VPS, index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS syntax structure is not signalled, instead it is inferred.

The proposed changes are shown highlighted below compared to VVC WD.

#### 7.3.2.2 Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| … |  |
| vps_general_hrd_params_present_flag | u(1) |
| if( vps_general_hrd_params_present_flag ) { |  |
| general_hrd_parameters( ) |  |
| if( vps_max_sublayers_minus1 > 0 ) |  |
| vps_sublayer_cpb_params_present_flag | u(1) |
| if( TotalNumOlss > 1 ) |  |
| num_ols_hrd_params_minus1 | ue(v) |
| for( i = 0; i <= num_ols_hrd_params_minus1; i++ ) { |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |
| hrd_max_tid[ i ] | u(3) |
| firstSubLayer = vps_sublayer_cpb_params_present_flag ? 0 : hrd_max_tid[ i ] |  |
| ols_hrd_parameters( firstSubLayer, hrd_max_tid[ i ] ) |  |
| } |  |
| if( TotalNumOlss != (num_ols_hrd_params_minus1+1) ) { |  |
| if( num_ols_hrd_params_minus1 > 0 ) |  |
| for( i = 10; i < TotalNumOlss; i++ ) |  |
| ols_hrd_idx[ i ] | ue(v) |
| } |  |
| } |  |
| vps_extension_flag | u(1) |
| if( vps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| vps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

num_ols_hrd_params_minus1 plus 1 specifies the number of ols_hrd_parameters( ) syntax structures present in the general_hrd_parameters( ) syntax structure. The value of num_ols_hrd_params_minus1 shall be in the range of 0 to 63, inclusive. When TotalNumOlss is equal to 1, the value of num_ols_hrd_params_minus1 is inferred to be equal to 0.

hrd_max_tid[ i ] specifies the TemporalId of the highest sublayer representation for which the HRD parameters are contained in the i-th ols_hrd_parameters( ) syntax structure. The value of hrd_max_tid[ i ] shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. When vps_max_sublayers_minus1 is equal to 0, the value of hrd_max_tid[ i ] is inferred to be equal to 0. When vps_max_sublayers_minus1 is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of hrd_max_tid[ i ] is inferred to be equal to vps_max_sublayers_minus1.

ols_hrd_idx[ i ] specifies the index of the ols_hrd_parameters( ) syntax structure that applies to the i-th OLS. The value of ols_hrd_idx[[ i ] shall be in the range of 0 to num_ols_hrd_params_minus1, inclusive. When TotalNumOlss is equal to (num_ols_hrd_params_minus1+1), ols_hrd_idx[ i ] is inferred to be equal to i. Otherwise (or when TotalNumOlss is not equal to (num_ols_hrd_params_minus1+1)), and wWhen not present, the value of ols_hrd_idx[ i ] is inferred to be equal to 0.

## Proposal 4

A missing inference rule is proposed for ols_ptl_idx[ i ], which specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS.

It is asserted that currently in VVC WD, if vps_num_ptls is equal to 1 and NumLayersInOls[ i ] is greater than 1, ols_ptl_idx[ i ] is not signalled and not inferred.

#### Video parameter set RBSP semantics

…

vps_general_hrd_params_present

ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the i-th OLS when NumLayersInOls[ i ] is greater than 1. When present, the value of ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls − 1, inclusive.

When NumLayersInOls[ i ] is equal to 1, the profile_tier_level( ) syntax structure that applies to the i-th OLS is present in the SPS referred to by the layer in the i-th OLS.

When ols_ptl_idx[ i ] is not present and vps_num_ptls is equal to 1 and NumLayersInOls[ i ] is greater than 1, ols_ptl_idx[ i ] is inferred to be equal to 0.

# Conclusion

This document proposes modifications to VPS and output layer set information signalling in VVC. It is proposed to adopt the proposals in the next version of VVC draft.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-P2001-vE, Versatile Video Coding (Draft 7), Geneva, October 2019