| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0225-v1 |
| --- | --- |

| Title: | AHG9/AHG12: On raster scan slice within a tile |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen, Chun-Chia Chen, Chih-Wei Hsu, Yu-Ling Hsiao, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {lulin.chen, chunchia.chen, cw.hsu, yuling.hsiao, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to allow and support the raster scan slice in units of CTU within a tile. It is alleged the proposed support may compromise the tile partition and the slice in CTUs within a picture in flexibility and coding efficiency.

# Introduction

In the VVC Draft 7 [1], the picture partition and slice layout information are specified in the PPS. When no_pic_partition_flag is equal to 1, it specifies that no picture partitioning is applied to each picture referring to the PPS. When no_pic_partition_flag is equal to 0, it specifies each picture referring to the PPS may be partitioned into more than one tile or slice. The tile and slice syntax structures are specified in the PPS, as highlighted in cyan, covering both cases of uniform and non-uniform tile columns and rows for the slice layout specification. For rectangular slices, the slice-in-tile case (a tile may be further partitioned into slices) is also specified in the PPS, with signalling only the slice height in units of CTU as the width is the tile width.

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| … |  |
| output_flag_present_flag | u(1) |
| mixed_nalu_types_in_pic_flag | u(1) |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| entropy_coding_sync_enabled_flag | u(1) |
| … |  |
| picture_header_extension_present_flag | u(1) |
| slice_header_extension_present_flag | u(1) |
| pps_extension_flag | u(1) |
| if( pps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| pps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Proposal

In our contribution JVET-Q0224 (AHG9/AHG12: On raster scan slice within a picture), it is proposed to specify raster scan slice in a picture without tile partition. In the same spirit, and considering a reasonable number of tiles with tile partition in a picture may be used practically for parallel processing, this contribution proposes to extend the slice-in-tile case to support the raster scan slice within a tile, i.e., the slice is specified in units of CTUs in raster scan within a tile and the slice shape may or may not be rectangular. This is in part similar to that in HEVC. It is alleged the proposed feature support in this contribution may compromise the tile partition and the slice in CTUs within a picture in flexibility and coding efficiency. To the hardware manufactures the related hardware modules used in the HEVC product today may be easily transferred to implement the VVC product in the future.

The illustration is as below with two tiles, four slices in the left tile and two slices in the right tile.

The proposed syntax and semantics for the slice_header( ) is as follows and highlighted in green.

| slice_header( ) { | Descriptor |
| --- | --- |
| slice_pic_order_cnt_lsb | u(v) |
| if( subpics_present_flag ) |  |
| slice_subpic_id | u(v) |
| if( rect_slice_flag ¦ ¦ NumTilesInPic > 1 ) |  |
| slice_address | u(v) |
| if( !rect_slice_flag && NumTilesInPic > 1 ) { |  |
| num_tiles_in_slice_minus1 | ue(v) |
| if( !num_tiles_in_slice_minus1 ) |  |
| slices_in_tile_present_flag | u(1) |
| if( slices_in_tile_present_flag ) |  |
| num_ctus_in_slice_minus1 | ue(v) |
| } |  |
| slice_type | ue(v) |
| … |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

slices_in_tile_present_flag equal to 1 specifies that the mulitple slices are present in the tile. slices_in_tile_present_flag equal to 0 specifyies no multiple slices is present in the current tile. When not present, slices_in_tile_present_flag is inferred to be equal to 0.

num_ctus_in_slice_minus1 plus 1 specifies the number of CTUs in the current slice. When slices_in_tile_present_flag is equal to 1, the value of num_ctus_in_slice_minus1 shall be in the range of 0 to TileSizeInCtbsY – 1, inclusive, where TileSizeInCtbsY = RowHeight[ tileY ] * RowWidth[ tileX ], tileX is the tile column index containing the current slice, and tileY is the tile row index containing the current slice. When not present, num_ctus_in_slice_minus1 is inferred to be equal to PicSizeInCtbsY − 1.

slice_address specifies the slice address of the slice. When not present, the value of slice_address is inferred to be equal to 0.

If slices_in_tile_present_flag is equal to 1, the following applies:

- The slice address is the first CTB in the slice, in CTB raster scan of a picture.

- The length of slice_address is Ceil( Log2 ( TileSizeInCtbsY ) ) bits.

- The value of slice_address shall be in the range of 0 to TileSizeInCtbsY − 1, inclusive, and the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

Otherwise, iIf rect_slice_flag is equal to 0, the following applies:

- The slice address is the raster scan tile index.

- The length of slice_address is Ceil( Log2 ( NumTilesInPic ) ) bits.

- The value of slice_address shall be in the range of 0 to NumTilesInPic − 1, inclusive.

Otherwise (rect_slice_flag is equal to 1), the following applies:

- The slice address is the slice index of the slice within the SubPicIdx-th subpicture.

- The length of slice_address is Ceil( Log2( NumSlicesInSubpic[ SubPicIdx ] ) ) bits.

- The value of slice_address shall be in the range of 0 to NumSlicesInSubpic[ SubPicIdx ] − 1, inclusive.

It is a requirement of bitstream conformance that the following constraints apply:

- If rect_slice_flag is equal to 0 or subpics_present_flag is equal to 0, the value of slice_address shall not be equal to the value of slice_address of any other coded slice NAL unit of the same coded picture.

- Otherwise, the pair of slice_subpic_id and slice_address values shall not be equal to the pair of slice_subpic_id and slice_address values of any other coded slice NAL unit of the same coded picture.

- When rect_slice_flag is equal to 0, the slices of a picture shall be in increasing order of their slice_address values.

- The shapes of the slices of a picture shall be such that each CTU, when decoded, shall have its entire left boundary and entire top boundary consisting of a picture boundary or consisting of boundaries of previously decoded CTU(s).

num_tiles_in_slice_minus1 plus 1, when present, specifies the number of tiles in the slice. The value of num_tiles_in_slice_minus1 shall be in the range of 0 to NumTilesInPic − 1, inclusive.

The variable NumCtuInCurrSlice, which specifies the number of CTUs in the current slice, and the list CtbAddrInCurrSlice[ i ], for i ranging from 0 to NumCtuInCurrSlice − 1, inclusive, specifying the picture raster scan address of the i-th CTB within the slice, are derived as follows:

if( rect_slice_flag ) {
	picLevelSliceIdx = SliceSubpicToPicIdx[ SubPicIdx ][ slice_address ]
	NumCtuInCurrSlice = NumCtuInSlice[ picLevelSliceIdx ]
	for( i = 0; i < NumCtuInCurrSlice; i++ )
		CtbAddrInCurrSlice[ i ] = CtbAddrInSlice[ picLevelSliceIdx ][ i ]	(7-80)
} else if( slices_in_tile_present_flag )

NumCtuInCurrSlice = num_ctus_in_slice_minus1 + 1

} else {
	NumCtuInCurrSlice = 0
	for( tileIdx = slice_address; tileIdx  <=  slice_address + num_tiles_in_slice_minus1[ i ]; tileIdx++ ) {
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		for( ctbY = tileRowBd[ tileY ]; ctbY < tileRowBd[ tileY + 1 ]; ctbY++ ) {
			for( ctbX = tileColBd[ tileX ]; ctbX < tileColBd[ tileX + 1 ]; ctbX++ ) {
				CtbAddrInCurrSlice[ NumCtuInCurrSlice ] = ctbY * PicWidthInCtb + ctbX
				NumCtuInCurrSlice++
			}
		}
	}
}

# Test results

The test results attached in the contribution JVET-Q0224 (AHG9/AHG12: On raster scan slice within a picture) show generally the coding efficiency benefits from a larger tile size partition in a picture.

# Conclusion

This contribution proposes to allow and support the raster scan slice in units of CTU within a tile. The proposed support may compromise the tile partition and slice in CTUs within in picture in flexibility and coding efficiency. The proponents suggest the group adopt the support of the above raster scan slice in units of CTU within a tile.

# References

- “Versatile Video Coding (Draft 7),” JVET-P2001-vE, October 2019, Geneva, Switzerland.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).