| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0228 |
| --- | --- |

| Title: | AHG12: Singalling of rectangular slices |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bae-Keun Lee (XRis Corp) Dongsan Jun (Kyungnam University) | Tel: Email: | +82-31-620-1200 baekeun.lee@xris.co.kr dsjun9643@kyungnam.ac.kr |
| Source: | Xris Corporation, Kyungnam University |  |  |

_____________________________

- Abstract

This contribution proposes to replace syntax of tile_idx_delta and num_slices_in_tiles_minus1 for rectangular slice signalling. This contribution is claimed to efficient representation of rectangular slices compared to VVC draft 7.

Introduction

This contribution proposes two modifications in terms of the rectangular slice signalling. Following is applied

- Proposal 1: It is proposed to replace tile_idx_delta[i] into tile_idx_delta_minus1[i] and tile_idx_delta_sign[i]

- Proposal 2: It is proposed to explicitly signal of number of slices in a tile which contains i-th slice to reduce bit amount signalling of rectangular slice.

Proposal

Proposal 1

In the VVC draft 7[1], tile_idx_delta[i] is signalled using se(v) to indicates tile index delta between tile index of (i+1)-th rectangular slice and that of i-th rectangular slice.

If i-th rectangular slice and (i+1) rectangular slice are located in the same tile, tile_idx_delta[i] is not signalled, therefore, tile_idx_delta[i] could be greater than 0. It is not efficient to signal tile_idx_delta[i] using se(v).

Therefore, it is proposed to tile_idx_delta_minus1[i] and tile_idx_delta_sign[i] as shown in the Table 1.

Table 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ){ |  |
| tile_idx_delta_minus1[i] | ue(v) |
| if (i>0) |  |
| tile_idx_delta_sign[i] | u(1) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |

- 7.4.3.4 Picture parameter set RBSP semantics

tile_idx_delta[ i ] specifies the difference in tile index between the i-th rectangular slice and the ( i + 1 )-th rectangular slice. The value of tile_idx_delta[ i ] shall be in the range of –NumTilesInPic + 1 to NumTilesInPic − 1, inclusive. When not present, the value of tile_idx_delta[ i ] is inferred to be equal to 0 [Ed. (HD): This inference seems either not correct or not sufficient. At least the inference should handle the value of tile_idx_delta[ i ] when i refer to the last slice of the picture and when tile_idx_delta_present_flag is equal to 0. These two cases seem to have different inference.]. In all other cases, the value of tile_idx_delta[ i ] shall not be equal to 0.

tile_idx_delta_minus1[i] + 1 specifies absolute value of difference in the tile index between the i-th rectangular slice and the ( i + 1 )-th rectangular slice., The value of tile_idx_delta_minus1[i] shall be in the range of 0 to NumTileInPic-2, inclusive. When not present,

tile_idx_delta_sign[i] equal to 1 indicates a positive sign for tile_idx_delta_minus1[i]. tile_idx_delta_sign[i] equal to 0 indicates a negative sign for tile_idx_delta_minus1[i]. When not present, the value of tile_idx_delta_sign[i] is inferred to be equal to 1.

- 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

if( tile_idx_delta_present_flag )
			tileIdx  +=  tile_idx_delta[ i ]

tileIdx += tile_idx_delta_sign[i] ?  (tile_idx_delta_minus1[ i ] + 1) : -(tile_idx_delta_minus1[ i ] + 1)

Proposal 2

It is proposed to explicitly signal of number of slices in a tile which contains i-th slice to reduce bit amount signalling of rectangular slice.

The proposed syntax changes are shown in the Table 2.

Table 2

| rect_slice_flag | u(1) |
| --- | --- |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_exp_slices_in_tile[ i ] | ue(v) |
| numExpSlicesInTile = num_exp_slices_in_tile[ i ] |  |
| for ( j = 0; j < numExpSlicesInTile; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| if( numExpSlicesInTile ) |  |
| i += RemNumSlices |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |

num_exp_slices_in_tile[ i ] specifies the slices number of explicitly provided in a tile which contains i-th slice. The value of num_exp_slices_in_tile[i] shall be in the range of 0 to RowHeight[TileY]-1, inclusive. When num_exp_slices_in_tiles[i] is euqal to 0, then there is no tile which contains i-th slice which is composed of multiple slices.

- 6.5.1 CTB raster scanning, tile scanning, and subpicture scanning processes

tileIdx = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
		NumCtuInSlice[ i ] = 0
	for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
		tileX = tileIdx % NumTileColumns
		tileY = tileIdx / NumTileColumns
		if( i  = =  num_slices_in_pic_minus1 ) {
			slice_width_in_tiles_minus1[ i ] = NumTileColumns − 1 − tileX
			slice_height_in_tiles_minus1[ i ] = NumTileRows − 1 − tileY
			num_slices_in_tile_minus1[ i ] = 0
		}
		if( slice_width_in_tiles_minus1[ i ]  = =  0  &&  slice_height_in_tiles_minus1[ i ]  = =  0 ) {	(29)
			ctbY = tileRowBd[ tileY ]

SumExpCtuHeight = 0 ; 
			numExpSlicesInTile = num_exp_slices_in_tile[ i ]
			for( j = 0; j < numExpSlicesInTile; j++ ) {
				AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )
				ctbY  +=  slice_height_in_ctu_minus1[ i ] + 1

SumExpCtuHeight += slice_height_in_ctu_minus1[ i ] + 1
				i++
			}
			AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, tileRowBd[ tileY + 1 ] )

RemNumSlices = (RowHeight[TileY] – SumExpCtuHeight+ slice_height_in_ctu_minus1[i-1]) / (slice_height_in_ctu_minus1[i-1] + 1)

for( j = 0; j < RemNumSlices; j++ ) {

AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + slice_height_in_ctu_minus1[ i-1 ] + 1 )

ctbY  +=  slice_height_in_ctu_minus1[ i-1 ] + 1
			}
		} else
			for( j = 0; j  <=  slice_height_in_tiles_minus1[ i ]; j++ )
				for( k = 0; k  <=  slice_width_in_tiles_minus1[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
						tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )
		if( tile_idx_delta_present_flag )
			tileIdx  +=  tile_idx_delta[ i ]
		else {
			tileIdx  +=  slice_width_in_tiles_minus1[ i ] + 1
			if( tileIdx % NumTileColumns  = =  0 )
				tileIdx  +=  slice_height_in_tiles_minus1[ i ] * NumTileColumns
		}
	}

References

[1] JVET-P2001-vE, Versatile Video Coding (Draft 7), Geneva, Oct 2019

Patent rights declaration(s)

Xris Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Kyungnam University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).